NEXALERT

MASTER SYSTEM ARCHITECTURE & ARCHITECTURE INVARIANTS

Final Engineering Specification — V1 Architecture Freeze Baseline

Document purpose This document is the master source of truth for the NexAlert product architecture. It defines what the product is, how the complete system works end-to-end, which subsystem owns each responsibility, how data moves, how the system behaves under failure, and which architectural rules are non-negotiable during implementation.

| Field | Value |
| --- | --- |
| Product | NexAlert |
| SIH Problem Statement | SIH26178 — Resilient AI-powered environmental monitoring and early warning |
| Document | 01 — Master System Architecture & Architecture Invariants |
| Status | FINAL ARCHITECTURE BASELINE |
| Audience | Engineering team, hardware team, software team, coding agents, reviewers, SIH judges |
| Primary use | Implementation reference and architectural guardrail |
| Scope | Prototype/V1 architecture with explicit V2/roadmap boundaries |

Implementation rule No implementation decision may silently contradict this document. If a real implementation constraint forces a change, the change must be documented, reviewed, and reflected here before downstream specifications are updated.

# Document Control & Navigation

| Version | Status | Purpose |
| --- | --- | --- |
| 1.0 | FINAL BASELINE | Architecture freeze after brainstorming and adversarial review |
| 1.x | Controlled change | Only approved clarifications or implementation-driven changes |

## How to use this document

Read this document first before implementing any NexAlert subsystem.

Use the architecture invariants as hard constraints when making design or coding choices.

Use subsystem-specific final specifications for implementation detail. Those documents must not redefine architecture in conflict with this document.

Treat terms such as Incident, Hazard State, Physical Footprint, Operational Buffer, Confidence, Severity, Operational Risk, and Information Condition as canonical terms.

Treat real hardware data and synthetic telemetry as two sources entering the same post-ingestion processing pipeline.

## Document map

| Section | Topic |
| --- | --- |
| 1 | Product definition and design principles |
| 2 | System scope and responsibilities |
| 3 | Complete end-to-end product flow |
| 4 | Layered architecture |
| 5 | Field node and edge architecture |
| 6 | Master / local-gateway architecture |
| 7 | Backend and cloud architecture |
| 8 | Intelligence architecture |
| 9 | Hazard, incident, and geospatial architecture |
| 10 | Exposure and operational decision architecture |
| 11 | Citizen and authority product surfaces |
| 12 | Communication and resilience |
| 13 | Simulation and replay |
| 14 | Security and trust |
| 15 | Persistence, events, and audit |
| 16 | Technology and deployment boundaries |
| 17 | Failure and degraded-operation model |
| 18 | Canonical data-flow contracts |
| 19 | Architecture invariants |
| 20 | V1 scope, non-goals, and roadmap |
| 21 | Implementation ownership matrix |
| 22 | End-to-end example |
| 23 | Acceptance gates before calling architecture implemented |

# 1. Product Definition

## 1.1 What NexAlert is

NexAlert is a resilient, distributed environmental monitoring and early-warning platform designed to detect developing hazards from real sensor evidence, reason about the quality and reliability of that evidence, convert credible hazard states into geospatially meaningful impact information, and provide actionable information to authorities and nearby communities.

The platform is intentionally designed as a complete decision-support loop rather than a sensor dashboard. A sensor reading by itself is not the product. The product is the chain from sensing to trustworthy interpretation to spatial consequence to human action.

Product sentence A disaster sensor network that keeps making trustworthy decisions even when it is operating with degraded connectivity, and clearly communicates uncertainty when the evidence is insufficient.

## 1.2 Product pillars

| Pillar | Meaning in the implemented system | Concrete evidence |
| --- | --- | --- |
| Access | People can reach emergency information without requiring a mandatory native app or permanent cloud dependency. | Local NexAlert Wi-Fi/captive portal path; web/PWA experience; accessible emergency page. |
| Resilience | The system continues useful local operation during connectivity loss and supports store-and-forward recovery. | Node/Master buffering, local networking, reconnection sync, explicit degraded state. |
| Intelligence | The system evaluates health, quality, anomaly, evidence, confidence, severity, risk, and state rather than merely displaying readings. | Health/quality/reliability pipeline; deterministic hazard reasoning; explainability. |

## 1.3 Problem framing

NexAlert addresses a class of disaster-management problem where environmental signals can be noisy, incomplete, delayed, contradictory, or locally unavailable. The architecture therefore treats uncertainty and failure as first-class operating conditions rather than exceptional bugs.

Distributed sensing provides localized evidence rather than relying on a single central observation.

Edge/local processing allows hazard reasoning to continue when upstream connectivity is lost.

Evidence quality and sensor reliability are kept separate from hazard severity and operational risk.

Geospatial reasoning turns an incident into a changing area, risk surface, exposure, and operational context.

Authority actions are recommended and tracked; NexAlert does not autonomously dispatch emergency responders.

Citizen communication is designed as a safety surface with explicit safe-route and shelter-in-place fallbacks.

# 2. System Scope & Responsibility Boundaries

## 2.1 System boundary

The system begins at physical/environmental sensing and ends at human-facing warning, authority decision support, response workflow, and historical/audit evidence. It includes field hardware, local connectivity, ingestion, intelligence, geospatial simulation, exposure analysis, response management, alert delivery, citizen interaction, authority dashboards, simulation, and persistence.

The system does not claim to replace government command structures, field responders, official emergency communication authorities, or full operational wildfire/flood forecasting systems.

## 2.2 High-level ownership

| Subsystem | Primary responsibility | Must NOT own |
| --- | --- | --- |
| Field Node | Sense, self-test, basic filtering, diagnostics, local networking, buffering, authenticated telemetry. | Large geospatial simulation, population analysis, authority dispatch decisions. |
| Master / Local Gateway | Aggregate node data, local intelligence where enabled, local service continuity, buffering, synchronization. | Permanent authority of truth for cloud history when disconnected; it reconciles on reconnect. |
| Telemetry Ingestion | Validate, authenticate, normalize, timestamp, deduplicate, persist canonical telemetry. | Hazard interpretation. |
| Intelligence Engine | Health, quality, reliability, baseline, anomaly, evidence, confidence, severity, risk, state. | UI rendering, geospatial polygonization, autonomous dispatch. |
| Incident Manager | Create, correlate, update, merge/split under defined policy, resolve incidents. | Raw sensor correction. |
| Fire/Geospatial Engine | Terrain, fuel, moisture, wind/slope, spread, arrival-time field, footprint, buffer, risk surface, geometry. | Sensor trust and authority response policy. |
| Exposure Engine | Population, citizens, roads, schools, hospitals, infrastructure overlay. | Hazard physics. |
| Response Engine | Recommend response actions and priorities; track approval and operational progress. | Autonomous dispatch. |
| Alert Layer | Fan out canonical emergency events through available communication channels. | Invent hazard state. |
| Citizen UI | Display safe, plain-language information and SOS controls. | Recompute hazard or risk. |
| Authority Dashboard | Operational visualization, acknowledgement, action workflow, investigation, audit. | Recompute domain truth. |
| Simulation | Generate synthetic telemetry/scenarios and replay them through the same pipeline. | Use a parallel intelligence path. |

# 3. Complete End-to-End Product Flow

## 3.1 Master flow

REAL HARDWARE PATH
Sensors → ESP32 Node → authenticated telemetry → Local/Master → Ingestion
→ Health/Quality/Reliability → Baseline/Anomaly → Hazard Reasoning
→ Incident Manager → Geospatial/Spread/Risk → Exposure
→ Response Recommendation → Human Approval → Alerts/Actions
→ Authority Dashboard + Citizen Experience → Audit/History

SIMULATION PATH
Scenario Generator → canonical telemetry payload → SAME INGESTION
→ SAME INTELLIGENCE → SAME INCIDENT → SAME GEOSPATIAL
→ SAME EXPOSURE → SAME RESPONSE → SAME UI/ALERT PATH

OFFLINE PATH
Sensors → Node → Local buffering / local intelligence → Local emergency network
→ Local authority/citizen access
→ reconnect → reconciliation → cloud synchronization

## 3.2 The product loop in plain language

A field node measures environmental signals and reports them with timestamps, diagnostics, sequence information, and authenticated identity.

The ingestion layer checks that the data is valid, not spoofed, not duplicated, and expressed in canonical units.

The intelligence layer estimates whether each sensor is healthy, whether the signal is trustworthy, whether the measurement is anomalous relative to a baseline, and how those observations combine into hazard evidence.

The hazard reasoning layer produces a hazard-specific state such as NORMAL, WATCH, SUSPECTED, CONFIRMED, or CRITICAL plus confidence, severity, operational risk, and an information condition.

The incident manager determines whether the event belongs to an existing incident or should create a new incident using deterministic spatial/temporal correlation.

For fire, the geospatial engine calculates how the hazard can propagate using terrain, fuel, moisture, wind, and slope, then generates arrival times, footprints, projections, and derived risk surfaces.

The exposure engine overlays hazard geometry with real population and infrastructure data and classifies current, warning, and projected exposure.

The response engine converts the incident context into categorical operational recommendations such as MONITOR, VERIFY, NOTIFY, DISPATCH RECOMMENDATION, URGENT RESPONSE, or PUBLIC WARNING.

A human authority operator approves or rejects actions. NexAlert does not autonomously dispatch responders.

The alert layer creates one canonical emergency event and delivers it through the available citizen/authority channels.

The citizen surface provides simple action guidance, map context, safe-place/route information, and SOS. If no viable route exists, it returns SHELTER_IN_PLACE rather than inventing a dangerous route.

Every important state change and operator action is recorded for auditability and later validation.

## 3.3 What makes this a product rather than a collection of demos

All components share canonical entities and data contracts.

Simulation is not a second application; it is an alternate telemetry source.

The same incident can drive geospatial analysis, authority response, citizen alerts, and audit history.

Failure states are explicit and propagated rather than hidden behind zeros or silent fallbacks.

The authority and community experiences are both connected to the same underlying event.

# 4. Layered Architecture

## 4.1 Twenty-layer architecture

| # | Layer | Inputs | Core logic | Outputs | Failure | Degraded behavior | Source of truth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Field node + sensors | Raw sensor readings, diagnostics | Sampling, self-test | Raw telemetry + diagnostics | Sensor dead | Hard-gate failed sensor | Node hardware |
| 2 | Local connectivity + Master | LoRa/Wi-Fi/local link | Routing, heartbeat, buffering | Delivered telemetry | Link lost | Local buffer/store-forward | Link state |
| 3 | Telemetry ingestion | Raw signed payload | Validate, dedup, HMAC verify, normalize | Canonical telemetry | Malformed/spoofed | Reject + log | Ingestion record |
| 4 | Health/Quality/Reliability | Telemetry + diagnostics | H, Q, R computation | Reliability-weighted signal | Diagnostic conflict | Lower reliability | Intelligence state |
| 5 | Baseline/Anomaly | Reliable readings | Robust baseline + anomaly logic | A_i + baseline state | Baseline not ready | Learning / unavailable anomaly | Baseline state machine |
| 6 | Hazard reasoning | Features/anomaly/reliability | Hazard-specific reasoning | E_h,C_h,S_h,R_h,state | Evidence conflict | DEGRADED/UNKNOWN | Hazard state |
| 7 | Incident manager | Node hazard states | Spatial/temporal correlation | Incident entity | Ambiguous merge | Create new incident + flag | Incident store |
| 8 | Fire spread engine | Ignition, wind, terrain, fuel, moisture | Directional ROS + arrival-time propagation | Arrival-time field | Missing input | Labeled assumption / degraded | Arrival field |
| 9 | Affected geometry | Arrival-time field | Thresholding + polygonization + validity repair | Current/warning/projection + buffer | Invalid geometry | Repair or flag | Raster truth |
| 10 | Risk/heatmap | Arrival-time + intensity | Spatial risk index | Hazard risk surface | Degraded info | Uncertain label | Risk surface |
| 11 | Exposure | Geometry + real datasets | Spatial overlay | Population/infrastructure counts | Dataset unavailable | Explicit unavailable | Source dataset |
| 12 | Response/action | Incident + exposure + SOS | Policy rules + categorical priority | Recommendation + actions | Ambiguous priority | VERIFY; never auto-dispatch | Response state |
| 13 | Authority dashboard | Domain state | Render + interact | Operator UI | Stale data | Visible staleness | Backend domain state |
| 14 | Citizen interface | Incident + routing + alerts | Plain-language render | Emergency experience | Route unavailable | SHELTER_IN_PLACE | Backend domain state |
| 15 | SOS | Citizen input | Tiering + dedup | SOS record | Repeated presses | Consolidate, preserve history | SOS store |
| 16 | Alerts/communications | Canonical event | Fan-out + delivery tracking | Alert records | Channel failure | Fallback channel | Alert log |
| 17 | Persistence/audit | State changes | Immutable/append-only records | History + audit | Write failure | Retry queue | Audit store |
| 18 | Offline/store-forward | Buffered telemetry/events | Queue + reconciliation | Synced state | Master failure | Node/local gateway continuity | Local buffer |
| 19 | Security/auth | Keys + signatures | HMAC and event-signing verification | Trust decisions | Invalid auth | Reject + flag | Key store |
| 20 | Simulation/replay | Synthetic telemetry | Same path as live | Same outputs, SIMULATION-tagged | Mislabeled data | Persistent simulation banner | Source metadata |

# 5. Field Node & Edge Architecture

## 5.1 Node responsibility

The ESP32-class field node is the physical sensing and resilient edge endpoint. It is deliberately kept lightweight. Its job is to sense, validate basic electrical/sensor conditions, package trustworthy telemetry, maintain identity and communication, buffer data during outages, support local emergency networking where required, and run only those intelligence functions that are computationally appropriate for the embedded platform.

## 5.2 Node responsibilities

Sensor acquisition and timestamping.

Basic filtering and signal conditioning where appropriate.

Sensor diagnostics and self-test status.

Heartbeat reporting.

Battery reporting as a separate subsystem from sensor health.

Communication management and reconnect logic.

Per-node authenticated telemetry using a provisioned secret and HMAC.

Sequence numbers for ordering/deduplication.

Local buffering during communication loss.

Local emergency page/gateway capability needed for Master-independent citizen access.

Selected lightweight anomaly or threshold logic only when explicitly assigned to the embedded implementation.

## 5.3 Node non-responsibilities

No large fire-spread raster simulation.

No population/infrastructure spatial analysis.

No route optimization across a geographic network.

No authority dispatch decision.

No replacement of the canonical backend intelligence model.

Embedded intelligence rule The mathematical specification is shared, but implementation may differ by compute target. Python is the reference implementation for server-side intelligence; C/C++ is the embedded implementation on ESP32 where local intelligence is required. Golden-vector tests must compare the two implementations within defined tolerances.

# 6. Master / Local Gateway Architecture

## 6.1 Purpose

The Master is the local coordination and compute point for the field network. In the prototype it may be a Raspberry Pi-class computer. It aggregates field data, can run the heavier local domain services, maintains local continuity, and synchronizes with the cloud/backend when connectivity is available.

## 6.2 Master responsibilities

Receive authenticated node telemetry.

Maintain node/network status.

Run or host the modular backend services required for local autonomy.

Run geospatial computation locally when configured.

Persist local events required for continuity.

Expose authority-local UI access.

Coordinate store-and-forward synchronization.

Publish updated incident and action state to connected clients.

## 6.3 Master failure model

Master failure is not equivalent to internet failure. The architecture explicitly distinguishes them. If the Master is powered off or otherwise dead, the local emergency citizen path must remain available through the field node or a lightweight local gateway function. This is a mandatory resilience capability, not merely a demo convenience.

CONNECTIVITY FAILURE
Internet/cloud unavailable
→ Master remains alive
→ local intelligence and local service continue
→ data buffered
→ reconnect → reconcile → sync

MASTER FAILURE
Master physically unavailable
→ node/local gateway remains available
→ local emergency page remains reachable
→ node buffers telemetry/events
→ Master recovery → reconciliation

NODE FAILURE
One node unavailable
→ other nodes continue
→ stale/dead node marked unavailable
→ incident confidence may degrade if that node was core evidence

# 7. Backend & Cloud Architecture

## 7.1 Deployment philosophy

For the SIH prototype, NexAlert uses a modular monolith with background workers rather than a distributed microservice deployment. The architecture remains modular in code and domain ownership, but deployment complexity is intentionally limited.

## 7.2 Logical backend modules

backend/
  ingestion/
  security/
  intelligence/
  hazards/
  incidents/
  geospatial/
    terrain/
    fire_spread/
    affected_area/
    risk/
    exposure/
  response/
  alerts/
  sos/
  simulation/
  users/
  audit/
  api/
  workers/
  common/

## 7.3 Synchronous vs asynchronous work

| Execution type | Typical work |
| --- | --- |
| Near-real-time / synchronous | Telemetry validation, authentication, normalization, health/quality, anomaly/evidence update, incident state update. |
| Background / asynchronous | Large spread calculations, population exposure, route computation, alert fan-out, historical aggregation. |
| Interactive | Dashboard drill-down, citizen map, operator approval, simulation control. |

## 7.4 Source-of-truth rule

The backend/domain state is authoritative for incident, hazard, response, and exposure status. Frontends may cache and render this state, but they must not independently create a competing version of the truth.

# 8. Intelligence Architecture

## 8.1 Intelligence chain

Canonical telemetry
  ↓
Validation + authentication
  ↓
Sensor Health H_i
  ↓
Signal Quality Q_i
  ↓
Reliability R_i
  ↓
Baseline state B_i
  ↓
Anomaly A_i
  ↓
Hazard evidence E_h
  ↓
Evidence confidence C_h
  ↓
Severity S_h
  ↓
Operational risk R_h
  ↓
Hazard state
  ↓
Information Condition
  ↓
Incident Manager

## 8.2 Canonical conceptual quantities

| Quantity | Meaning | Must not be confused with |
| --- | --- | --- |
| H_i | Sensor health based on diagnostics/hard-failure state. | Battery percentage or signal anomaly. |
| Q_i | Signal quality/integrity/stability. | Hazard confidence. |
| R_i | Evidence reliability after health, quality, and calibration/context factors. | Operational risk. |
| A_i | Bounded anomaly magnitude relative to an appropriate baseline. | Probability of hazard. |
| E_h | Hazard-specific evidence from the relevant sensor/features. | Calibrated probability unless explicitly calibrated. |
| C_h | Confidence in the evidence assessment. | Severity. |
| S_h | Hazard severity index. | Probability. |
| R_h | Operational risk index considering consequence/context. | Physical intensity or probability. |
| Information Condition | How trustworthy/current/complete the available information is. | Hazard severity. |
| Hazard State | Operational state of the hazard evidence. | Response action. |

Mathematical authority The complete formulas and parameter definitions live in Document 04 — Mathematical Intelligence & Hazard Reasoning Specification. This master document fixes ownership and conceptual separation; it does not redefine those formulas.

# 9. Hazard, Incident & Geospatial Architecture

## 9.1 Hazard vector

Each telemetry assessment may evaluate multiple hazards independently. A single sample can result in a vector such as Fire=CONFIRMED, Flood=NORMAL, Pollution=WATCH, Landslide=UNKNOWN. Multi-hazard behavior is not represented by a single universal hazard score.

## 9.2 Incident entity

An Incident is the persistent representation of a real-world or simulated hazard situation. It may receive evidence from multiple nodes and across time. Its lifecycle is independent from the lifecycle of any individual sensor packet.

## 9.3 Incident correlation rule

For V1, reports of the same hazard type are correlated into one incident when they fall within a configured spatial radius and temporal window. If either boundary is exceeded, a new incident is created. This is deterministic and intentionally avoids a complex clustering model during the SIH prototype.

same hazard
AND distance(report, existing_incident) <= spatial_threshold
AND time_delta <= temporal_threshold
→ attach to existing incident

otherwise
→ create new incident

## 9.4 Fire geospatial chain

Ignition
  ↓
DEM → slope/aspect
Fuel raster → fuel behavior
Moisture/context → moisture response
Wind → FROM→TO conversion + midflame adjustment
  ↓
Wind/slope vector combination
  ↓
Directional ROS
  ↓
Arrival-time field
  ↓
Current footprint / warning / projection
  ↓
Geometry validity + MultiPolygon
  ↓
Operational safety buffer (policy, not physics)
  ↓
Hazard Risk Surface / heatmap
  ↓
Exposure overlays

## 9.5 Geometry truth

The arrival-time raster/field is the primary geometric source of truth. Polygons are derived representations for display, targeting, and area statistics. The physical fire footprint is distinct from any operational buffer or policy margin.

## 9.6 Risk truth

The Hazard Risk Surface is an operational index derived from spatial urgency/time-to-arrival and local hazard intensity/trend inputs. It is not a calibrated probability unless a future version is explicitly validated as one.

Multi-hazard rule Each hazard risk surface remains independently addressable and toggleable. The system must never create a misleading universal multi-hazard scalar by algebraically adding or averaging unrelated hazard risk surfaces.

# 10. Exposure & Operational Decision Architecture

## 10.1 Exposure

Exposure is a consequence/context layer, not the same thing as physical hazard intensity. Population, registered citizens, buildings, roads, schools, hospitals, and other infrastructure are spatially overlaid against current, warning, and projected geometries.

Current, warning, and projected exposure are kept separately.

Nested zones must not be naively summed.

Unavailable population/infrastructure data must never be represented as numeric zero.

Real datasets are required for any displayed real-world exposure number.

## 10.2 Response

The response engine converts incident state, severity, operational risk, trend, information condition, exposure, SOS context, and response status into an operational recommendation. It does not directly dispatch responders.

INCIDENT CONTEXT
    ↓
Response Recommendation
    ↓
Human Approval Gate
    ↓
Approved Action
    ↓
Notification / Assignment / Response Tracking

Hard product invariant NexAlert recommends. A human approves and acts. There is no autonomous dispatch path, including no special 'demo mode' exception.

# 11. Product Surfaces

## 11.1 Authority experience

The authority dashboard is an operational command surface. Its primary question is: What needs my attention now, why, where, how certain is the information, what is exposed, and what action is pending?

Overview: live operational map, active incidents, nodes, Master/network health, risk overlays, SOS, current alerts.

Incidents: incident list, command card, evidence explanation, timeline, state and information condition.

Fire Spread: LIVE/SIMULATION, current footprint, projected spread, time controls, environment context.

Affected Area & Impact: current/warning/projection zones, population, roads, schools, hospitals, critical infrastructure.

Multi-Hazard & Simulation: independent hazard layers plus same-pipeline synthetic scenarios.

Nodes & Network: health, battery, link state, heartbeat, topology/isolation.

Telemetry & Intelligence: raw telemetry through the intelligence chain.

Citizen SOS: map, queue, categorical priority, profile/contact actions.

Alert Management: issued alerts, delivery state, escalation/stand-down.

Historical Analysis: incidents, trends, response times, node reliability, outcomes.

Response & Actions: approved actions and operational state.

Audit Trail: immutable history of system/operator changes.

System: configuration and system health.

## 11.2 Citizen experience

The citizen experience is intentionally one-page and action-first. In emergency mode, the first viewport shows hazard, severity, state, distance/direction, freshness, and the most important recommended action. The map then shows incident/affected/projected geometry, safe place and route when available.

Normal state: nearby status, recent alerts, map, SOS.

Emergency state: one clear action, hazard context, live map, safe destination guidance.

No viable safe route: SHELTER_IN_PLACE.

SOS remains available independent of whether a known hazard is active.

Plain-language labels replace raw internal scores by default.

# 12. Communication & Resilience Architecture

## 12.1 Canonical communication graph

Field Node
  ├─→ Master / local gateway
  │      ├─→ Local authority UI
  │      ├─→ Local citizen Wi-Fi/captive portal
  │      └─→ Cloud synchronization when online
  └─→ buffered telemetry/events

Cloud/backend
  ├─→ Authority dashboard
  ├─→ Web/push alert channels
  └─→ Historical/audit storage

## 12.2 Normal operation

Telemetry moves upward from nodes to the Master/backend. Domain state changes are then distributed to connected authority clients and alert channels. One canonical emergency event may drive multiple delivery channels.

## 12.3 Offline operation

Offline behavior is designed around useful local autonomy, not the claim that every cloud feature remains available. Core local detection, local incident state, local emergency access, and buffering continue to the degree supported by the deployed hardware. Cloud-dependent history and external notification are clearly marked unavailable until reconnection.

## 12.4 Store-and-forward

packet generated
→ sequence + measurement timestamp
→ local buffer
→ connectivity returns
→ authenticate
→ order using receive timestamp + sequence rules
→ deduplicate
→ domain processing uses measurement timestamp
→ persist / publish reconciled state

## 12.5 Degraded information

Degraded connectivity must not silently convert into false freshness. The system should surface stale data, missing feeds, and information condition changes explicitly.

# 13. Simulation & Replay Architecture

## 13.1 Purpose

Simulation exists to bootstrap software development, validate deterministic logic, rehearse multi-hazard behavior, inject failure cases, and support a controlled SIH demonstration. It is not allowed to create a second intelligence implementation.

## 13.2 Simulation contract

Scenario definition
→ synthetic telemetry
→ POST /api/v1/telemetry
→ identical validation
→ identical intelligence
→ identical incident creation/update
→ identical geospatial engine
→ identical exposure
→ identical response
→ identical dashboard/citizen outputs

## 13.3 Synthetic scenario capabilities

Normal baseline periods.

Slow and rapid hazard onset.

Correlated multi-sensor escalation.

Sensor noise and drift.

Missing measurements and stale packets.

Sensor failure and diagnostics changes.

Communication outage and recovery.

Multi-node corroboration.

Contradictory sensor evidence.

Multi-hazard combinations without collapsing them into one risk number.

Simulation honesty Synthetic data can demonstrate that the system behaves correctly under a specified scenario. It cannot, by itself, validate real-world sensor accuracy or prove operational disaster prediction accuracy.

# 14. Security & Trust Architecture

## 14.1 Node-to-Master trust

Every field node must have a provisioned identity/secret. Telemetry is authenticated with an HMAC before entering the canonical ingestion pipeline. Invalid, missing, or tampered authentication is rejected and logged.

## 14.2 Citizen/authority event trust

Emergency events may be signed so local clients can verify authenticity and reject stale/replayed messages. Event IDs, timestamps, nonces/sequence values, and signature verification are part of the trust boundary.

## 14.3 Replay/tamper model

tampered packet
→ HMAC verification fails
→ reject
→ audit/security log

replayed packet
→ sequence/timestamp/idempotency checks
→ reject duplicate/stale delivery

invalid emergency event signature
→ reject locally
→ mark untrusted

## 14.4 Security scope

V1 uses practical prototype-grade security: per-node credentials, HMAC telemetry authentication, application authentication/authorization, signed emergency events, replay protection, and auditable operator actions. Full enterprise PKI, sophisticated zero-trust service meshes, and deep RBAC are roadmap items rather than V1 blockers.

# 15. Persistence, Events & Audit

## 15.1 Core entities

| Entity | Purpose |
| --- | --- |
| Node | Field device identity, configuration, network status. |
| Sensor | Sensor identity, type, calibration/diagnostic metadata. |
| Telemetry | Canonical measurement record. |
| Sensor Diagnostics | Health/quality inputs and diagnostics. |
| Hazard Assessment | Per-node/per-hazard reasoning output. |
| Incident | Persistent hazard situation. |
| Incident Event | Immutable change or occurrence in an incident lifecycle. |
| Spread Run | Geospatial fire simulation context and result. |
| Exposure Snapshot | Spatial exposure counts by zone and time. |
| Action | Authority response recommendation/approved action. |
| Alert | Canonical emergency communication event/delivery record. |
| Citizen | NexAlert user/contact entity. |
| SOS | Citizen distress request. |
| Simulation Scenario | Synthetic scenario control and metadata. |
| Audit Event | Operator/system action history. |

## 15.2 Incident vs event

An Incident persists across time. An Incident Event describes something that happened to that incident: created, state changed, spread updated, exposure increased, action approved, alert issued, response completed, or resolved.

## 15.3 Idempotency

Every canonical record that can be retried must have an identifier or deterministic key allowing duplicate detection. A duplicated telemetry packet must not create a duplicated anomaly event, incident, alert, or action.

## 15.4 Auditability

Operator overrides, response approvals, action changes, alert issuance/stand-down, simulation mode transitions, and system security failures must be auditable.

# 16. Technology & Deployment Boundaries

## 16.1 Technology posture

| Area | Primary direction | Reason |
| --- | --- | --- |
| ESP32 firmware | ESP-IDF + C/C++ | Predictable embedded control, networking, timing, memory behavior. |
| Reference intelligence / geospatial | Python | Fast iteration, scientific stack, testing, simulation. |
| Master software | Python services | Local compute and reusable backend logic. |
| Backend API | FastAPI-style Python backend | Typed API contracts and rapid development. |
| Database | PostgreSQL + PostGIS | Transactional state plus native spatial operations. |
| Frontend | React/Next.js + TypeScript | Componentized operational UI and strong tooling. |
| Styling | Tailwind/CSS | Controlled visual system; glassmorphism design system. |
| Real-time | WebSocket/SSE | Low-latency operational updates. |
| Simulation | Python | Scenario generation and scientific workflows. |
| Deployment | Modular monolith + workers; containerized where useful | Scope appropriate to a six-person SIH team. |

## 16.2 Python vs ESP32

Python is not copied wholesale onto the ESP32 as the default embedded architecture. Where lightweight intelligence is required on the device, the relevant mathematics is reimplemented in C/C++ against the same specification. The Python implementation is the reference implementation. Golden-vector tests verify equivalence within tolerance. This prevents the embedded version from becoming an undocumented fork.

## 16.3 Compute allocation

| Computation | ESP32 | Master | Cloud/backend |
| --- | --- | --- | --- |
| Sensor acquisition | YES | NO | NO |
| Basic diagnostics | YES | YES/aggregate | YES/aggregate |
| Heartbeat/buffering | YES | YES | YES |
| Lightweight embedded anomaly logic | OPTIONAL | YES | YES |
| Full intelligence chain | LIMITED | YES | YES |
| Fire spread raster simulation | NO | YES | YES |
| Population/infrastructure overlay | NO | YES | YES |
| Routing | NO | YES | YES |
| Authority workflow | NO | YES | YES |

# 17. Failure, Degraded-Mode & Recovery Model

| Failure | Expected behavior | User-visible condition |
| --- | --- | --- |
| Sensor dead | Hard-fail the sensor contribution; other evidence continues. | Sensor unhealthy; information may degrade. |
| Sensor drift | Lower quality/reliability or trigger baseline recovery path. | Evidence confidence may degrade. |
| Stale telemetry | Stop silently trusting it; mark stale and update information condition. | Stale/degraded indicator. |
| Contradictory sensors | Use reliability/agreement logic; do not force certainty. | Preliminary/unclear information. |
| Internet down | Local Master/node operation and buffering continue. | Cloud unavailable/degraded. |
| Master dead | Node/local gateway provides emergency page and buffers data. | Local emergency mode. |
| Cloud unavailable | Local domain remains useful; synchronize on return. | Cloud status unavailable. |
| Missing DEM | Use flat-terrain fallback only if explicitly configured and label it. | Terrain unavailable. |
| Missing wind | Use explicit calm/assumed wind policy; never silent stale substitution. | Wind assumption/degraded. |
| Missing population | Do not display zero; report data unavailable. | Exposure unavailable. |
| Invalid geometry | Attempt geometry validity repair; flag if repair fails. | Geometry warning. |
| No safe route | Return SHELTER_IN_PLACE. | No safe route available. |
| Invalid HMAC | Reject packet and log security event. | Rejected/unauthenticated. |
| Duplicate telemetry | Deduplicate without duplicate downstream events. | No visible duplication. |
| Fire reaches map boundary | Clip to domain and state that output is domain-limited. | Projection domain edge. |

# 18. Canonical Data-Flow Contracts

## 18.1 Telemetry

TelemetryRecord
  telemetry_id
  node_id
  sequence
  measurement_timestamp
  receive_timestamp
  location
  measurements[]
  diagnostics[]
  battery
  source
  schema_version
  auth

Ordering/deduplication uses receive_timestamp plus sequence semantics. Domain reasoning uses measurement_timestamp. This distinction is mandatory.

## 18.2 Hazard assessment

HazardAssessment
  node_id
  hazard_type
  evidence
  confidence
  severity
  operational_risk
  information_condition
  state
  contributing_evidence_groups[]
  timestamp

## 18.3 Incident

Incident
  incident_id
  hazard_type
  state
  severity
  information_condition
  origin
  contributing_nodes[]
  created_at
  updated_at
  current_geometry
  response_status

## 18.4 Canonical emergency event

EmergencyEvent
  alert_id
  incident_id
  hazard_type
  state
  severity
  information_condition
  affected_geometry
  recommended_action
  issued_at
  expires_at
  signature

## 18.5 Simulation metadata

Every synthetic event is permanently tagged with source=SIMULATION and the scenario ID. The authority and citizen surfaces must make simulation status impossible to confuse with a live event.

# 19. Architecture Invariants — NON-NEGOTIABLE

READ THIS SECTION BEFORE CODING These rules are the architectural guardrails. Any implementation that violates one of them is not considered conformant to the NexAlert V1 architecture.

1. Missing does not mean zero.

2. Measurement timestamp is not receive timestamp.

3. Receive timestamp + sequence govern ordering/deduplication; measurement timestamp governs domain reasoning.

4. Node state is not the same as regional incident state.

5. Sensor health is not battery health.

6. Signal quality is not hazard confidence.

7. Hazard evidence is not hazard probability unless separately calibrated.

8. Confidence is not severity.

9. Severity is not operational risk.

10. Operational risk is not probability.

11. Information Condition is separate from hazard severity and state.

12. Correlated sensors are grouped appropriately; they are not blindly treated as independent votes.

13. A core-evidence coverage floor prevents missing critical evidence from being hidden by total sensor count.

14. Synthetic telemetry MUST enter the same ingestion endpoint/pipeline as hardware telemetry.

15. The frontend MUST NOT calculate canonical intelligence or domain truth.

16. The simulation layer MUST NOT bypass canonical ingestion.

17. Incident correlation is deterministic and explicit in V1.

18. Physical fire footprint comes from the arrival-time field.

19. Operational danger buffers are policy outputs, not physics.

20. Current, warning, and projected exposure are not naively summed.

21. Multi-hazard risk surfaces remain separate and independently toggleable.

22. Real-world exposure numbers require real sourced data.

23. Fire spread runs on Master/cloud compute, not the ESP32.

24. Wind directions reported as FROM must be converted to TO before being used as propagation vectors.

25. Diagonal grid propagation uses the correct geometric distance.

26. Area/distance calculations use an appropriate projected/geodesic method, never raw lat/lon treated as planar metres.

27. Changing environmental conditions invalidate the untouched future portion of an old arrival field; recomputation begins from the current fire perimeter.

28. MultiPolygon geometry is supported.

29. Invalid geometry is repaired or explicitly flagged; it is not silently used.

30. NexAlert recommends; a human approves and acts. There is no autonomous dispatch path.

31. If no viable citizen safe route exists, the system returns SHELTER_IN_PLACE.

32. Spoofed/tampered node telemetry is rejected before domain ingestion.

33. Master failure and internet failure are treated as distinct failure modes.

34. Local emergency access must not depend exclusively on the Master being alive.

35. Operator and system actions that change operational state are auditable.

36. Simulation mode must always be visibly and persistently identified.

37. Do not introduce microservices, Kafka-class streaming, 3D, or heavy ML simply because they are technically possible; architecture complexity must earn its place.

# 20. V1 Scope, Non-Goals & Roadmap

## 20.1 V1 / prototype-critical

Real fire sensing and controlled trials.

Fire + flood as the primary multi-hazard pathways; other hazards supported at framework level where data allows.

Health/quality/reliability-aware intelligence.

Deterministic incident correlation.

Physics-informed geospatial fire spread with documented assumptions.

Affected-area geometry and hazard risk heatmap.

Real population/infrastructure exposure where displayed.

Human-approved response workflow.

Authority dashboard.

Citizen emergency experience and SOS.

Store-and-forward and local emergency access.

Node-to-Master HMAC.

Real + synthetic telemetry parity.

## 20.2 Explicitly deferred / roadmap

Full multi-hop self-healing mesh.

LoRaWAN/NB-IoT variants beyond the chosen prototype transport.

Satellite data integration.

Full CFD wildfire physics.

Ember spotting/reignition.

Advanced supervised ML without sufficient real labeled data.

Deep enterprise RBAC.

Microservices/Kafka-scale streaming.

3D terrain visualization.

Large-scale operational agency deployment certification.

Native mobile app unless core product is already stable and time remains.

## 20.3 Claims boundary

| Avoid saying | Say instead |
| --- | --- |
| AI predicts disasters. | Local intelligence detects developing hazards from real-time sensor evidence. |
| We predict exact fire spread. | We model probable spread direction/rate from terrain, fuel, moisture, and wind assumptions and update with changing conditions. |
| Risk is a probability. | The displayed risk is an operational index unless separately calibrated. |
| We detect every disaster. | We provide a multi-hazard framework with data-appropriate confidence; fire is the best-validated prototype pathway. |
| Every phone receives alerts. | People within the node's local network can receive local emergency information; subscribed online users can receive supported push alerts. |
| Self-healing network. | The prototype supports buffering and reconnection; full self-healing mesh is roadmap. |
| Production accurate. | Prototype-scale validation with clearly stated limits. |

# 21. Implementation Ownership Matrix

| Subsystem | Primary team owner | Depends on | Primary output |
| --- | --- | --- | --- |
| Firmware/Node | Hardware + embedded | Sensor contracts, security | Authenticated telemetry + local services |
| Intelligence | Software/AI | Telemetry + diagnostics | Hazard assessment |
| Incident Manager | Backend | Hazard assessment | Incident state |
| Fire/Geo | Software/Geo | Environmental data + incident | Spread + geometry + risk |
| Exposure | Software/Data | Geometry + datasets | Impact statistics |
| Response | Backend/Product | Incident + exposure + SOS | Recommended/approved actions |
| Communication | Backend/Systems | Event contracts | Delivery status |
| Authority UI | Frontend | Domain APIs/WebSocket | Operator experience |
| Citizen UI | Frontend/Product | Alert + routing APIs | Emergency experience |
| Simulation | Software/AI | Telemetry contract | Synthetic events |
| Validation | Whole team | All modules | Evidence of correctness |

# 22. End-to-End Example: Real Fire Event

## 22.1 Initial sensing

An ESP32 node observes a combination of temperature increase, PM/smoke increase, gas elevation, and supporting humidity conditions. It packages measurements, diagnostics, battery state, timestamps, sequence number, node identity, and HMAC.

## 22.2 Ingestion

The Master/backend authenticates the packet, validates schema and ranges, records measurement and receive timestamps, normalizes units, and deduplicates.

## 22.3 Intelligence

The intelligence layer evaluates sensor health, quality, reliability, baseline readiness, anomaly strength, and fire-specific evidence. Correlated evidence is grouped appropriately. The system may move the fire state to CONFIRMED or CRITICAL depending on the defined thresholds and evidence persistence.

## 22.4 Incident

The incident manager either attaches the assessment to an existing nearby fire incident or creates a new incident using the configured spatial/temporal correlation rule.

## 22.5 Geospatial response

The fire spread engine reads ignition location, DEM, fuel, moisture/context, and wind. Wind direction is converted from FROM to TO. Wind and slope influence are combined directionally. The engine propagates an arrival-time field and derives current, warning, and projection geometries.

## 22.6 Exposure

The exposure engine intersects the relevant geometries with population and infrastructure data and reports current/warning/projected exposure separately.

## 22.7 Response

The response engine sees the incident state, severity, risk, information condition, exposure, trend, and any SOS requests. It generates a categorical recommendation such as P1 / URGENT RESPONSE and identifies target organizations.

## 22.8 Human approval

An authority operator reviews the command card and approves or rejects the recommended action. Only after approval can the action move into notification/assignment states.

## 22.9 Citizen

A canonical emergency event drives the supported citizen alert channels. The citizen sees the hazard, severity, affected area, recommended action, route/safe-place guidance, and SOS.

## 22.10 Audit

The system records telemetry provenance, hazard state transitions, incident updates, spread calculations, exposure snapshots, alert issuance, operator approval, and response completion.

# 23. Architecture Implementation Gates

## 23.1 Mandatory pre-freeze execution items

| Gate | Required evidence |
| --- | --- |
| Gas sensor selection | Confirmed component, calibration approach, cross-sensitivity considerations, ESP32 compatibility. |
| ESP32 concurrency benchmark | Measured timing/jitter/resource behavior with sensing + Wi-Fi AP + telemetry + local processing. |
| Incident correlation | Automated tests for merge/non-merge cases. |
| Master-death resilience | Physical Master power-off test with local emergency page still reachable. |
| Human-approval response gate | Test that dispatch/notification action cannot proceed without approval. |
| Node HMAC | Invalid/missing/tampered/replayed packet tests. |
| Shelter-in-place | Synthetic all-directions-hazard test returns SHELTER_IN_PLACE. |
| Fire-spread corrections | FROM→TO, diagonal distance, vector wind/slope, projected CRS, recomputation behavior verified. |
| Geometry | No self-intersecting output; MultiPolygon and boundary cases verified. |
| Exposure | Sourced population/infrastructure data and no double-counting. |
| Simulation parity | Identical processing path proven for synthetic vs hardware-origin telemetry. |

## 23.2 Definition of architecture implemented

The architecture is considered implemented only when the team can trace a real or synthetic event through the canonical path without a parallel shortcut, can explain every state transition, can identify which subsystem owns each calculation, and can demonstrate the key failure behaviors listed in the final validation specification.

# Appendix A — Canonical Vocabulary

| Term | Canonical meaning |
| --- | --- |
| Telemetry | A time-stamped sensor/data record entering the canonical ingestion path. |
| Sensor Health | Condition of the sensor/device signal source. |
| Signal Quality | Quality/integrity/stability of the measurement signal. |
| Reliability | Trust weight applied to evidence after health/quality/context. |
| Anomaly | Deviation from an appropriate baseline or modality-specific expectation. |
| Evidence | Hazard-specific support derived from observed signals/features. |
| Confidence | How strongly the available evidence supports the assessment. |
| Severity | How severe the hazard condition is considered. |
| Operational Risk | Consequence/context-oriented index for operational prioritization. |
| Information Condition | Good/degraded/unknown view of the completeness and freshness of available information. |
| Hazard State | Normal/watch/suspected/confirmed/critical/resolved lifecycle. |
| Incident | Persistent representation of a real or simulated hazard situation. |
| Physical Footprint | Geometry derived from the arrival-time field or analogous physical hazard model. |
| Operational Buffer | Policy-defined safety margin around a physical footprint. |
| Exposure | Population/infrastructure/citizen consequence within a geometry. |
| Response Recommendation | System-generated suggested action for a human operator. |
| Approved Action | Operational action explicitly approved by an authorized human. |
| Emergency Event | Canonical alert payload derived from an incident/action state. |
| Simulation | Synthetic input/scenario using the same downstream pipeline as live data. |
| Degraded | System has reduced information or capability but continues with known limitations. |
| Unknown | System lacks sufficient information to make a trustworthy assessment. |

# Appendix B — Architecture at a Glance

NEXALERT
                             │
                  ┌──────────┴──────────┐
                  │                     │
              REAL DATA             SIMULATION
                  │                     │
                  └──────────┬──────────┘
                             ▼
                    CANONICAL TELEMETRY
                             │
                     AUTH + VALIDATION
                             │
                             ▼
              HEALTH → QUALITY → RELIABILITY
                             │
                 BASELINE → ANOMALY
                             │
               EVIDENCE → CONFIDENCE
                             │
                 SEVERITY → RISK → STATE
                             │
                    INCIDENT MANAGER
                             │
              ┌──────────────┼──────────────┐
              ▼              ▼              ▼
         FIRE/GEOSPATIAL  EXPOSURE       RESPONSE
              │              │              │
              └──────────────┼──────────────┘
                             ▼
                      CANONICAL EVENT
                             │
                   ┌─────────┴─────────┐
                   ▼                   ▼
              AUTHORITY             CITIZEN
             DASHBOARD             EXPERIENCE
                   │                   │
                   └─────────┬─────────┘
                             ▼
                           AUDIT
                             │
                           HISTORY

# Appendix C — Master Rule for Future Changes

When implementation begins, every new requirement should be classified before code is written: (1) clarification of an existing rule, (2) parameter change, (3) bug fix, (4) data limitation, or (5) genuine architecture change. Only category 5 changes the architecture itself. This prevents normal coding friction from reopening the entire product design.

Final architecture position The NexAlert architecture is now treated as frozen at the conceptual level. Remaining work is implementation, empirical validation, parameter calibration, and closing explicitly identified execution gaps. The team should now build against this document and the downstream final specifications rather than continuing open-ended architecture brainstorming.