NEXALERT

TECHNOLOGY STACK, DEPLOYMENT & COMPUTE ALLOCATION

Final V1 Engineering Baseline

Document role This specification locks the implementation technology families, runtime boundaries, compute allocation, repository/deployment posture, embedded-vs-Python intelligence strategy, data/runtime dependencies, and environment rules for NexAlert V1. It is subordinate to Document 01 (Master Architecture) and Document 02 (TRD), and it is used by the implementation/vibe-coding specification.

| Field | Value |
| --- | --- |
| Product | NexAlert |
| Problem | SIH26178 — resilient, AI-powered environmental monitoring and early warning |
| Document | 03 — Technology Stack, Deployment & Compute Allocation |
| Status | FINAL V1 TECHNOLOGY BASELINE |
| Primary hardware | ESP32-S3-class field node + Raspberry Pi-class Master |
| Primary software | C/C++ embedded + Python backend/intelligence/geospatial + TypeScript frontend |
| Data store | PostgreSQL + PostGIS |
| Deployment posture | Modular monolith + background workers; local/edge profile + cloud profile |

Versioning policy Technology families are locked here. Exact patch versions MUST be pinned in firmware/toolchain configuration, Python lock files, frontend lock files, container image digests/tags, and deployment manifests at implementation freeze. Do not silently upgrade a major dependency during implementation.

# 1. Technology Architecture Principles

## 1.1 Principle: one architecture, multiple compute targets

NexAlert is a distributed product, not a single process. The same domain concepts cross several runtimes, but each runtime has a bounded responsibility.

FIELD NODE
ESP32-S3 / C or C++
  sensors → diagnostics → local intelligence → HMAC → communication → local emergency gateway

MASTER / LOCAL EDGE
Raspberry Pi / Linux / Python
  aggregation → regional fusion → local backend → geospatial compute → local UI/service → buffering/sync

CLOUD / CENTRAL BACKEND
Linux / Python
  canonical APIs → persistence → workers → targeting → alerts → historical/audit

FRONTEND
Browser / TypeScript
  authority dashboard + citizen emergency experience

OPTIONAL NATIVE APP
Android/iOS endpoint only; same canonical event semantics

## 1.2 Principle: Python is the reference implementation; C/C++ is the embedded implementation

The environmental intelligence mathematics is specified independently of programming language. Python is the reference implementation used for rapid development, simulation, validation and server-side execution. Only the lightweight subset explicitly assigned to the ESP32 is reimplemented in C/C++ for deterministic embedded execution. The C/C++ implementation is verified against Python using shared golden-vector fixtures.

Do not copy Python code blindly onto the ESP32 MicroPython can run on ESP32-class devices, but NexAlert V1 does not depend on it. The default embedded architecture is ESP-IDF + C/C++ because the node must simultaneously handle sensors, networking, buffering, authentication, timing and local intelligence with predictable resource usage.

# 2. Final Technology Stack

| Layer | Technology / family | V1 lock | Notes |
| --- | --- | --- | --- |
| MCU firmware | ESP-IDF + C/C++ | LOCKED | Official Espressif development framework; C/C++ application. |
| Embedded RTOS | FreeRTOS through ESP-IDF | LOCKED | Use tasks/queues/event groups/timers only where needed; avoid unnecessary concurrency. |
| Embedded crypto | ESP-IDF crypto/security facilities + vetted component/library | LOCKED | HMAC telemetry authentication; secure key handling. |
| Embedded DSP | ESP-DSP where useful | OPTIONAL/APPROVED | Use only where signal-processing workload justifies it. |
| Master OS | Linux on Raspberry Pi-class computer | LOCKED | Local compute and service host. |
| Master/backend language | Python | LOCKED | Same domain modules can run locally and in cloud profile. |
| Backend framework | FastAPI | LOCKED | REST + WebSocket/SSE-facing API layer. |
| Validation/model schemas | Pydantic-style typed schemas | LOCKED | Canonical API/data validation. |
| Scientific stack | NumPy / SciPy-compatible Python stack | LOCKED | Mathematical calculations and simulation. |
| Geospatial Python | GeoPandas / Shapely / Rasterio-class stack | LOCKED | Vector/raster preparation and geometry operations. |
| Spatial database | PostgreSQL + PostGIS | LOCKED | Canonical spatial persistence/query layer. |
| Task execution | Python worker process/task queue | LOCKED CONCEPT | Specific lightweight worker implementation chosen during build; no Kafka-class infrastructure. |
| Frontend | Next.js + React + TypeScript | LOCKED | App Router preferred; strongly typed UI. |
| Styling | Tailwind CSS + CSS custom properties | LOCKED | Dark grey/black glassmorphism design system. |
| Map | MapLibre GL JS | LOCKED | Interactive WebGL map; backend-provided GeoJSON/raster/vector sources. |
| Charts | Lightweight React chart library | APPROVED | Exact library chosen during UI implementation; avoid unnecessary dependencies. |
| Real-time | WebSocket; SSE fallback where appropriate | LOCKED | Dashboard/citizen live updates. |
| API client | Typed frontend API layer | LOCKED CONCEPT | Prefer generated/hand-maintained types from canonical schema. |
| Containerization | Docker + Docker Compose for development/demo | LOCKED | Local reproducible stack. |
| Reverse proxy | Nginx/Caddy or platform ingress | DEPLOYMENT CHOICE | Use only when needed; simple demo can expose app directly behind secure ingress. |
| Testing | pytest + frontend test framework + firmware unit/integration tests | LOCKED | Tests traced to TRD requirements. |
| CI | GitHub Actions or equivalent | SHOULD | Lint/test/build on push; exact provider is not architecture-critical. |

The MapLibre choice is appropriate for the interactive map requirements: its official documentation supports GeoJSON polygons, raster/COG-style sources, realtime updates, layer interactivity and browser rendering through WebGL. citeturn169935search1turn169935search3

PostGIS extends PostgreSQL with spatial storage, indexing and functions for geometry measurement/intersection/buffering and raster support, matching NexAlert's spatial data needs. citeturn169935search5turn169935search6

# 3. Compute Allocation — What Runs Where

## 3.1 Allocation matrix

| Workload | ESP32-S3 | Raspberry Pi Master | Cloud/backend |
| --- | --- | --- | --- |
| Sensor acquisition | PRIMARY | NO | NO |
| Sensor self-test/diagnostics | PRIMARY | Aggregate | Aggregate |
| Battery/solar monitoring | PRIMARY | Aggregate | Aggregate |
| Heartbeat | PRIMARY | Track | Track |
| Basic filtering | PRIMARY | Optional | Optional |
| Signal quality | PRIMARY | Recompute/verify if required | Recompute/verify if required |
| Sensor health | PRIMARY | Aggregate/track | Aggregate/track |
| Reliability | PRIMARY | Aggregate/track | Aggregate/track |
| Baseline readiness | PRIMARY | Read/aggregate | Read/aggregate |
| Short-window anomaly | PRIMARY | YES | YES |
| Hazard evidence | PRIMARY for configured lightweight hazards | YES / regional fusion | YES |
| Evidence confidence | PRIMARY | Regional enrichment | Regional enrichment |
| Edge severity | PRIMARY | Regional enrichment | Regional enrichment |
| Edge operational risk | PRIMARY | Regional enrichment | Regional enrichment |
| Local hazard state/action | PRIMARY | Regional state | Regional state |
| Cross-node fusion | NO | PRIMARY | YES |
| Incident correlation | NO | PRIMARY | PRIMARY |
| Fire spread raster simulation | NO | PRIMARY | PRIMARY |
| DEM processing | NO | PRIMARY | PRIMARY |
| Fuel raster handling | NO | PRIMARY | PRIMARY |
| Risk heatmap generation | NO | PRIMARY | PRIMARY |
| Population/infrastructure exposure | NO | PRIMARY | PRIMARY |
| Routing | NO | PRIMARY | PRIMARY |
| Authority workflow | NO | PRIMARY | PRIMARY |
| Cloud history/audit | NO | Local buffer only | PRIMARY |

## 3.2 Rule for edge AI

The ESP32 is an autonomous sensing/reasoning endpoint, but it is not a miniature GIS server. The edge AI boundary stops before heavy geospatial computation. This supports the Qualcomm/edge-intelligence framing without forcing scientifically inappropriate workloads onto a constrained MCU.

## 3.3 Rule for Master vs cloud

The Master and cloud use the same domain model where practical. The Master is the local continuity profile; the cloud is the centralized persistence and broad-scale coordination profile. A network outage must not require rewriting domain logic or creating a special dashboard truth.

# 4. ESP32-S3 Firmware Architecture

## 4.1 Why ESP-IDF + C/C++

ESP-IDF is Espressif's official development framework for ESP32 devices and explicitly supports C++ applications. The ESP32-S3 documentation covers C++ support, FreeRTOS-backed threading, toolchain behavior, and device-specific development. citeturn424934search0turn424934search8

## 4.2 Firmware module structure

firmware/
  main/
    app_main.cpp
  components/
    sensors/
      sensor_manager.*
      temp_sensor.*
      humidity_sensor.*
      pm_sensor.*
      gas_sensor.*
      water_level_sensor.*
      rainfall_sensor.*
      soil_sensor.*
      vibration_sensor.*
    diagnostics/
      self_test.*
      health.*
      signal_quality.*
    intelligence/
      baseline.*
      anomaly.*
      hazard_reasoning.*
      confidence.*
      severity.*
      risk.*
      state_machine.*
    telemetry/
      telemetry_model.*
      encoder.*
      validator.*
      sequence.*
    security/
      hmac_auth.*
      key_store.*
      replay_guard.*
    network/
      transport.*
      reconnect.*
      wifi_ap.*
      optional_radio_adapter.*
    storage/
      ring_buffer.*
      flash_store.*
    system/
      watchdog.*
      time_sync.*
      config.*
      logging.*
  test/
  sdkconfig.defaults
  CMakeLists.txt

## 4.3 Firmware task model

Task: sensor_task
  acquire measurements on configured cadence
        ↓
Task: diagnostics_task
  update self-test / stability / availability
        ↓
Task: intelligence_task
  update baseline / anomaly / hazard reasoning
        ↓
Task: telemetry_task
  serialize canonical packet → HMAC → queue/send
        ↓
Task: network_task
  connection/reconnect + local AP service
        ↓
Task: storage_task
  buffer unsent telemetry/events

The exact task decomposition is subject to the ESP32 concurrency benchmark. Do not create a task per sensor by default. Prefer a small bounded task set and queues to prevent unnecessary scheduling overhead.

# 5. Embedded Intelligence Strategy

## 5.1 Same specification, two implementations

| Function | Python reference | ESP32 C/C++ |
| --- | --- | --- |
| Normalization | Reference | Equivalent deterministic implementation |
| Health H_i | Reference | Implemented where diagnostics exist locally |
| Signal quality Q_i | Reference | Implemented |
| Reliability R_i | Reference | Implemented |
| Baseline readiness B_i | Reference | Implemented |
| Median/MAD or selected baseline | Reference | Lightweight implementation |
| Anomaly A_i | Reference | Implemented |
| Hazard reasoning | Reference | Implemented for configured lightweight hazards |
| Confidence | Reference | Implemented |
| Severity | Reference | Implemented |
| Operational risk | Reference | Implemented |
| Hazard state machine | Reference | Implemented |
| Fire spread GIS | Reference/server | NOT ON ESP32 |
| Population/infrastructure | Reference/server | NOT ON ESP32 |

## 5.2 Golden-vector validation

For every embedded mathematical function that must match the Python reference, create a fixture containing inputs, expected intermediate values where useful, final output and tolerance.

golden_vectors/
  health/
  quality/
  reliability/
  baseline/
  anomaly/
  fire_reasoning/
  confidence/
  severity/
  risk/
  state_machine/

Example:
input:
  temperature_c = 42.4
  baseline = 31.2
  scale = 3.4
expected:
  z = ...
  anomaly = ...
python_result = ...
esp32_result = ...
assert abs(python_result - esp32_result) <= tolerance

Definition of parity Parity does not mean bit-for-bit equality for every floating-point computation. It means the embedded implementation stays within an explicitly declared numerical tolerance and produces the same qualitative state transition under the same test vector.

# 6. Raspberry Pi Master Architecture

## 6.1 Master role

The Raspberry Pi-class Master is the local control/compute center. It receives node telemetry, can run the complete Python intelligence stack, performs cross-node fusion and incident correlation, hosts heavy geospatial services, exposes the local dashboard/API, supports local citizen access where configured, and synchronizes with the cloud.

## 6.2 Master software profile

Linux
  ↓
Python runtime
  ↓
FastAPI application
  ├── ingestion
  ├── intelligence
  ├── hazard reasoning
  ├── incident manager
  ├── geospatial/fire spread
  ├── exposure
  ├── response
  ├── alerts
  ├── SOS
  ├── simulation
  └── audit

PostgreSQL + PostGIS
  ↓
background workers
  ↓
local UI / local API clients
  ↓
optional cloud sync

## 6.3 Local-first requirement

The Master profile must be deployable without cloud access. The local instance should be able to ingest node data, run essential intelligence, maintain incident state, and expose the authority UI locally. Cloud-specific functions should fail gracefully rather than blocking the core local safety loop.

# 7. Backend Software Architecture

## 7.1 Modular monolith

NexAlert V1 uses a modular monolith. Domain modules are separated in code and APIs, but they run in one deployable backend process/profile plus background workers. Do not prematurely split them into microservices.

backend/
  app/
    api/
    core/
    config/
    ingestion/
    security/
    intelligence/
    hazards/
    incidents/
    geospatial/
      terrain/
      fire_spread/
      affected_area/
      risk/
      exposure/
    response/
    alerts/
    sos/
    simulation/
    users/
    audit/
    workers/
    repositories/
    models/
    schemas/
    services/
  tests/
    unit/
    integration/
    system/
    fixtures/

## 7.2 Recommended Python environment

| Area | Choice | Rule |
| --- | --- | --- |
| Python | Current stable compatible Python; freeze exact version in project configuration | As of Sep 2026, Python 3.14 is a stable release; pin exact minor/patch for the project after compatibility testing. citeturn424934search4 |
| API | FastAPI | Use typed request/response schemas and explicit API versioning. |
| Schema validation | Pydantic-style models | One canonical schema per domain object. |
| ORM/data access | SQLAlchemy-style typed data access | Avoid database logic in route handlers. |
| Migrations | Alembic-style migrations | Every schema change is versioned. |
| Geospatial | Shapely/GeoPandas/Rasterio-class tools | Use PostGIS for persistent/query-heavy spatial operations. |
| Math | NumPy/SciPy-class tools | Keep deterministic reference math testable. |
| HTTP client | httpx-style async/sync client | External services have timeout/retry policies. |
| Testing | pytest | Every TRD requirement maps to a test where possible. |

FastAPI's official deployment guidance treats production deployment as running the API behind a suitable server/deployment setup rather than the development server; the NexAlert deployment specification follows that separation. citeturn424934search2

# 8. Frontend Technology & Application Architecture

## 8.1 Framework

Use Next.js with React and TypeScript for both authority and citizen web applications, either as two route groups in one frontend repository or two apps sharing a component package. The default recommendation for V1 is one frontend codebase with explicit authority and citizen route boundaries to reduce duplication.

Next.js is a React framework for full-stack web applications, and its official documentation supports the App Router approach; React's documentation supports first-class TypeScript usage. citeturn169935search0turn424934search6

## 8.2 Frontend structure

frontend/
  app/
    authority/
      overview/
      incidents/
      fire-spread/
      affected-impact/
      multi-hazard/
      nodes/
      telemetry/
      sos/
      alerts/
      history/
      response/
      audit/
      system/
    citizen/
      page.tsx
    api/
  components/
    map/
    command/
    incidents/
    alerts/
    sos/
    charts/
    glass/
  lib/
    api/
    websocket/
    geo/
    formatting/
    auth/
    event-verification/
  styles/
    tokens.css
    glass.css
  types/
  tests/

## 8.3 Rendering rule

Server-side/data-fetching approaches MAY be used where appropriate, but interactive maps and live operational components are client-side components.

The frontend MUST receive canonical domain outputs from the backend.

The frontend may format, filter and animate but must not recalculate hazard state, risk, fire spread, exposure or response priority.

Keep raw technical values in an optional advanced view rather than the primary citizen experience.

# 9. Geospatial Frontend Stack

## 9.1 MapLibre

MapLibre GL JS is the locked web mapping engine. It supports interactive maps, GeoJSON sources/layers, raster visualization, realtime feature updates and browser rendering. citeturn169935search1turn169935search3

## 9.2 Map architecture

Backend geospatial output
  ├── GeoJSON current footprint
  ├── GeoJSON warning zone
  ├── GeoJSON projection
  ├── GeoJSON operational buffer
  ├── GeoJSON safe route
  ├── node/incident/SOS points
  └── raster/derived risk source
        ↓
MapLibre sources
        ↓
named layers
        ↓
layer controls
        ↓
click / hover / drill-down

## 9.3 Map layer ownership

| Layer | Backend source | UI semantics |
| --- | --- | --- |
| Physical footprint | Arrival-time-derived geometry | Physical modeled area |
| Operational buffer | Policy-derived buffer | Operational caution; not physics |
| Warning zone | Arrival-time threshold | Projected near-term reach |
| Projection | Arrival-time threshold | Longer-horizon modeled reach; assumption-labeled |
| Risk heatmap | Hazard Risk Surface | Continuous operational index |
| Nodes | Node registry | Sensor/network status |
| Incidents | Incident store | State + severity cues |
| SOS | SOS store | Categorical priority |
| Population | Exposure source | Context/exposure |
| Infrastructure | OSM/other sourced data | Impact context |

## 9.4 Visual safety rule

Do not render the footprint, operational buffer, warning zone, projection and heatmap with equally opaque fills. Use layer hierarchy, transparency, labels and toggles so that the map remains interpretable.

# 10. Database & Storage Architecture

## 10.1 PostgreSQL + PostGIS

PostgreSQL is the canonical relational store. PostGIS provides spatial types/indexes/functions needed for nodes, incidents, polygons, routes, exposure and map queries. citeturn169935search5turn169935search6

## 10.2 Core logical tables

| Table/entity | Key contents |
| --- | --- |
| nodes | node_id, identity, location, firmware, connectivity, status |
| sensors | sensor_id, node_id, type, calibration/configuration |
| telemetry | telemetry_id, node_id, timestamps, measurements, source, auth metadata |
| sensor_diagnostics | health inputs, quality inputs, diagnostic timestamps |
| hazard_assessments | node/hazard, evidence, confidence, severity, risk, state, information condition |
| incidents | incident_id, hazard, state, origin, geometry, contributing nodes, timestamps |
| incident_events | immutable lifecycle/event records |
| spread_runs | incident, environmental context, grid/domain, run metadata |
| spread_outputs | arrival field references, geometry/risk output references |
| exposure_snapshots | incident, zone, population, citizens, roads, infrastructure |
| actions | recommendation, approval, target, status, reason |
| alerts | canonical emergency event, targeting/delivery state, signature |
| citizens | NexAlert user identity + consent metadata |
| sos | citizen distress, location, priority, status |
| safe_places | candidate destination geometry, availability, accessibility |
| simulation_scenarios | scenario configuration and status |
| simulation_runs | run metadata and source identifiers |
| audit_events | actor, action, target, reason, timestamp |

## 10.3 Raster storage policy

Very large raster assets such as DEMs, population grids and arrival-time surfaces should not be naïvely stored as large JSON blobs. Keep metadata and references in PostgreSQL/PostGIS and use a file/object storage layout for immutable raster assets where appropriate. The specific raster format and storage backend will be selected during the geospatial implementation, but the data contract must remain stable.

# 11. Communication & Transport Technology

## 11.1 Transport abstraction

NexAlert separates the physical transport from the application data contract. The telemetry record is independent of whether it arrived over Wi-Fi/IP, an optional LoRa-class link, or another supported local transport.

Transport Adapter
  ├── WiFi/IP adapter
  ├── Optional LoRa/raw-radio adapter
  └── Future adapter
        ↓
Canonical Telemetry Envelope
        ↓
Authentication / validation
        ↓
Ingestion

## 11.2 V1 practical transport

For the SIH prototype, use IP-based Wi-Fi/HTTP(S) or local TCP/IP connectivity where available because it simplifies the direct real-hardware demo, local emergency portal, telemetry API and debugging. A long-range radio adapter may coexist when the selected hardware requires it. No LoRaWAN/NB-IoT network deployment is required for V1.

## 11.3 Application protocols

| Path | Protocol | Use |
| --- | --- | --- |
| Node → Master | HTTPS/HTTP or MQTT-over-IP depending on chosen adapter | Telemetry upload; schema/authenticated payload. |
| Master → Cloud | HTTPS REST + outbound sync/WebSocket as needed | Synchronization and central services. |
| Dashboard ↔ backend | HTTPS REST + WebSocket | Queries and live updates. |
| Citizen ↔ backend | HTTPS REST + WebSocket/SSE | Emergency page and updates. |
| Citizen ↔ local node/gateway | HTTP over local Wi-Fi | Offline emergency page. |
| Outbound notification | Web Push / optional FCM/APNs later | Subscribed online citizens. |

## 11.4 Canonical event independence

The emergency event object is transport-neutral. Delivery services adapt the transport envelope, not the hazard semantics.

# 12. Deployment Profiles

## 12.1 Development profile

Developer machine
  Docker Compose
    ├── backend
    ├── worker
    ├── postgres/postgis
    ├── frontend
    └── optional local tile/data helper

ESP32
  ↕
Developer LAN / test network

Docker Compose provides a reproducible way to define and run multiple development/test containers, networks and volumes from a YAML configuration. citeturn169935search8

## 12.2 Master/field demo profile

Raspberry Pi
  ├── backend
  ├── worker
  ├── PostgreSQL + PostGIS
  ├── local frontend/static assets or local frontend host
  └── local emergency gateway services

ESP32 nodes
  ↓
local transport
  ↓
Raspberry Pi
  ↓
local dashboard / citizen portal
  ↓
optional cloud sync

## 12.3 Cloud profile

Managed/VM Linux environment
  ├── reverse proxy / TLS
  ├── backend
  ├── workers
  ├── PostgreSQL + PostGIS
  ├── object/raster storage
  └── optional notification service

## 12.4 Single-machine SIH fallback

A full cloud deployment is not required for the demonstration. A single Raspberry Pi or developer machine may host the backend, worker, database and frontend for demonstration, provided the interfaces remain separated in code and the local/offline behavior remains testable.

# 13. Environment & Configuration Management

## 13.1 Environments

| Environment | Purpose |
| --- | --- |
| local-dev | Fast development; mock/external services optional; synthetic data. |
| integration | Full backend + DB + frontend + simulation + selected hardware path. |
| master-demo | Raspberry Pi local deployment with field nodes and local gateway. |
| cloud-demo | Public/remote demo environment with real TLS and restricted access. |
| test | Automated unit/integration/system test profile. |

## 13.2 Required configuration groups

Database connection and migration settings.

API host/port and CORS/origin settings.

Node credentials/key references.

Telemetry freshness/timeouts.

Intelligence parameters and baseline windows.

Fire/geospatial grid resolution and projection horizon.

Risk/response thresholds.

Alert providers.

Map style/source configuration.

Dataset paths and provenance identifiers.

Simulation scenario paths.

Logging and audit settings.

No secrets in source control HMAC node secrets, signing private keys, database passwords, push credentials and API secrets MUST be injected through environment/secret management. Frontend bundles must never contain server-side secrets.

# 14. Build, Packaging & Versioning

## 14.1 Firmware

Use ESP-IDF's CMake/Ninja-based build flow and lock the target to the actual ESP32-S3 board configuration. Keep sdkconfig defaults under version control. Use a reproducible toolchain/environment for CI where practical. Espressif's ESP32-S3 getting-started documentation explicitly uses ESP-IDF, toolchain, CMake and Ninja as the development workflow. citeturn424934search9

## 14.2 Python

pyproject.toml
  ↓
locked Python version
  ↓
locked dependencies
  ↓
unit/integration tests
  ↓
container image

## 14.3 Frontend

package.json
package-lock.json / equivalent lockfile
  ↓
Node.js version file
  ↓
npm/pnpm/yarn chosen once
  ↓
typecheck
lint
tests
build

## 14.4 API versioning

All public application APIs start at /api/v1. Domain schemas carry schema_version when a persistent/transport-level version is required. Backward-incompatible changes require a deliberate new version.

# 15. Performance & Resource Budgeting

## 15.1 Principle

No performance target should be invented merely to make the product sound fast. Requirements are measured on the actual ESP32 and Raspberry Pi configuration.

## 15.2 ESP32 benchmark

| Metric | Measure |
| --- | --- |
| Sensor acquisition latency | Time from sampling trigger to normalized reading. |
| Telemetry serialization | Time to construct canonical packet. |
| HMAC overhead | Time and CPU cost per packet. |
| Local intelligence | Time per evaluation cycle. |
| Wi-Fi AP responsiveness | Connection/HTTP response under load. |
| Packet delivery | Loss/latency/jitter under representative load. |
| Memory | Free heap and high-water marks under combined workload. |
| CPU | Task utilization under combined workload. |
| Power | Relative consumption in idle/active/communication states where measured. |

## 15.3 Master benchmark

| Workload | Measure |
| --- | --- |
| Telemetry ingestion | Packets/sec sustainable without queue growth. |
| Intelligence | Observations/sec and processing latency. |
| Incident correlation | Reports/sec and query latency. |
| Fire spread | Simulation wall time for demo grid/horizon. |
| Polygonization | Geometry derivation time. |
| Risk surface | Raster computation time. |
| Exposure | Population/infrastructure overlay time. |
| WebSocket updates | Event-to-client latency. |

## 15.4 Browser benchmark

Initial page load and map initialization.

Time from backend event to visible update.

Map layer update/render responsiveness for the demo geometry.

Memory behavior during fire-spread replay.

Degraded behavior on slow network or low-end device.

# 16. Security Architecture by Technology Boundary

| Boundary | Mechanism | Required behavior |
| --- | --- | --- |
| Node → Master | Per-node secret + HMAC | Reject unauthenticated/tampered telemetry. |
| Master → cloud | TLS + authenticated client/service path | Protected synchronization. |
| Backend API | Application authentication/authorization | Protect authority actions and data. |
| Emergency event | Digital signature / trusted public key | Local verification of authenticity. |
| Replay | Sequence + timestamp + event ID/nonce | Reject duplicates/stale replay. |
| Database | Credentials + least-privileged service user | No public DB exposure. |
| Frontend | No embedded private secrets | Use server-side secure APIs. |
| Logs | Structured + access-controlled | Do not log raw secrets or unnecessary citizen data. |

## 16.1 Key storage

Node credentials are provisioned per device. Private signing keys remain server-side/local-gateway protected and are never shipped to ordinary browser clients. Public verification keys may be distributed to citizen clients for event verification.

## 16.2 Threat assumptions

A malicious device can attempt to inject telemetry.

A malicious Wi-Fi network can imitate an SSID.

A network attacker can replay captured messages.

A browser can be offline or compromised; the backend remains authoritative.

Physical node compromise is possible; V1 focuses on protocol-level identity and integrity.

# 17. Logging, Monitoring & Debugging

## 17.1 Structured logging

All server-side modules MUST emit structured logs with timestamp, service/module, event type, correlation/incident ID where applicable, severity and useful diagnostic context.

## 17.2 Debug view

A simple internal developer/operator debug view is required for demo reliability. It should expose the raw-to-decision chain without making it the normal authority UX:

Telemetry
 ↓
Health H_i
 ↓
Quality Q_i
 ↓
Reliability R_i
 ↓
Baseline state
 ↓
Anomaly A_i
 ↓
Hazard evidence E_h
 ↓
Confidence C_h
 ↓
Severity S_h
 ↓
Operational risk R_h
 ↓
Hazard state
 ↓
Incident
 ↓
Spread / Exposure / Response

## 17.3 Correlation identifiers

A single incident should be traceable across telemetry processing, incident events, spread runs, exposure snapshots, response actions and alerts through incident_id and linked event/action/alert IDs.

# 18. External Data & Dataset Runtime Handling

## 18.1 Data is a dependency, not a hardcoded asset

Geospatial and environmental datasets must be loaded through configuration and recorded with provenance. Demo datasets should be pre-cached for repeatability.

## 18.2 Categories

| Data | Examples / purpose | Runtime treatment |
| --- | --- | --- |
| DEM | Terrain slope/aspect | Preprocessed/cached region; versioned. |
| Fuel | Simplified demo fuel classes | Preprocessed raster/config; clearly labeled. |
| Weather | Wind/environment context | Live adapter or controlled demo input; source labeled. |
| Rainfall | Flood and moisture context | Live/history adapter or scenario input. |
| Population | WorldPop/GHSL-class source | Precached raster with provenance. |
| Infrastructure | OSM-class source | Preprocessed/cached vectors. |
| Fire observation | Satellite/other external context | Validation/context, not unquestioned ground truth. |

## 18.3 Source provenance record

dataset_id
source_name
source_url_or_identifier
version/date
download_timestamp
spatial_extent
CRS
processing_version
license/usage_note
checksum (where practical)

# 19. Simulation Runtime Architecture

## 19.1 Simulator as telemetry producer

The simulator runs as a Python process or worker. It generates canonical telemetry and submits it to POST /api/v1/telemetry or the equivalent internal ingestion interface. It MUST NOT call the incident manager, risk engine or dashboard state directly.

## 19.2 Scenario engine

Scenario
  ├── source = SIMULATION
  ├── start/end
  ├── node set
  ├── baseline conditions
  ├── hazard progression
  ├── sensor correlations
  ├── faults
  ├── environmental context
  └── private evaluation ground truth (never sent to intelligence)
        ↓
Telemetry generator
        ↓
Canonical telemetry
        ↓
REAL INGESTION PIPELINE

## 19.3 LIVE/SIMULATION parity

The only permitted semantic difference is source metadata/scenario controls. Domain calculations are shared. Any simulation-only shortcut is an architecture violation unless explicitly classified as a presentation-only test harness.

# 20. Detailed Network & Deployment Topology

┌─────────────────────┐
                          │   CLOUD BACKEND      │
                          │ FastAPI + Workers    │
                          │ PostgreSQL/PostGIS   │
                          └──────────▲──────────┘
                                     │
                               HTTPS / Sync
                                     │
                     ┌───────────────┴───────────────┐
                     │                               │
             ┌───────┴────────┐             ┌────────┴─────────┐
             │  MASTER / EDGE │             │ Authority        │
             │ Raspberry Pi   │             │ Browser          │
             │ Python + DB    │             │ Next.js/React    │
             │ Geospatial     │             └──────────────────┘
             └───────▲────────┘
                     │
               Local transport
                     │
          ┌──────────┴───────────┐
          │                      │
   ┌──────┴──────┐        ┌──────┴──────┐
   │ ESP32 Node  │  ...   │ ESP32 Node  │
   │ Sensors     │        │ Sensors     │
   │ Edge AI     │        │ Edge AI     │
   │ HMAC        │        │ HMAC        │
   └──────▲──────┘        └──────▲──────┘
          │                      │
          └──── local environment ┘

Citizen online → Cloud/Web Push + Web UI
Citizen offline → Node/local gateway Wi-Fi → local emergency UI

## 20.1 Local traffic

Keep local sensor-to-Master traffic inside the local network/transport and do not require a public cloud round-trip for basic edge intelligence.

## 20.2 Cloud traffic

Cloud connectivity is used for central persistence, broad-area coordination, subscribed online citizen delivery, remote authority access and synchronization—not as a prerequisite for edge hazard reasoning.

# 21. Technology Failure Behavior

| Failure | Technology behavior | System behavior |
| --- | --- | --- |
| ESP32 reboot | Watchdog/reboot, persistent config restored | Node returns with status; missing intervals visible. |
| Node storage full | Bounded ring buffer/eviction policy or backpressure | Oldest buffered data handled by policy; no memory blow-up. |
| Wi-Fi link lost | Reconnect; buffer | Store-and-forward. |
| Internet lost | Master remains local | Cloud state delayed; local functions continue. |
| Master dead | Local node/gateway path remains available | Citizen local page and buffering survive to the degree designed. |
| Cloud down | Retries bounded | Master/local operation continues. |
| Postgres unavailable | Queue/retry critical writes | No silent state loss; service degrades. |
| Worker crash | Supervisor/restart | Core API remains alive where possible. |
| Frontend map fails | Render text/status and cached data | Emergency action remains visible. |
| Invalid event | Reject/flag | No trusted UI rendering. |
| Simulation process dies | Scenario stops cleanly | Existing live system unaffected. |

## 21.1 Bound the blast radius

A failure in one module must not cascade into a total system crash. For example, failure of population data must not stop fire detection; failure of the map renderer must not stop the emergency action message; failure of cloud sync must not stop local edge intelligence.

# 22. Technology Invariants

🔒 ESP32 V1 firmware uses ESP-IDF + C/C++ as the default embedded environment.

🔒 Python is the reference implementation for shared intelligence mathematics.

🔒 C/C++ embedded intelligence is verified with golden vectors against Python.

🔒 Fire-spread geospatial simulation does not run on the ESP32.

🔒 Population/infrastructure overlay does not run on the ESP32.

🔒 Raspberry Pi Master and cloud backend share domain concepts and canonical contracts.

🔒 V1 backend is a modular monolith plus background workers, not microservices.

🔒 PostgreSQL + PostGIS is the canonical spatial datastore.

🔒 Next.js + React + TypeScript is the web frontend baseline.

🔒 MapLibre GL JS is the map engine.

🔒 Frontend does not own canonical hazard, risk, spread, exposure or response calculations.

🔒 Simulation uses the same telemetry ingestion interface as real data.

🔒 Docker Compose is used for reproducible local/demo service orchestration where useful.

🔒 Exact dependency versions are pinned; unreviewed major-version upgrades are prohibited during implementation.

🔒 Secrets never live in source code or browser bundles.

🔒 No cloud dependency is allowed inside the essential local intelligence path.

🔒 No new infrastructure technology is added merely for perceived sophistication.

🔒 If an implementation change alters a technology boundary, update this document and the Master Architecture before proceeding.

# 23. Technology Decision Matrix — What We Deliberately Did Not Choose

| Candidate | Decision | Reason |
| --- | --- | --- |
| MicroPython on ESP32 | NOT DEFAULT | Technically possible, but V1 requires predictable embedded concurrency and low-level control; use C/C++ under ESP-IDF. |
| C++ fire-spread engine on ESP32 | NO | Wrong compute target; heavy geospatial/raster workload belongs on Master/cloud. |
| C++ backend | NO | Python is materially better for the selected scientific/geospatial/backend stack and rapid iteration. |
| Microservices | NO | Operational complexity is unjustified for a six-person SIH team. |
| Kafka-class streaming | NO | No V1 scale requirement justifies it. |
| Kubernetes | NO | No need for cluster orchestration in prototype. |
| 3D/WebGL terrain engine | NO | Explicitly deferred; 2D geospatial command view is the required product. |
| Vector-tile backend | DEFERRED | Use GeoJSON/appropriate raster sources for demo-scale data; optimize later if datasets require. |
| Native app | DEFERRED | Web/local citizen path is the V1 product. |
| Satellite live dependency | DEFERRED | External evidence remains additive, not required for local decisions. |
| Advanced ML for fire | NO V1 | Real data volume does not justify a trained classifier; deterministic/calibrated reasoning is locked. |

## 23.1 Technology choices can change only for evidence

A technology may be reconsidered only when it fails a measurable requirement, introduces an unacceptable security/reliability problem, or becomes unavailable. Popularity, novelty, or a desire for more buzzwords is not sufficient justification.

# 24. Implementation Order Driven by Technology Dependencies

| Phase | Build | Exit condition |
| --- | --- | --- |
| 1 | Repo + Python environment + frontend scaffold + firmware scaffold | All projects build cleanly. |
| 2 | Postgres/PostGIS + migrations + canonical schemas | Database initializes reproducibly. |
| 3 | ESP32 telemetry + heartbeat + HMAC + buffering | Real packet reaches ingestion reliably. |
| 4 | Python ingestion + validation + idempotency | Hardware + simulation packet path works. |
| 5 | Intelligence reference implementation | TRD intelligence tests pass. |
| 6 | ESP32 intelligence subset + golden vectors | Embedded parity verified. |
| 7 | Incident manager | Merge/split and lifecycle tests pass. |
| 8 | Fire geospatial engine | Core spread tests pass. |
| 9 | Exposure + risk | Sourced demo data produces reproducible outputs. |
| 10 | Response/alerts/SOS | Human approval and citizen workflows pass. |
| 11 | Authority dashboard | Live command center consumes canonical APIs. |
| 12 | Citizen UI/local gateway | Online/offline emergency experience works. |
| 13 | Simulation/replay | Same pipeline verified. |
| 14 | Resilience/security/benchmarks | Mandatory pre-freeze gates pass. |
| 15 | Polish | Glassmorphism/UI polish, animation, demo instrumentation. |

## 24.1 Rule for parallelization

Frontend can begin against mocked canonical response fixtures before the backend is complete, but mocks must conform to the final schemas. Geospatial UI can use frozen fixture GeoJSON while the engine is being implemented. No mock may silently evolve into a second undocumented schema.

# 25. Vibe-Coding Instructions for Agents

## 25.1 Agent operating procedure

Read Document 01 and this document before editing architecture-sensitive code.

Read the domain-specific document before implementing that domain.

Use requirement IDs from Document 02 in issue/commit/test references where practical.

Implement the smallest conformant module; do not introduce unrelated frameworks.

Keep computation server/edge-side according to the compute allocation matrix.

Use typed interfaces at module boundaries.

Write tests before changing shared formulas where practical.

For Python/C++ parity, add/update a golden-vector fixture whenever the reference math changes.

Do not hard-code configurable thresholds in application logic.

Log enough context to diagnose a demo failure without logging secrets or unnecessary citizen data.

After implementation, run unit, integration and relevant system tests before declaring a requirement complete.

## 25.2 Prohibited agent behavior

Do not add a new database, message broker, ML framework or frontend framework without explicit approval.

Do not migrate the architecture to microservices.

Do not move fire spread into the ESP32 because it seems convenient.

Do not replace PostGIS with ad hoc JSON geometry storage.

Do not add a second simulation path that bypasses telemetry ingestion.

Do not auto-dispatch responders.

Do not silently replace missing datasets with fabricated values.

Do not change the visual design language from the locked dark grey/black glassmorphism system without UI/UX spec approval.

# 26. Final Stack Summary — Implementation Card

| Component | Final V1 choice | Execution target |
| --- | --- | --- |
| Field firmware | ESP-IDF + C/C++ | ESP32-S3 |
| Field scheduler | FreeRTOS via ESP-IDF | ESP32-S3 |
| Embedded crypto | HMAC + secure key handling | ESP32-S3 |
| Reference intelligence | Python | Developer/Master/Cloud |
| Embedded intelligence | C/C++ equivalent subset | ESP32-S3 |
| Master application | Python + FastAPI-style backend | Raspberry Pi/Linux |
| Geospatial compute | Python + geospatial libraries + PostGIS | Raspberry Pi/Linux or cloud |
| Database | PostgreSQL + PostGIS | Master/cloud |
| Background workers | Python worker profile | Master/cloud |
| Web frontend | Next.js + React + TypeScript | Browser/Node build |
| Visual system | Tailwind + CSS custom properties | Browser |
| Maps | MapLibre GL JS | Browser |
| Real-time | WebSocket/SSE | Backend/browser |
| Simulation | Python | Master/developer/cloud |
| Containers | Docker + Compose for reproducible environments | Dev/demo |
| Testing | pytest + frontend tests + firmware tests | CI/dev |
| Native app | Android/iOS, optional | Deferred |

The core architecture in one line ESP32-S3 senses and reasons locally → Raspberry Pi aggregates and performs heavy geospatial/operational computation → backend persists and coordinates → TypeScript web clients render the same canonical truth.

# 27. Technology Baseline Acceptance Checklist

☐ 01  ESP-IDF firmware builds for the exact selected ESP32-S3 target.

☐ 02  C/C++ firmware has a documented task/concurrency model.

☐ 03  ESP32 can perform sensing + Wi-Fi/AP/transport + HMAC + local intelligence under benchmarked load.

☐ 04  Python reference intelligence is implemented and testable independent of the API.

☐ 05  Golden-vector tests exist for Python vs embedded C/C++ shared functions.

☐ 06  Raspberry Pi can run the local backend and database profile.

☐ 07  PostGIS spatial operations are validated on the chosen deployment.

☐ 08  Frontend builds with Next.js/React/TypeScript and consumes typed API fixtures.

☐ 09  MapLibre renders fixture current/warning/projection geometries and live node/incident markers.

☐ 10  Local citizen emergency page can operate without cloud.

☐ 11  Cloud synchronization is optional to the essential local loop.

☐ 12  Simulation publishes canonical telemetry through the same endpoint.

☐ 13  Secrets/keys are supplied through configuration/secret handling, not committed.

☐ 14  Dependency versions are pinned and reproducible.

☐ 15  Docker Compose can bring up the local software stack reproducibly.

☐ 16  Structured logs and an internal debug path exist.

☐ 17  No implementation has introduced an unauthorized framework or infrastructure dependency.

# 28. Technology Reference Notes

The stack choices above were cross-checked against current official documentation where relevant. Exact dependency versions must still be pinned and tested as part of implementation.

| Technology | Reference note |
| --- | --- |
| ESP-IDF / ESP32-S3 C++ | Espressif documents C++ application support for ESP32-S3 and provides the ESP-IDF framework/toolchain workflow. citeturn424934search0turn424934search8turn424934search9 |
| Python | Python 3.14 is a current stable release; project implementation should pin an exact compatible patch version rather than track 'latest'. citeturn424934search4 |
| FastAPI | Official deployment documentation describes deployment as serving the API through an appropriate production/runtime setup. citeturn424934search2 |
| Next.js / React / TypeScript | Next.js is a React framework; official React documentation supports TypeScript with React. citeturn169935search0turn424934search6turn424934search1 |
| MapLibre GL JS | Official docs support interactive maps, GeoJSON, raster sources and realtime updates; it is suitable for NexAlert's operational mapping layer. citeturn169935search1turn169935search3 |
| PostGIS | PostGIS extends PostgreSQL with spatial storage, spatial indexes and spatial analysis functions including distance, area, intersection and buffering. citeturn169935search5turn169935search6 |
| Docker Compose | Official Docker documentation describes Compose as the mechanism for defining/running multi-container environments with networks and volumes. citeturn169935search8 |