NEXALERT

FIRE SPREAD, GEOSPATIAL SIMULATION, AFFECTED AREA & RISK/HEATMAP

Final V1 Engineering Specification — Physical Model, Geometry, Risk & Visualization Contract

Document role This is the definitive V1 engineering specification for NexAlert's fire-spread and geospatial chain. It specifies how ignition becomes a spatially evolving arrival-time field, how the field becomes current/warning/projected geometry, how physical footprint is separated from policy safety buffers, how the hazard risk surface is computed, and how all geospatial outputs are validated and exposed to downstream systems.

| Field | Value |
| --- | --- |
| Product | NexAlert |
| Problem | SIH26178 — resilient AI-powered environmental monitoring and early warning |
| Document | 05 — Fire Spread, Geospatial Simulation, Affected Area & Risk/Heatmap |
| Status | FINAL V1 IMPLEMENTATION BASELINE |
| Hero hazard | Forest fire |
| Primary compute | Raspberry Pi Master and/or cloud/backend |
| Edge restriction | Heavy raster/geospatial fire simulation MUST NOT run on ESP32 |
| Model family | Rothermel-family reduced directional ROS + arrival-time / minimum-travel-time style propagation |
| Primary spatial source of truth | Arrival-time field |

Honesty boundary V1 is a physics-informed engineering model derived from the Rothermel family, not a full operational wildfire simulator. Simplified fuel classes, moisture proxying, reduced wind treatment and finite grid resolution are deliberate V1 assumptions. They must be disclosed wherever relevant and must never be presented as full operational Rothermel/FARSITE fidelity.

# Document Control & Authority

| Status | Meaning |
| --- | --- |
| LOCKED | Algorithmic or architectural structure that implementation must preserve. |
| PARAMETER | Numerical value requiring calibration, measurement, sensitivity testing or deliberate policy selection. |
| ASSUMPTION | Explicit simplification used by V1; visible in metadata/UI when material. |
| DEGRADED | A required input/capability is unavailable but a controlled fallback remains usable. |
| UNKNOWN | There is insufficient information to produce a trustworthy output. |
| ROADMAP | Potential future enhancement; outside V1. |
| REJECTED | Approach intentionally excluded from V1. |

## Authority documents

| Topic | Primary source |
| --- | --- |
| Overall system architecture | 01 — Master Architecture & Invariants |
| Technical requirement | 02 — TRD |
| Technology/runtime boundary | 03 — Technology Stack / Deployment |
| Intelligence/hazard evidence | 04 — Mathematical Intelligence & Hazard Reasoning |
| Hardware telemetry | 06 — Hardware/Edge |
| Data provenance | 08 — Data/Dataset |
| Dashboard rendering | 13 — Authority Dashboard |
| Validation | 17 — Validation |

Red-team basis The final adversarial fire/geospatial review confirmed the arrival-time/event-based approach, while requiring explicit handling of wind FROM→TO conversion, diagonal √2 distances, vector wind+slope treatment, projected CRS, mid-run environmental recomputation, MultiPolygon support, and footprint-vs-buffer semantics. fileciteturn19file0 fileciteturn19file16

# 1. Purpose & Design Goals

## 1.1 What this engine does

The Fire/Geospatial Engine converts a validated fire incident plus environmental and geographic context into an evolving spatial model. Its outputs are not merely a visual animation. The engine produces an arrival-time field that is then deterministically transformed into current physical footprint, warning/projection regions, a policy-defined operational buffer, a spatial hazard-risk index, and geometry/statistics for downstream exposure and response.

## 1.2 What it must prove

Wind changes the direction and anisotropy of spread in a physically interpretable way.

Slope and aspect influence propagation direction and speed.

Different fuel classes produce different local spread behavior.

Moisture/context suppresses or allows spread.

Non-burnable barriers block propagation and force the front around them.

Propagation is driven by computed arrival times, not a decorative expanding circle.

Current, warning and projected areas are derived from one source-of-truth arrival field.

The risk heatmap is distinct from physical footprint and exposure.

Environmental changes can update the future projection without rewriting elapsed history.

## 1.3 What it does NOT do

No CFD/FIRETEC/WFDS-class fluid/combustion simulation.

No ember spotting/reignition in V1.

No exact flame-front turbulent physics.

No claim of operationally validated wildfire forecasting.

No hidden fabricated fuel, population, weather or infrastructure values.

No fire-spread computation on the ESP32.

# 2. Canonical Terminology

| Term | Definition |
| --- | --- |
| Cell | One element of the spatial simulation grid. |
| Ignition point | Geographic point where the fire starts; mapped to a seed cell. |
| Fuel cell | Grid cell with a configured fuel class/behavior. |
| Burnable | Boolean/property indicating whether the cell can propagate fire in V1. |
| ROS | Rate of spread, expressed as a distance per time under the model. |
| Directional ROS | ROS for a specific propagation direction. |
| Arrival time T_a(c) | Earliest simulated time at which fire reaches cell c. |
| Current footprint | Cells already reached by T_a(c) <= now. |
| Warning zone | Cells predicted to be reached within the short future horizon. |
| Projection zone | Cells predicted to be reached within the longer future horizon. |
| Physical footprint | Model-derived fire area; a physics/model output. |
| Operational buffer | Policy-defined margin around the physical footprint; not physics. |
| Hazard Risk Surface | Continuous operational index over space; not probability. |
| Exposure | Population/infrastructure/citizen consequence associated with geometry; downstream concept. |
| Simulation domain | Finite spatial region within which propagation is computed. |

Naming rule The UI and API must use 'physical footprint' when referring to arrival-time-derived fire geometry and 'operational buffer' when referring to a policy margin. Never call a policy buffer 'the fire footprint.'

# 3. Input Contract

## 3.1 Required input categories

| Input | Required | Source | Purpose |
| --- | --- | --- | --- |
| Ignition point | YES | Incident/node location | Seed cell. |
| Simulation CRS | YES | Configuration | Distance/area-safe spatial operations. |
| Grid/domain | YES | Configuration/geospatial preprocessing | Cell topology and extent. |
| DEM / elevation | YES for claimed slope modeling | Preprocessed dataset | Slope/aspect. |
| Fuel class | YES | Preprocessed configuration/raster | Base spread behavior. |
| Fuel moisture/context | YES for moisture-aware output | Environmental context / proxy | Moisture suppression. |
| Wind speed | YES for wind-aware output | Sensor/external context/configured assumption | Wind forcing. |
| Wind direction | YES for directional wind | Sensor/external context/configured assumption | Propagation direction. |
| Current time | YES | Simulation clock | Current footprint extraction. |
| Projection horizons | YES | Configuration | Warning/projection extraction. |

## 3.2 Optional/advanced inputs

Higher-resolution fuel behavior.

Live weather updates.

External observational evidence.

More detailed canopy/wind-reduction information.

Additional infrastructure/context layers.

## 3.3 Canonical cell model

Cell
  cell_id
  row
  col
  x
  y
  lat
  lon
  elevation_m
  slope_rad
  aspect_rad
  fuel_model
  fuel_class
  burnable
  moisture_index
  base_ros
  arrival_time_s
  state
  ignition_time_s
  burn_time_s
  local_ros_features
  local_risk

The backend may use a more optimized internal representation (NumPy arrays, raster arrays, typed structs) rather than this object layout. The semantic fields remain canonical.

# 4. Coordinate Reference System & Spatial Units

## 4.1 Non-negotiable geospatial rule

Never treat degrees as metres Raw latitude/longitude is geographic angular coordinate data. All grid spacing, distance, spread-rate, buffering, polygon area and route geometry calculations that require metric units MUST use an appropriate projected CRS or an explicit geodesic method.

## 4.2 Recommended V1 strategy

Reproject the demo region into a local metric projected CRS suitable for the region (for example, the correct UTM zone for the chosen demo location). Keep WGS84 latitude/longitude for external identity and display, but use the projected coordinate system for simulation geometry and metric calculations.

## 4.3 Required transformations

Input:
  WGS84 (lat/lon)
      ↓
Project to local metric CRS
      ↓
Build square simulation grid in metres
      ↓
Run spread / distance / buffer / area computations
      ↓
Polygon geometry in projected CRS
      ↓
Reproject to WGS84 for web map display

## 4.4 CRS metadata

| Field | Requirement |
| --- | --- |
| crs_id | Persist exact EPSG/CRS identifier. |
| resolution_m | Persist cell size. |
| domain_extent | Persist simulation bounds. |
| transform | Persist/reconstruct raster transform. |
| source_crs | Persist original dataset CRS where relevant. |

The fire output metadata must state the CRS used for the simulation and the display transformation.

# 5. Terrain Engine — DEM, Slope & Aspect

## 5.1 DEM

The terrain engine samples or resamples the chosen DEM onto the simulation grid. The DEM value represents elevation in metres unless the source explicitly defines another unit and a conversion is performed.

## 5.2 Central-difference gradient

∂z/∂x = (z_E − z_W) / (2Δx)

∂z/∂y = (z_N − z_S) / (2Δy)

g = √[(∂z/∂x)^2 + (∂z/∂y)^2]

β = atan(g)

Aspect is the uphill direction of the terrain gradient and is retained as a directional quantity for the slope vector.

## 5.3 Edge cells

| Case | Behavior |
| --- | --- |
| Interior cell | Use two-sided central differences. |
| Domain edge | Use one-sided difference or explicitly mark lower-confidence terrain derivative. |
| Missing elevation | Use configured flat-terrain fallback only if enabled; mark TERRAIN_DEGRADED. |

## 5.4 Slope effect

The exact slope-response function is a configurable Rothermel-family reduction rather than a claim of full operational fidelity. A simplistic linear approximation is acceptable only when documented and validated. A steeper-terrain response must be monotonic in the intended direction and must not create physically nonsensical negative or explosive rates.

Important The V1 document does not invent a universal slope coefficient. The selected function and coefficients belong in the parameter registry and must be sensitivity-tested. The red-team review specifically warned that a pure linear slope factor can understate steep-slope acceleration. fileciteturn19file14

# 6. Fuel Model

## 6.1 V1 fuel representation

V1 uses a simplified categorical fuel scheme for the demo region. It is explicitly not a full Anderson/Scott-Burgan operational fuel-model library.

| Class | Conceptual behavior |
| --- | --- |
| NON_BURNABLE | ROS=0; fire cannot enter this cell. |
| GRASS | Fast/low-residence fuel behavior. |
| SHRUB | Intermediate behavior. |
| LIGHT_FOREST | Moderate/variable spread. |
| DENSE_FOREST | Higher fuel load and context-dependent spread. |
| URBAN_MIX | Configured cautiously; burnable status and behavior depend on the demo assumption. |

## 6.2 Fuel profile

FuelProfile:
  fuel_model_id
  fuel_class
  base_ros_r0
  wind_response_parameters
  slope_response_parameters
  moisture_response_parameters
  burnable
  provenance
  calibration_status

## 6.3 Provenance rule

Every demo fuel map must have an explicit source/provenance. Hand-labeled fuel classes are acceptable for a prototype, but the dashboard and documentation must identify them as simplified demo classification.

# 7. Moisture / Environmental Suppression

## 7.1 V1 moisture model

The fire engine uses a normalized moisture index or equivalent environmental proxy to scale the base spread behavior. Relative dryness should increase spread capability and wetter conditions should suppress it.

F_M(c) ∈ (0,1]

ROS_moisture(c,d) = ROS_directional(c,d) × F_M(c)

## 7.2 Proxy rule

Do not equate humidity to fuel moisture Relative humidity and rainfall are environmental proxies/context inputs. They are not literally the same physical quantity as multi-timelag fuel moisture. V1 may use a calibrated proxy, but the implementation and UI must label it as a moisture proxy/simplification.

## 7.3 Example proxy structure

A configurable proxy can map recent humidity/rainfall/context into a normalized dryness/moisture index. Exact coefficients must remain configurable and calibration-driven.

recent_rain
humidity_context
optional temperature_context
        ↓
normalized moisture_index
        ↓
moisture response F_M ∈ (0,1]

# 8. Wind Model

## 8.1 Wind direction convention

Critical correctness rule Meteorological wind direction is reported as FROM. Fire propagation uses the reciprocal TO direction.

θ_TO = (θ_FROM + 180°) mod 360°

This conversion MUST occur in the backend/fire model before the wind becomes a propagation vector. The frontend must not compensate for a backend error.

## 8.2 Midflame wind

U_mid = WAF × U_input

| Term | Meaning |
| --- | --- |
| U_input | Input wind speed at the measurement/source height. |
| WAF | Wind adjustment/reduction factor; configurable. |
| U_mid | Estimated wind speed relevant to fire-spread formulation. |

The fixed reduction factor is a prototype simplification of more detailed canopy/vegetation adjustment. If a more detailed method becomes available, it can replace the factor without changing the surrounding architecture.

## 8.3 Calm/missing wind

| Condition | Behavior |
| --- | --- |
| U_mid below configured calm threshold | Treat wind forcing as effectively zero and use near-isotropic directional spread. |
| Wind missing | Use only an explicitly configured calm-wind/assumption path; label output DEGRADED. |
| Wind stale | Do not silently treat as fresh; mark information degraded and use policy-defined fallback. |
| Wind direction invalid | Reject directional wind input; fall back only if configured and labeled. |

# 9. Directional Wind + Slope Combination

## 9.1 Design

Wind and slope are represented as directional influence vectors rather than independent arbitrary multipliers applied to each grid direction. This reduces the number of free directional parameters and produces a coherent head-fire direction when the forcings are aligned or opposed.

## 9.2 Vector convention

Use a local Cartesian coordinate system for the projected simulation grid. Define +x and +y consistently with the projected coordinates. Convert the TO wind bearing and uphill slope aspect into unit vectors in this coordinate frame.

v_hat(θ) = [cos(θ_cartesian), sin(θ_cartesian)]

V_w = M_w(U_mid, fuel, context) × v_hat(θ_wind_to)
V_s = M_s(slope, fuel, context) × v_hat(θ_uphill)

V_eff = V_w + V_s
U_eff = ||V_eff||
θ_eff = atan2(V_eff_y, V_eff_x)

## 9.3 Resultant interpretation

| Output | Meaning |
| --- | --- |
| U_eff | Effective directional forcing strength used by the reduced directional model. |
| θ_eff | Preferred head-fire direction under combined wind/slope forcing. |
| Direction anisotropy | Degree to which spread differs by direction; determined by the approved directional/elliptical formulation. |

Implementation note The exact conversion from U_eff to ellipse eccentricity and directional ROS belongs to the calibrated directional-spread parameterization. Do not hard-code an unexplained linear 'eccentricity = wind × constant' relationship. The model must be monotonic, bounded and validated.

# 10. Base & Directional Rate of Spread (ROS)

## 10.1 Base ROS

Each burnable cell begins with a fuel-specific baseline spread rate R0(c), representing the no-wind/no-slope reference under the configured reference moisture/context.

R_base(c) = R0(c) × F_M(c)

## 10.2 Directional response

The approved directional model maps the effective wind+slope forcing and the candidate propagation bearing d into a directional multiplier/ROS. For implementation, this should be represented by an explicit, testable function:

ROS(c,d) = R_base(c) × F_dir(c,d; V_eff, θ_eff, fuel)

F_dir must be positive for burnable cells, equal to zero for non-burnable cells, and must be bounded according to the calibrated profile.

## 10.3 Elliptical interpretation

The directional response is represented as an ellipse-like anisotropic spread around the head-fire direction. The ellipse is a model representation of directional ROS, not a decorative shape drawn independently of the propagation.

For each candidate bearing d:
  angular_offset = normalized_angle(d - θ_eff)
  directional_factor = EllipseDirectionalFactor(
      angular_offset,
      eccentricity,
      calibrated parameters
  )
  ROS(c,d) = R_base(c) × directional_factor

## 10.4 No independent per-neighbor hand tuning

Rejected implementation Do not create eight unrelated multipliers such as north×1.0, northeast×1.2, east×0.9, etc. without a physical/directional parameterization. Directional differences must emerge from the common vector/ellipse formulation and cell-specific inputs.

# 11. Simulation Grid & Neighborhood

## 11.1 Grid

The simulation uses a regular 2D grid in projected metric coordinates.

| Property | V1 rule |
| --- | --- |
| Grid shape | Rectangular raster grid. |
| Neighbourhood | 8-neighbor topology. |
| Orthogonal distance | Δ. |
| Diagonal distance | Δ√2. |
| Cell coordinate | Projected x/y plus row/col index. |

d(c,n) = Δ, if orthogonal;  Δ√2, if diagonal

Critical test Using Δ for diagonal neighbors is a correctness bug. It overestimates diagonal spread by √2 and can produce diamond/star artifacts in calm-wind uniform tests. fileciteturn19file1

# 12. Arrival-Time Propagation

## 12.1 Source of truth

The primary output of the spread simulation is an arrival-time field T_a(c), not a frame-by-frame animation state. Any visualization of the fire front must be derived from this field.

## 12.2 Initialization

For all cells c:
  T_a(c) = +∞
  state(c) = UNBURNED

Map ignition point to seed cell s:
  T_a(s) = t0
  state(s) = IGNITED

## 12.3 Candidate arrival

T_candidate(n) = T_a(c) + d(c,n) / ROS(c,d)

T_a(n) = min[T_a(n), T_candidate(n)]

The direction d is the bearing from cell c to neighbour n in the model's coordinate convention.

## 12.4 Priority queue

Use a min-priority queue keyed by the current best arrival time. This produces event-based propagation without requiring a tiny fixed simulation timestep. It is appropriate for the SIH prototype because the output is a minimum-travel-time/arrival field rather than a full transient combustion PDE.

push(seed, T_a(seed))

while queue not empty:
    c = pop_min()
    if popped_time > T_a(c):
        continue

    for each neighbour n:
        if outside_domain: continue
        if not burnable(n): continue
        if already_burned_or_frozen(n): continue

        d = neighbor_distance(c,n)
        bearing = direction(c,n)
        ros = compute_ros(c, bearing)

        if ros <= 0:
            continue

        candidate = T_a(c) + d/ros

        if candidate < T_a(n):
            T_a(n) = candidate
            parent(n) = c
            push(n, candidate)

## 12.5 Cell state semantics

| State | Meaning |
| --- | --- |
| UNBURNED | Arrival time not reached yet. |
| IGNITED | Seed/current event initialization state. |
| BURNING | Cell's arrival time is current/past and active under the run's semantic model. |
| BURNED | Cell has passed through the active burn window; V1 does not support reignition. |

## 12.6 No reignition

Once a cell has been treated as burned in a run, V1 does not model ember-driven re-ignition. This is a documented scope boundary.

# 13. Current, Warning & Projection

## 13.1 Current footprint

A_current(t) = { c | T_a(c) ≤ t }

## 13.2 Warning zone

A_warning(t) = { c | t < T_a(c) ≤ t + τ_w }

## 13.3 Projection zone

A_projection(t) = { c | t < T_a(c) ≤ t + τ_p }

τ_p > τ_w > 0

Projection is conditional on the model assumptions remaining valid for the forecast horizon. It must be labeled accordingly.

## 13.4 Nested geometry rule

A_current ⊆ A_warning_cumulative ∪ A_current ⊆ A_projection_cumulative

For UI clarity, the preferred dashboard representation is cumulative zones derived from arrival thresholds. If the UI presents disjoint ring populations/areas, explicitly define those rings. Never mix cumulative and disjoint semantics silently.

## 13.5 Replay

Replay uses the same arrival-time field. Moving the timeline changes the threshold t; it must not invoke a different rendering algorithm that disagrees with the field.

# 14. Changing Environmental Conditions During a Run

## 14.1 Problem

A precomputed arrival-time field assumes its environmental inputs remain valid over the modeled period. If meaningful inputs change—especially wind direction/speed or other material forcing—the future field must be recomputed.

## 14.2 Mandatory recompute strategy

At update time t_update:
  1. Freeze historical truth:
       cells with T_a(c) <= t_update remain historical/current truth.
  2. Extract current perimeter/frontier:
       cells at or near the current boundary.
  3. Apply new environmental/fuel/context inputs.
  4. Reset future/unreached arrival times for affected domain.
  5. Seed queue from current perimeter with arrival=t_update
       (preserve elapsed history).
  6. Recompute future propagation.
  7. Generate a new projection version.
  8. Record "projection updated" event.

Important Do not silently rewrite the past. A dashboard should show that the future projection changed because model inputs changed. The red-team review explicitly requires recomputation from the current perimeter rather than patching an old field. fileciteturn19file14

# 15. Non-Burnable Cells & Barriers

## 15.1 Rule

if burnable(c) = false → ROS(c,d) = 0

A non-burnable cell is not merely painted as a firebreak. It is excluded from propagation. The event-based algorithm naturally routes around it through reachable burnable neighbors.

## 15.2 Barrier examples

Water body.

Configured non-burnable land cover.

Explicit synthetic firebreak test strip.

Map-domain masked-out area.

## 15.3 Ignition on non-burnable cell

An ignition point mapping to a non-burnable cell is an invalid simulation input and must be rejected/flagged rather than producing an empty or contradictory simulation.

# 16. Domain Boundaries & Missing Geodata

| Condition | Required behavior |
| --- | --- |
| Fire reaches simulation edge | Clip geometry; display 'Projection clipped to simulation domain.' |
| Ignition outside domain | Reject or require explicit domain expansion; do not silently clamp. |
| Missing DEM | Optional flat-terrain fallback only if configured; label TERRAIN_DEGRADED. |
| Missing fuel | Use only an explicit simplified/default fuel profile if configured; label assumption. |
| Missing moisture | Use explicit default moisture profile only if configured; label assumption. |
| Missing wind | Calm/assumed-wind path only if configured; label DEGRADED. |
| Invalid fuel value | Reject cell or map to explicit unknown/unusable class; never silently choose extreme fuel. |
| Data extent mismatch | Clip/mark unavailable rather than extrapolate silently. |

# 17. Affected-Area Geometry Engine

## 17.1 Raster-to-region

Arrival-time raster
   ↓
threshold selection
   ↓
boolean cell mask
   ↓
connected-component analysis
   ↓
raster/contour polygonization
   ↓
geometry validation
   ↓
optional display simplification
   ↓
GeoJSON / spatial DB representation

## 17.2 Geometry validity

Every derived polygon must be checked for validity. Self-intersections, ring defects and invalid topology must be repaired using a standard geometry-validity operation where safe; if repair cannot produce a trustworthy result, flag the output rather than silently displaying corrupted geometry.

## 17.3 MultiPolygon

The output type must support Polygon and MultiPolygon from the beginning. Disconnected burnable regions, barriers, masked domains and multiple future reachable components can produce multiple regions.

## 17.4 Simplification

Display-only simplification MAY be applied to reduce web payload and visual noise. The unsimplified raster/derived geometry remains the source for scientific/statistical calculations.

## 17.5 Geometry versioning

| Field | Purpose |
| --- | --- |
| geometry_version | Identify regeneration after model/input changes. |
| source_arrival_run_id | Tie geometry to arrival-field computation. |
| generated_at | Trace when geometry was produced. |
| model_assumptions_hash | Identify relevant parameter/input configuration. |

# 18. Physical Footprint vs Operational Buffer

## 18.1 Physical footprint

The physical footprint is derived from the arrival-time field. It answers: where does the modeled fire reach under the current model assumptions?

## 18.2 Operational buffer

B_operational = buffer(A_physical, m_policy)

m_policy is a policy-defined safety margin. The buffer may be larger than the modeled footprint to account for operational caution, uncertainty or response policy.

Non-negotiable distinction The operational buffer is not a more accurate fire physics model. It is a policy margin. The dashboard must use separate labels, styles and layer semantics for footprint versus buffer.

## 18.3 Reprojection/buffer rule

Buffer distances must be calculated in the appropriate metric projected CRS. Never apply a metre buffer directly to WGS84 degrees.

# 19. Hazard Risk Surface / Heatmap

## 19.1 Purpose

The hazard risk surface expresses spatial urgency and operational concern. It is not the physical footprint and it is not probability.

## 19.2 Spatial components

U_c = exp(−λ_τ τ_c)

For cells already reached, set U_c=1. For unreachable/invalid future cells, handle according to domain policy rather than inventing a large finite arrival time.

R_haz,c = w_U U_c + w_I I_c + w_E E_c

w_U + w_I + w_E = 1

| Component | Meaning |
| --- | --- |
| U_c | Time-to-arrival urgency; closer imminent arrival produces larger value. |
| I_c | Local hazard intensity/spread-strength proxy, normalized within configured bounds. |
| E_c | Local escalation/trend contribution where a valid temporal field exists. |

## 19.3 Intensity input

For V1 fire, I_c may be derived from local normalized directional ROS or another approved local intensity proxy. It must be normalized before combination.

## 19.4 Operational index label

Display semantics Call this 'Hazard Risk Surface', 'Spread Urgency Index' or equivalent. Do not label it 'probability of fire' unless a separate statistical calibration process has established that interpretation.

## 19.5 Exposure separation

Population density, school count, hospital presence and infrastructure consequence are not direct ingredients of the physical hazard risk surface. They belong to exposure/impact. This prevents double counting the same consequence information in both spatial hazard intensity and exposure.

# 20. Risk Classification & Map Layers

## 20.1 Continuous vs discrete

| Output | Representation |
| --- | --- |
| Physical footprint | Discrete polygon(s) derived from arrival-time threshold. |
| Operational buffer | Distinct polygon(s) derived from physical footprint + policy margin. |
| Warning/projection | Discrete polygon(s) derived from arrival-time thresholds. |
| Hazard risk | Continuous raster/heatmap. |
| Exposure | Separate spatial/statistical layers. |

## 20.2 Risk bands

Risk classification thresholds are configurable parameters. Example labels may be LOW, MODERATE, ELEVATED, HIGH and CRITICAL. The numeric cutoffs must be held in the parameter registry and validated for monotonic behavior.

## 20.3 Multi-hazard rule

Permanent rule Fire risk, flood risk, pollution risk and other hazard risk surfaces remain separate and independently toggleable. Do not algebraically add or average unrelated hazard surfaces into a single universal disaster-risk number. fileciteturn19file16

# 21. Area, Growth & Spatial Statistics

## 21.1 Area

Area is calculated from the valid projected geometry or an appropriate geodesic/equal-area method.

A(t) = area(PhysicalFootprint(t))

## 21.2 Area growth

G_A(t) = [A(t) − A(t−Δt)] / Δt

The UI may display growth as m²/min, ha/min or another clearly labeled unit.

## 21.3 Mean spread rate

Where a scalar summary is required, define it explicitly as an aggregate diagnostic (for example perimeter-normal or area-equivalent growth rate) rather than confusing it with directional ROS. Directional ROS and footprint growth are different quantities.

## 21.4 Do not conflate

| Quantity | Meaning |
| --- | --- |
| ROS | Local directional propagation rate. |
| Arrival time | Time required to reach a cell. |
| Area | Area already reached by a chosen threshold. |
| Area growth | Rate of area change over time. |
| Risk surface | Operational spatial index. |

# 22. Downstream Exposure Interface

## 22.1 What this engine provides

GeospatialOutput
  incident_id
  run_id
  crs
  current_geometry
  warning_geometry
  projection_geometry
  operational_buffer_geometry
  arrival_time_source
  risk_surface_source
  generated_at
  assumptions
  domain_extent
  projection_horizon

## 22.2 Exposure engine responsibilities

Population and infrastructure calculations are handled by the Exposure module. This document only defines the geometry and spatial artifacts that it consumes.

## 22.3 Population semantics

Downstream exposure must choose either cumulative zone semantics or disjoint ring semantics and document the choice. It must never naively add nested current/warning/projection populations. fileciteturn19file13

# 23. LIVE vs SIMULATION

## 23.1 Same engine

The Fire Spread Engine is identical in LIVE and SIMULATION modes. Only the source and control of environmental input values may differ.

| Mode | Input source |
| --- | --- |
| LIVE | Validated incident + current environmental/geospatial context. |
| SIMULATION | Scenario-controlled telemetry/environmental inputs marked SIMULATION. |

## 23.2 Simulation rule

Synthetic telemetry reaches the same ingestion and intelligence path before a fire incident is created. The fire engine receives the resulting structured hazard context just like a live incident.

## 23.3 Visualization rule

No fake spread animation The frontend must animate the same arrival-time solution produced by this engine. It must not draw an independent expanding circle, ellipse, or hand-authored polygon just to look smoother.

# 24. Frontend / Dashboard Geospatial Contract

## 24.1 Map objects

| Object | Source |
| --- | --- |
| Current footprint | Physical arrival-time-derived geometry. |
| Warning zone | Short-horizon arrival threshold. |
| Projection | Longer-horizon arrival threshold. |
| Operational buffer | Policy buffer around physical footprint. |
| Risk heatmap | Hazard Risk Surface raster/source. |
| Wind vector | Current wind TO direction + speed. |
| Head-fire direction | θ_eff / derived model direction. |
| Terrain | DEM/slope analytical layer where enabled. |
| Fuel | Fuel-class context layer where enabled. |
| Nodes | Node registry. |
| SOS | SOS store. |
| Population/infrastructure | Exposure data. |

## 24.2 Required map semantics

Current footprint and operational buffer must be visually distinguishable.

Warning and projection must be labeled with time horizon.

Risk heatmap must not be mistaken for exact physical geometry.

Projection must be marked conditional on current/model assumptions.

Stale/degraded environmental inputs must be visible.

Map controls must allow independent toggling of each major layer.

## 24.3 Fire Spread page

The Fire Spread page must expose LIVE/SIMULATION state, play/pause/reset, time slider, current footprint, projection, risk layer, wind/environment context, and model assumptions. The dashboard may show advanced technical data, but must obtain them from this backend output rather than recomputing them.

# 25. Failure & Edge-Case Matrix

| Condition | Expected behavior |
| --- | --- |
| Calm wind | Near-isotropic spread; no division/degenerate ellipse errors. |
| Missing wind | Explicit DEGRADED / calm-wind assumption. |
| Stale wind | Information degraded; projection updated or constrained. |
| Wind from 0/90/180/270 | Direction conversion tested; footprint rotates correctly. |
| Wind + slope aligned | Faster preferred spread in aligned direction. |
| Wind + slope opposed | Resultant direction reflects opposition; no sign errors. |
| Cross-slope wind | Resultant rotates between forcings rather than choosing one independently. |
| Uniform fuel, flat, calm | Near-circular spread with no grid-artifact star/diamond. |
| Different adjacent fuels | Different local spread response visible. |
| Non-burnable barrier | Fire routes around; never through barrier. |
| Ignition on non-burnable | Reject/flag. |
| Missing DEM | Explicit flat-terrain assumption or UNKNOWN. |
| Unknown fuel | Explicit default/unknown behavior; no silent extreme class. |
| Domain boundary reached | Clip and label domain limitation. |
| Disconnected burnable islands | MultiPolygon output supported. |
| Self-intersecting derived polygon | Attempt validity repair; otherwise flag. |
| Environmental change mid-run | Recompute from current perimeter; preserve historical elapsed field. |
| High ROS | No fixed-timestep instability expected, but queue/runtime benchmark must be measured. |
| No population data | Exposure unavailable; fire geometry still valid. |

# 26. Mathematical & Geospatial Acceptance Tests

| ID | Test | Pass condition |
| --- | --- | --- |
| GEO-001 | Wind FROM→TO | Set wind FROM north and verify preferred fire propagation is southward (TO). |
| GEO-002 | Wind rotation | Rotate wind input by 90° in a uniform landscape; major spread axis rotates correspondingly. |
| GEO-003 | Calm circularity | Uniform flat fuel + calm wind produces near-circular spread. |
| GEO-004 | Diagonal distance | Verify diagonal step uses Δ√2. |
| GEO-005 | Barrier routing | Insert a non-burnable strip; fire routes around it. |
| GEO-006 | Fuel heterogeneity | Adjacent fuel classes create different local spread rates. |
| GEO-007 | Moisture response | Increase moisture index while holding others fixed; spread decreases monotonically. |
| GEO-008 | Slope response | Increase slope in controlled direction; spread response follows calibrated monotonic slope function. |
| GEO-009 | Wind+slope vector | Apply orthogonal wind/slope forcings; preferred direction reflects vector resultant. |
| GEO-010 | Opposing forcings | Apply opposing wind/slope vectors; no sign reversal or NaN. |
| GEO-011 | Arrival ordering | Known small grid produces increasing arrival times along valid propagation path. |
| GEO-012 | Footprint extraction | At time t, footprint equals cells with T_a <= t. |
| GEO-013 | Projection extraction | Projection equals configured future threshold on same T_a field. |
| GEO-014 | Replay consistency | Changing time cursor produces same geometry as recomputing threshold on stored T_a. |
| GEO-015 | Mid-run recompute | Change wind; future projection changes while historical state stays unchanged. |
| GEO-016 | Boundary clip | Fire reaching domain edge yields clipped geometry with explicit label. |
| GEO-017 | MultiPolygon | Disconnected burnable regions yield MultiPolygon where appropriate. |
| GEO-018 | Polygon validity | All displayed output geometries pass validity test. |
| GEO-019 | Projected CRS | Known metric test passes for distance/area/buffer. |
| GEO-020 | Buffer separation | Operational buffer differs geometrically and semantically from physical footprint. |
| GEO-021 | Risk monotonicity | Earlier arrival, all else fixed, does not reduce urgency term. |
| GEO-022 | Risk independence | Population overlay does not change physical hazard risk surface. |
| GEO-023 | No risk aggregation | Fire and flood surfaces remain separate outputs. |
| GEO-024 | Missing wind labeling | Withhold wind and verify explicit degraded assumption. |
| GEO-025 | No silent fabricated data | Missing fuel/population/infrastructure never becomes an unlabelled numeric value. |

# 27. Calibration & Validation Strategy

## 27.1 Fire sensing vs fire spread

Sensor validation and fire-spread validation are related but distinct. Real controlled fire trials primarily calibrate the sensing/evidence pathway. Geospatial spread behavior is validated with controlled synthetic or empirical benchmark scenarios plus sensitivity tests.

## 27.2 Calibration sequence

Lock final fuel class map and provenance for the demo region.

Establish baseline R0 values/behavior from declared sources or controlled calibration assumptions.

Establish moisture response calibration/parameter ranges.

Validate wind direction and speed response independently.

Validate slope response independently.

Validate combined wind+slope behavior.

Validate heterogeneous fuel behavior.

Validate barriers and geometry.

Benchmark runtime on the actual Master/cloud target.

## 27.3 No pseudo-accuracy

Do not report a single 'fire spread accuracy = 94%' unless a scientifically appropriate validation protocol and reference observations support that statement. For SIH, report model assumptions, sensitivity tests, controlled validation observations, and measured runtime.

## 27.4 Reference-image/video visual quality

The physics engine and visualization polish are separate acceptance dimensions. A smooth UI cannot compensate for incorrect physics, and correct physics can be visually polished independently. Any reference video comparison must therefore be assessed on (a) physical behavior, (b) geospatial geometry, (c) temporal evolution, and (d) rendering quality.

# 28. Performance & Runtime

## 28.1 Compute target

The spread engine runs on Raspberry Pi/cloud-class compute, not on ESP32. The first benchmark target is the actual Raspberry Pi hardware intended for the demo.

## 28.2 Metrics

| Metric | Definition |
| --- | --- |
| Grid size | Rows × columns. |
| Cells evaluated | Number of cells popped/relaxed or otherwise processed. |
| Queue operations | Push/pop count. |
| Wall time | Simulation runtime. |
| Memory peak | Maximum RAM used by raster/queue/geometry. |
| Polygonization time | Time from mask to valid geometry. |
| Risk surface time | Time to compute hazard risk raster. |
| End-to-end update | Incident update to dashboard render. |

## 28.3 Performance optimization order

Use efficient numeric arrays rather than Python object-per-cell loops where possible.

Keep priority queue implementation efficient.

Limit simulation domain to required geospatial extent.

Avoid recomputing immutable terrain/fuel preprocessing on every update.

Cache DEM/fuel transforms and display-ready data.

Regenerate future wavefront only when material input changes.

Optimize polygonization only after correctness is proven.

# 29. Storage & Provenance

## 29.1 Required persisted metadata

| Artifact | Persist |
| --- | --- |
| Spread run | incident_id, run_id, timestamps, mode, parameter-set ID, domain, CRS. |
| Environmental context | wind, moisture, temperature/context inputs used by run. |
| Terrain | dataset ID/version, CRS, preprocessing version. |
| Fuel | dataset/config ID, version/provenance, classification status. |
| Arrival field | file/reference ID, transform, resolution, source run. |
| Geometry | geometry version, source arrival run, generated timestamp. |
| Risk surface | formula/config version, source arrival run. |
| Assumptions | human-readable list of fallbacks/limitations active during run. |

## 29.2 Reproducibility

Given the same immutable inputs, configuration and model version, a spread run should be reproducible to the defined numerical tolerance. Store enough metadata to reconstruct that condition.

# 30. Recommended Software Module Structure

geospatial/
  fire_spread/
    domain.py
    grid.py
    wind.py
    terrain.py
    fuel.py
    moisture.py
    ros.py
    propagation.py
    arrival_time.py
    recompute.py
  affected_area/
    thresholds.py
    polygonize.py
    geometry_validity.py
    multipolygon.py
    buffers.py
  risk/
    urgency.py
    intensity.py
    escalation.py
    surface.py
    classification.py
  datasets/
    dem.py
    fuel.py
    population.py
    infrastructure.py
  io/
    raster_store.py
    geojson.py
    provenance.py
tests/
  fire_spread/
  affected_area/
  risk/
  fixtures/

## 30.1 Module ownership

| Module | Owns |
| --- | --- |
| wind.py | FROM→TO, wind vector, midflame adjustment. |
| terrain.py | DEM sampling, slope/aspect. |
| fuel.py | Fuel profiles/class lookup. |
| moisture.py | Moisture/context normalization. |
| ros.py | Base and directional ROS. |
| propagation.py | Priority queue / arrival-time field. |
| recompute.py | Mid-run environment updates. |
| polygonize.py | Raster-to-geometry. |
| geometry_validity.py | Validity checks/repair. |
| buffers.py | Operational policy buffer. |
| surface.py | Hazard risk raster. |
| classification.py | Risk bands. |

# 31. Service/API Contract

| Endpoint | Purpose |
| --- | --- |
| GET /api/v1/incidents/{id}/spread | Current spread summary and geometry references. |
| GET /api/v1/incidents/{id}/spread/run/{run_id} | Detailed spread-run metadata/output. |
| GET /api/v1/incidents/{id}/spread/arrival | Arrival-time source/tiles/reference where exposed. |
| GET /api/v1/incidents/{id}/spread/risk | Hazard risk surface metadata/source. |
| GET /api/v1/incidents/{id}/affected-area | Current/warning/projection/buffer geometry. |

## 31.1 Conceptual response

{
  "incident_id": "INC-FIRE-0041",
  "mode": "LIVE",
  "run_id": "RUN-0017",
  "updated_at": "...",

  "environment": {
    "wind_speed_mps": 4.8,
    "wind_from_deg": 240,
    "wind_to_deg": 60,
    "moisture_index": 0.34
  },

  "geometry": {
    "current": "GeoJSON/feature-reference",
    "warning": "GeoJSON/feature-reference",
    "projection": "GeoJSON/feature-reference",
    "operational_buffer": "GeoJSON/feature-reference"
  },

  "metrics": {
    "current_area_m2": 18420,
    "warning_area_m2": 27100,
    "projection_area_m2": 39200,
    "area_growth_m2_per_min": 510.2
  },

  "risk_surface": {
    "kind": "HAZARD_RISK_INDEX",
    "probability": false,
    "source": "..."
  },

  "assumptions": [
    "Simplified fuel classes",
    "Projection assumes current environmental conditions until next update"
  ]
}

# 32. Simulation Scenarios

## 32.1 Minimum scenario library

| Scenario | Purpose |
| --- | --- |
| S01 Uniform calm | Geometry sanity; circle/diagonal test. |
| S02 Wind rotation | Directional behavior. |
| S03 Wind + slope | Combined vector behavior. |
| S04 Fuel mosaic | Heterogeneous spread. |
| S05 Firebreak | Barrier routing. |
| S06 Moisture increase | Suppression response. |
| S07 Rapid escalation | Response/update timing. |
| S08 Wind shift mid-run | Recompute future wavefront. |
| S09 Domain edge | Boundary behavior. |
| S10 Disconnected fuel islands | MultiPolygon. |
| S11 Missing wind | Degraded assumption. |
| S12 Missing DEM | Terrain degraded. |
| S13 Invalid geometry | Repair/flag behavior. |
| S14 Multi-hazard context | Fire risk remains independent from other hazard risk. |

## 32.2 Scenario metadata

Every scenario must identify which variables are held constant and which are changed. This makes sensitivity tests interpretable.

# 33. SIH Demo Behavior

## 33.1 Fire demo narrative

REAL EVENT
  ↓
ESP32 detects validated fire evidence
  ↓
Incident becomes CONFIRMED/CRITICAL
  ↓
Master starts Fire Spread Engine
  ↓
terrain + fuel + moisture + wind
  ↓
directional ROS
  ↓
arrival-time field
  ↓
current footprint
  ↓
warning + projection
  ↓
risk heatmap
  ↓
affected population/infrastructure
  ↓
response recommendation
  ↓
authority dashboard + citizen alert

## 33.2 Technical judge interaction

The operator must be able to show the model rather than merely the animation: wind direction, terrain/fuel assumptions, current footprint, projection horizon, and the explanation that geometry is derived from the arrival-time field.

## 33.3 LIVE→SIMULATION

After the live event, switch to SIMULATION. A scenario generates telemetry through the standard ingestion path. The same Fire Spread page then renders the resulting incident. The fact that the screen and downstream outputs remain structurally identical is part of the proof that simulation is not a separate toy system.

# 34. Explicit V1 Non-Goals

| Feature | Decision |
| --- | --- |
| Full CFD / combustion physics | DO NOT BUILD. |
| FIRETEC/WFDS-class simulation | DO NOT BUILD. |
| Ember spotting/reignition | DO NOT BUILD. |
| Dynamic full atmospheric fluid field | DO NOT BUILD. |
| Full 13/40 fuel-model operational library | OUTSIDE V1; use simplified classified scheme. |
| Full live satellite dependency | DEFERRED. |
| 3D globe/terrain | DO NOT BUILD for V1. |
| Custom vector-tile pipeline | DO NOT BUILD for demo scale. |
| Pixel-perfect imitation of a reference video without matching physics | DO NOT BUILD. |

# 35. Fire/Geospatial Invariants

🔒 01  Fire spread is computed from an arrival-time field, not a decorative animation.

🔒 02  Meteorological wind FROM direction is converted to TO before propagation.

🔒 03  Diagonal grid distance is Δ√2.

🔒 04  Wind and slope are treated directionally through a common vector/resultant model.

🔒 05  Base ROS, moisture response and directional response remain explicit and configurable.

🔒 06  Non-burnable cells have zero propagation and route the front around barriers.

🔒 07  Physical footprint is arrival-time-derived.

🔒 08  Operational buffer is a separate policy layer.

🔒 09  Warning/projection are thresholded from the same arrival-time field.

🔒 10  Projection is conditional on stated assumptions.

🔒 11  Environmental changes trigger recomputation from current perimeter for future propagation.

🔒 12  Simulation and live use the same fire-spread engine.

🔒 13  Area/distance/buffer calculations use an appropriate metric CRS/geodesic method.

🔒 14  Polygon outputs are validity-checked.

🔒 15  MultiPolygon is supported.

🔒 16  Display simplification never changes scientific/source geometry.

🔒 17  Risk heatmap is an operational index, not a probability.

🔒 18  Population/infrastructure exposure does not redefine physical hazard risk.

🔒 19  Fire and other hazard risk surfaces remain independently represented.

🔒 20  Missing geodata produces explicit degraded/unknown behavior.

🔒 21  Fire spread runs on Master/cloud compute, never on ESP32.

🔒 22  Every run records parameter and data provenance sufficient for reproduction.

# 36. Implementation Readiness Checklist

☐ 01  Demo-region CRS selected and persisted.

☐ 02  DEM dataset selected, preprocessed and provenance recorded.

☐ 03  Simplified fuel map created, versioned and labeled.

☐ 04  Moisture proxy defined and parameterized.

☐ 05  Wind source defined; FROM→TO verified in code.

☐ 06  Midflame wind adjustment parameter exists.

☐ 07  Directional wind+slope vector model implemented.

☐ 08  Directional ROS/ellipse function implemented and unit-tested.

☐ 09  8-neighbor grid implemented with Δ and Δ√2 distances.

☐ 10  Priority-queue propagation implemented.

☐ 11  Arrival-time field persisted/recoverable.

☐ 12  Current/warning/projection generated only from arrival field.

☐ 13  Mid-run environment recompute implemented.

☐ 14  Non-burnable routing tested.

☐ 15  MultiPolygon supported.

☐ 16  Geometry validity/repair implemented.

☐ 17  Physical footprint and operational buffer are separate outputs.

☐ 18  Hazard Risk Surface implemented separately from footprint/exposure.

☐ 19  Risk surface clearly labeled non-probabilistic.

☐ 20  LIVE and SIMULATION use identical spread engine.

☐ 21  Runtime benchmark completed on actual Master target.

☐ 22  All mandatory GEO-* tests pass.

# 37. Source & Review Basis

This final specification incorporates the prior NexAlert fire/geospatial design and the adversarial review. The review explicitly confirmed that the arrival-time/event-based approach is appropriate for the SIH scope, while identifying concrete corrections: wind FROM→TO conversion, diagonal √2 distance, vector wind+slope combination, projected CRS, mid-run future-wavefront recomputation, MultiPolygon support, geometry validity repair, operational-buffer separation, and independent hazard risk layers. fileciteturn19file1 fileciteturn19file16

The dashboard reference further establishes that the Fire Spread page must render the same backend arrival-time solution rather than inventing a separate visual animation, and that current/warning/projection plus risk/fuel/terrain overlays are derived from the model outputs. fileciteturn19file2 fileciteturn19file13

The architecture baseline also establishes that external satellite data is additive/deferred rather than a dependency for minute-scale local decisions, and that the V1 fire pathway is manually configured/calibrated rather than a trained classifier. fileciteturn19file15