NEXALERT

HARDWARE, EDGE NODE, SENSOR & TELEMETRY SPECIFICATION

Final V1 Engineering Baseline — Field Hardware, Embedded Runtime, Diagnostics & Data Contract

Document role This is the definitive V1 hardware/edge specification for the field node. It defines the physical sensing boundary, ESP32-S3 firmware responsibilities, sensor interfaces, timing, diagnostics, telemetry, authentication, buffering, local emergency service, embedded intelligence boundary, and the exact contract presented to the Master.

| Field | Value |
| --- | --- |
| Product | NexAlert |
| Problem | SIH26178 — resilient AI-powered environmental monitoring and early warning |
| Document | 06 — Hardware, Edge Node, Sensor & Telemetry Specification |
| Status | FINAL V1 ENGINEERING BASELINE |
| Primary field compute | ESP32-S3-class MCU |
| Primary field firmware | ESP-IDF + C/C++ |
| Master | Raspberry Pi-class Linux compute |
| Primary purpose | Measure → validate → diagnose → reason locally → authenticate → communicate → buffer |
| Heavy geospatial compute | NOT on ESP32 |

Historical architecture warning Earlier NexAlert project drafts used a dual-MCU STM32 + ESP32 slave-node architecture, Meshtastic forks and a different responsibility split. That is NOT the current V1 architecture. This document follows the frozen architecture: ESP32-S3-class field node + Raspberry Pi Master, with local intelligence on the node and heavy geospatial/operational computation on the Master/cloud. The older STM32-centric proposal is reference history only. fileciteturn20file13 fileciteturn20file14

# Document Control & Authority

| Status | Meaning |
| --- | --- |
| LOCKED | Must be preserved unless hardware validation reveals a genuine incompatibility. |
| PARAMETER | To be selected/measured/calibrated. |
| CANDIDATE | A proposed hardware component/implementation that is not yet final. |
| DEFERRED | Architecturally supported but not required for V1. |
| ROADMAP | Future enhancement. |

## Authority order

| Topic | Authority |
| --- | --- |
| Overall responsibility split | 01 — Master Architecture |
| Technical requirements | 02 — TRD |
| Runtime/compute allocation | 03 — Technology Stack |
| Intelligence math | 04 — Mathematical Intelligence |
| Fire/geospatial calculations | 05 — Fire/Geospatial |
| This document | 06 — Hardware/Edge |

Component-selection status The architecture requires final selection of specific gas/PM and other sensor components. Existing NexAlert material mentions a Bosch BME688 for temperature/humidity/pressure/IAQ, but that component choice is not treated as a universal locked sensor assignment here. Final hardware selection must satisfy the signal contract, calibration requirements, power budget and availability. The previous slide deck listed BME688, ESP32-S3, SX1262/SX1276, LiFePO4 and solar hardware as a prior stack, but several of those choices are now treated as candidates unless explicitly revalidated. fileciteturn20file4

# 1. Field Node Purpose

The NexAlert field node is a distributed environmental sensing and edge-intelligence device. It is deliberately not just a sensor relay. It must be able to determine whether its own measurements are usable, whether they are unusual, whether the pattern provides hazard evidence, and whether the local hazard state warrants local action/notification—even when the Master, cloud or internet is unavailable.

## 1.1 Primary responsibilities

Acquire environmental measurements.

Normalize/validate sensor readings at the earliest practical point.

Track sensor diagnostic state and availability.

Monitor battery/power state separately from sensor health.

Maintain a heartbeat.

Compute local sensor health, signal quality and evidence reliability.

Maintain baseline readiness and controlled baseline updates.

Compute bounded anomaly measures.

Evaluate configured lightweight hazard reasoning locally.

Produce confidence, severity, operational-risk and hazard-state outputs.

Authenticate telemetry using a per-node credential/HMAC.

Maintain sequence numbers and timestamps.

Buffer telemetry/events while communications are unavailable.

Provide the local NexAlert emergency network/service path assigned to the selected hardware design.

Recover cleanly after reset/reconnection.

## 1.2 Explicit non-responsibilities

Large-scale fire-spread raster simulation.

DEM preprocessing and slope/aspect raster generation.

Population/infrastructure intersection.

Route optimization.

Authority dispatch decisions.

Cloud-only functionality.

A mandatory learned fire classifier.

Full operational GIS.

# 2. Hardware Platform

## 2.1 MCU baseline

The V1 field node uses an ESP32-S3-class MCU as the embedded compute target. Exact board/flash/PSRAM configuration must be recorded once the physical board is finalized.

| Item | V1 requirement |
| --- | --- |
| MCU family | ESP32-S3-class |
| Firmware environment | ESP-IDF + C/C++ |
| Runtime | FreeRTOS through ESP-IDF |
| Nonvolatile storage | On-board flash and/or configured external storage as required for buffering/configuration. |
| Connectivity | Wi-Fi/local IP plus the selected field-network transport where applicable. |
| Power | Solar/battery subsystem according to hardware team's final design. |
| Time source | RTC/NTP/GPS or other configured time source; domain semantics must survive intermittent connectivity. |

## 2.2 Why ESP32-S3

The architecture uses the ESP32-S3 as the field compute target because the required local intelligence operations are primarily bounded arithmetic, rolling statistics, comparisons, small buffers, and configuration-driven reasoning. The principal engineering challenge is concurrent execution of sensing, communications, local service, buffering, authentication and intelligence, which must be benchmarked rather than assumed safe.

## 2.3 Board selection record

| Field | Must be recorded before hardware freeze |
| --- | --- |
| Exact board/module | Manufacturer + module/board identifier. |
| Flash | Capacity and allocation. |
| PSRAM | Present/absent and capacity. |
| GPIO map | Per sensor/peripheral. |
| ADC/DAC | Channels/resolution used. |
| I2C | Bus address map. |
| SPI | Bus/device map. |
| UART | Device/console map. |
| Wi-Fi role | AP/client/concurrent mode configuration. |
| Power input | Voltage/current range. |
| Enclosure | Weatherproofing/rating and thermal assumptions. |

# 3. Sensor Architecture

## 3.1 Canonical sensor classes

| Signal | Canonical unit | Primary hazard relevance | Status |
| --- | --- | --- | --- |
| Temperature | °C | Fire, extreme heat, contextual pollution | REQUIRED class |
| Relative humidity | %RH | Fire/context, heat | REQUIRED class |
| Pressure | hPa | Context/weather; optional | OPTIONAL |
| PM2.5 | µg/m³ | Fire/smoke, pollution | RECOMMENDED; exact sensor pending |
| PM10 | µg/m³ | Fire/smoke, pollution | RECOMMENDED |
| Gas | ppm only if calibrated; otherwise sensor-native engineering unit | Fire/pollution | REQUIRED capability; exact sensor pending |
| Water level | m (cm acceptable if schema is normalized) | Flood | REQUIRED for physical/synthetic flood path |
| Rainfall rate | mm/h | Flood | REQUIRED capability for flood path where hardware supports it |
| Cumulative rainfall | mm | Flood | RECOMMENDED |
| Soil moisture | %VWC or calibrated native unit | Flood/landslide | RECOMMENDED |
| Vibration | m/s² or calibrated native unit | Landslide/structural context | RECOMMENDED |

## 3.2 Fire minimum

A fire-capable node must have a combination of temperature, smoke/PM and gas capabilities subject to final sensor calibration. Humidity is supporting evidence. A single sensor is never the entire fire decision.

## 3.3 Flood minimum

A flood-capable hardware configuration requires a water-level signal and rainfall/context input; soil moisture is useful supporting evidence. The exact hardware arrangement may use a dedicated node configuration or the same multi-sensor node.

## 3.4 Gas sensor rule

Gas sensor must not be a name-only decision A specific gas sensor must be selected based on the actual gas species or response quantity, calibration process, cross-sensitivity, response time, operating voltage, temperature dependence, and available driver/library support. If it cannot honestly be expressed in calibrated ppm, the telemetry must use the sensor's declared native engineering unit rather than a fabricated ppm conversion.

# 4. Electrical & Software Sensor Interfaces

## 4.1 Interface conventions

| Interface | Expected use |
| --- | --- |
| I²C | Temperature/humidity/pressure/air-quality class sensors where supported. |
| SPI | High-rate sensors, radios, displays or storage where applicable. |
| UART | Smart sensors, GNSS or external co-processors where required. |
| ADC | Analog gas/water/vibration/level inputs where final sensor requires it; must include calibration. |
| GPIO/interrupt | Rain gauge pulses, threshold interrupts, sensor-ready signals where applicable. |

## 4.2 Per-sensor configuration record

sensor_id
node_id
sensor_type
manufacturer
model
interface
bus/channel/address
unit
sampling_interval
calibration_status
calibration_version
valid_range
warmup_time
saturation_range
diagnostic_rules
driver_version
enabled
hazard_roles[]

## 4.3 Electrical assumptions

The final hardware document must record actual logic levels, supply voltage, pull-ups, ADC reference behavior, sensor warm-up, current draw and interface timing for every selected part. The software specification must never invent a sensor value that the electrical design cannot physically measure.

# 5. Sampling, Timing & Time Semantics

## 5.1 Observation cadence

Each sensor class gets a configured sampling interval based on its dynamics, power cost, driver limitations and hazard role. There is no requirement that every sensor sample at the same rate.

| Signal class | Typical V1 cadence direction | Reason |
| --- | --- | --- |
| Fast smoke/PM changes | Seconds to tens of seconds | Fire/pollution events can change quickly. |
| Temperature/humidity | Seconds to minutes | Environmental dynamics slower than PM. |
| Water level | Seconds to minutes | Flood rate-of-rise requires temporal resolution. |
| Rainfall | Event/pulse-based or short interval | Cumulative and rate calculations. |
| Soil moisture | Minutes | Slow-changing. |
| Vibration | Windowed high-rate acquisition then features | Frequency content may be more informative than raw samples. |

These are cadence classes, not locked numerical sampling frequencies. Exact intervals are parameters validated against real hardware/power budgets.

## 5.2 Timestamps

Each observation must retain a measurement timestamp. Node-generated timestamps should use a synchronized or monotonic time basis where possible. The backend records a separate receive timestamp.

## 5.3 Sequence numbers

Every telemetry packet must carry a per-node sequence number or equivalent monotonic identifier. Sequence values support duplicate detection, missing-packet diagnosis and store-and-forward reconciliation.

# 6. Diagnostics & Sensor Health

## 6.1 Diagnostic signals

| Diagnostic | Examples |
| --- | --- |
| Self-test | Sensor internal self-test/pass/fail. |
| Availability | Present/absent/responding. |
| Communication integrity | I²C/SPI/UART errors, retries/timeouts. |
| Calibration | Valid/expired/invalid/calibration version. |
| Stability | Short-window noise/variance. |
| Drift | Longer-term deviation from expected behavior. |
| Saturation | ADC/sensor max/min or declared saturation. |
| Electrical | Impossible voltage/current/state when available. |
| Warm-up | Sensor still in startup/warm-up. |

## 6.2 Sensor health

H_i^soft = Σ_j w_ij D_ij,   Σ_j w_ij = 1

H_i = 0  if F_i=1;  otherwise H_i = H_i^soft

The weighted diagnostic score and hard-failure gate are defined in Document 04. The embedded firmware must expose the component diagnostics that justify the scalar.

## 6.3 Battery is separate

Battery percentage, solar activity and power mode are operational quantities. They are not copied into H_i. Only an actual power-induced sensor degradation may reduce sensor health/availability.

## 6.4 Hard failure examples

Definitive self-test failure.

Sensor physically disconnected.

Sustained bus communication failure.

Impossible electrical state.

Sensor-specific fatal error.

# 7. Signal Quality & Reliability on the Node

## 7.1 Signal quality

Q_i(t) = q_integrity,i(t) × q_stability,i(t)

q_integrity represents current data validity; q_stability represents short-window consistency/noise. The embedded implementation must compute/approximate these using bounded, resource-appropriate algorithms.

## 7.2 Evidence reliability

R_i(t) = H_i(t) × Q_i(t) × K_i(t)

K_i captures current calibration/operating validity. The node uses R_i to weight hazard evidence. It does not add R_i again as a separate confidence factor.

## 7.3 Example

A healthy sensor with low current quality can legitimately have high H_i but low Q_i and therefore lower R_i. This distinction is useful during transient communication/noise problems.

# 8. Edge Baseline & Anomaly

## 8.1 Baseline states

| State | Node behavior |
| --- | --- |
| INITIALIZING | Collect initial observations. |
| LEARNING | Adapt under controlled rate. |
| READY | Normal anomaly interpretation available. |
| FROZEN | No baseline learning during confirmed/critical hazard. |
| RECOVERING | Controlled post-event relearning. |

## 8.2 Default robust baseline

m_i = median(x_i)

MAD_i = median(|x_i − m_i|)

s_i = 1.4826 × MAD_i + ε

z_i = (x_i − m_i) / s_i

## 8.3 Anomaly

A_i(t) = 1 − exp(−min(|z_i|, z_cap)/λ)

A_node(t) = [Σ_i R_i A_i] / [Σ_i R_i + ε]

A_h(t) = [Σ_i w_ih R_i A_i] / [Σ_i w_ih R_i + ε]

## 8.4 Modality-specific behavior

Water level and soil moisture may use trend-relative methods; vibration may use frequency-domain features. The common pipeline stays unchanged.

## 8.5 Edge resource rule

Do not maintain unbounded time series on the ESP32. Use ring buffers/windows sized to the exact feature/baseline configuration. Raw history should be persisted upstream or in bounded chunks when required.

# 9. Local Hazard Reasoning Boundary

## 9.1 Configuration-driven reasoning

The node evaluates each configured hazard independently. Fire reasoning is deterministic/manual-calibration driven in V1. Flood follows the same common framework with hazard-specific evidence rules.

ObservationFrame
  ↓
Diagnostics
  ↓
Baseline / features
  ↓
Anomaly
  ↓
Fire reasoning ──→ Fire assessment
Flood reasoning ─→ Flood assessment
Pollution ───────→ Pollution assessment
Landslide ───────→ Landslide assessment
Heat ────────────→ Heat assessment
  ↓
confidence / severity / risk / state
  ↓
Intelligence packet

## 9.2 Local state

The node may produce a local hazard state that can drive local safety behavior. The Master may build a separate regional state later. The Master must not erase a valid local state merely because other nodes disagree.

## 9.3 Local response

A local node may trigger configured local alert behavior (e.g., local emergency page state) according to policy. It must never autonomously dispatch external responders.

# 10. Local Emergency Network / Citizen Access

## 10.1 Purpose

The field node/local gateway must be capable of exposing the NexAlert emergency experience through the local network path when configured for V1. The exact web-serving architecture depends on the final memory/flash budget and network topology.

## 10.2 Minimal local service

Node enters CRITICAL/local alert condition
  ↓
Local emergency service becomes available
  ↓
Distinctive NexAlert Wi-Fi presence
  ↓
Citizen associates with network
  ↓
Local emergency page served
  ↓
Verified event rendered

## 10.3 Important limitation

The system cannot forcibly open a webpage on an arbitrary nearby phone that has never associated with the NexAlert network. The citizen must participate through a supported local connection path. This is a platform/operating-system constraint, not a software bug.

## 10.4 Master independence

Master-death requirement The local citizen path must not depend exclusively on the Raspberry Pi Master being alive. The final hardware design must therefore reserve enough node/gateway capability to serve the emergency experience and retain a minimal emergency state when the Master is unavailable.

# 11. Canonical Telemetry Packet

## 11.1 Packet requirements

{
  "schema_version": "1.0",
  "telemetry_id": "TEL-01JXYZ",
  "node_id": "NODE-001",
  "sequence": 1842,

  "measurement_timestamp": "...",
  "device_monotonic_ms": 12345678,

  "location": {
    "lat": 13.1542,
    "lon": 77.5881
  },

  "measurements": {
    "temperature_c": 42.4,
    "humidity_pct": 31.2,
    "pressure_hpa": 1007.4,
    "pm25_ug_m3": 186.0,
    "pm10_ug_m3": 271.0,
    "gas_value": 14.2,
    "gas_unit": "ppm",
    "water_level_m": null,
    "rainfall_mm_h": 0.0,
    "rainfall_cumulative_mm": 2.1,
    "soil_moisture_pct_vwc": null,
    "vibration_m_s2": 0.04
  },

  "diagnostics": {
    "self_test": "PASS",
    "communication_integrity": "GOOD",
    "calibration_status": "VALID",
    "stability": "GOOD",
    "availability": "GOOD",
    "saturation": false
  },

  "power": {
    "battery_pct": 87,
    "solar": "ACTIVE"
  },

  "source": "HARDWARE",
  "auth": {
    "algorithm": "HMAC-SHA256",
    "signature": "..."
  }
}

## 11.2 Missing vs zero

| Value | Meaning |
| --- | --- |
| null / explicit missing status | No usable measurement is available. |
| 0.0 | Sensor actually reports a measured zero. |

Never Do not serialize an unavailable water-level/gas/rain/sensor field as 0.0. Missingness is semantically meaningful to the intelligence layer.

# 12. Measurement Semantics & Validation

| Field class | Validation examples |
| --- | --- |
| Temperature | Finite numeric; sensor-specific range; reject NaN/Inf. |
| Humidity | 0–100 %RH unless sensor declares another valid representation. |
| Pressure | Finite; configurable plausible bounds. |
| PM2.5/PM10 | Finite non-negative; saturation explicitly flagged. |
| Gas | Valid native/calibrated unit; no fake ppm conversion. |
| Water level | Finite non-negative or explicitly signed if sensor requires datum convention. |
| Rainfall rate | Finite non-negative mm/h. |
| Cumulative rain | Non-decreasing except explicit reset/re-baseline event. |
| Soil moisture | Declared unit/range. |
| Vibration | Finite; preserve unit/calibration. |
| Timestamp | Must pass sanity checks; future/too-old values flagged. |

## 12.1 Validation outcomes

| Outcome | Meaning |
| --- | --- |
| ACCEPT | Usable observation. |
| ACCEPT_WITH_FLAGS | Usable but has a non-fatal diagnostic condition. |
| REJECT | Malformed, unauthenticated or invalid observation. |
| BUFFER | Valid packet that cannot currently be forwarded. |

## 12.2 Validation order

receive
→ authenticate
→ parse
→ schema validate
→ timestamp/sequence validate
→ range/unit validate
→ attach diagnostics
→ persist/buffer
→ intelligence

# 13. Node Authentication & Integrity

## 13.1 V1 node-to-Master authentication

Each node receives a unique provisioned secret. Telemetry packets are authenticated with HMAC-SHA256 or the final approved HMAC construction. The Master verifies the packet before accepting it into canonical ingestion.

node secret
   +
canonical serialized packet
   ↓
HMAC-SHA256
   ↓
signature
   ↓
Master verification

## 13.2 Replay protection

HMAC alone does not prevent replay. Use telemetry_id + sequence + measurement timestamp (and/or explicit nonce where required) so duplicate/stale packets can be rejected.

## 13.3 Key storage

Do not hard-code production secrets in public source.

Store per-node secret in protected nonvolatile storage appropriate to the board and threat model.

Provision a unique credential per node.

Support credential rotation/revocation at the backend level where practical.

## 13.4 Invalid packet

HMAC invalid
→ reject
→ security/audit event
→ no intelligence processing
→ no incident side effect

# 14. Local Buffering & Store-and-Forward

## 14.1 Why buffering exists

Disaster networks can be intermittent. A node should not lose every measurement simply because its communication path is temporarily down.

## 14.2 Buffer record

buffer_id
telemetry_id
node_id
sequence
measurement_timestamp
payload
auth_metadata
stored_at
retry_count
priority
status

## 14.3 Buffer policy

| Rule | V1 behavior |
| --- | --- |
| Ordering | Prefer per-node sequence order. |
| Deduplication | Same telemetry_id/sequence must not be ingested twice. |
| Retry | Bounded retry with backoff. |
| Full buffer | Apply explicit ring-buffer/retention policy; never unlimited memory growth. |
| Critical event | Higher transmission priority than routine telemetry where transport supports priority. |
| Recovery | Sync buffered data then resume normal flow. |

## 14.4 Synchronization

link restored
→ authenticate
→ send oldest eligible buffered records
→ receive ACK
→ mark synchronized
→ continue
→ deduplicate at receiver as a second defense

# 15. Heartbeat & Node Health Reporting

## 15.1 Heartbeat

The node must emit a lightweight heartbeat on the configured cadence. Existing NexAlert design uses a heartbeat mechanism; the intelligence layer extends this into component-level health reporting while fast diagnostics remain internal. The exact cadence remains a parameter.

| Heartbeat field | Purpose |
| --- | --- |
| node_id | Device identity. |
| sequence/heartbeat_seq | Ordering. |
| uptime | Node runtime health. |
| firmware_version | Compatibility. |
| last_sensor_sample_age | Sampling health. |
| battery_pct | Power state. |
| solar_state | Power state. |
| communications | Link state/quality summary. |
| storage_available | Buffer capacity. |
| sensor_health_summary | Aggregate plus per-sensor diagnostics reference. |
| current_local_state | Local hazard state summary. |

## 15.2 Approximate five-minute detailed health report

A component-level health report may be transmitted approximately every five minutes, while faster diagnostics continue internally. The final interval is configurable and must be benchmarked for bandwidth/power impact.

## 15.3 Staleness

Age = t_now − t_last_heartbeat

A heartbeat becomes stale after the configured timeout. Staleness changes node/network state; it does not automatically mean every sensor is physically dead.

# 16. Firmware Architecture

firmware/
  main/
  components/
    sensors/
    diagnostics/
    intelligence/
    telemetry/
    security/
    network/
    storage/
    local_gateway/
    system/
  test/

## 16.1 Logical task set

| Task/module | Purpose |
| --- | --- |
| Sensor acquisition | Sample sensors on schedule. |
| Diagnostics | Self-test, quality, availability, calibration. |
| Intelligence | Baseline, anomaly, hazard reasoning, state. |
| Telemetry | Serialize, sequence, metadata. |
| Security | HMAC, key access, replay protection. |
| Network | Transmit/reconnect/local network. |
| Storage | Ring buffer and persistence. |
| Local gateway | Emergency web/portal service where enabled. |
| System | Watchdog, time sync, configuration, logging. |

## 16.2 Concurrency principle

Do not create one FreeRTOS task per sensor by default. Start with a small bounded task architecture and use queues/event groups/timers. Increase concurrency only after measurement shows a bottleneck.

## 16.3 Watchdog/restart

Enable a watchdog appropriate to the final task model.

A blocked sensor driver must not indefinitely block telemetry/network tasks.

A network reconnect loop must have bounded delays.

After reboot, buffered valid data and configuration must be recovered safely.

# 17. Edge Intelligence Implementation

## 17.1 What may run locally

| Function | Embedded status |
| --- | --- |
| Sensor health H_i | MUST |
| Signal quality Q_i | MUST |
| Reliability R_i | MUST |
| Baseline readiness | MUST |
| Anomaly A_i | MUST for configured sensors |
| Hazard evidence | MUST for configured lightweight hazards |
| Confidence | MUST |
| Severity | MUST |
| Operational risk | MUST |
| Hazard state | MUST |
| Fire spread | MUST NOT run on ESP32 |
| Population exposure | MUST NOT run on ESP32 |
| Routing | MUST NOT run on ESP32 |

## 17.2 Numerical constraints

Embedded mathematics must use bounded numeric ranges, preallocated buffers and validated floating-point or fixed-point representations appropriate to the final firmware. Do not use dynamic allocation inside high-frequency sensor loops unless measured safe.

## 17.3 Golden vector

Every shared formula that exists both in Python and on the ESP32 must have reference vectors. The Python implementation is the reference for numerical intent; the C/C++ result must stay within declared tolerance.

# 18. Field Communication Interface

## 18.1 Transport abstraction

The node software must isolate application telemetry from physical transport. This prevents replacing Wi-Fi with a long-range radio from changing the intelligence schema.

Application telemetry
      ↓
Transport-independent envelope
      ↓
HMAC/authentication
      ↓
Transport adapter
  ├─ Wi-Fi/IP
  ├─ optional long-range/radio adapter
  └─ future adapter

## 18.2 V1 practical transport

The current prototype may use Wi-Fi/IP for the simplest real-hardware path. Long-range radio may be added through an adapter where hardware availability and time justify it. A full LoRaWAN/mesh deployment is not required to satisfy the V1 software architecture.

## 18.3 Legacy note

Earlier project materials described SX1262/SX1276 radios, Meshtastic forks and an STM32 routing controller. Those belong to an earlier hardware architecture and must not be copied into the V1 firmware simply because they appear in legacy presentations. fileciteturn20file4

# 19. Calibration & Sensor Characterization

## 19.1 Calibration layers

| Layer | Examples |
| --- | --- |
| Factory calibration | Sensor manufacturer baseline. |
| Node-level calibration | Known reference comparison before deployment. |
| Environmental compensation | Temperature/humidity dependence if documented. |
| Operational validity | Calibration freshness/quality status. |
| Software conversion | Unit/offset/scaling conversion with version. |

## 19.2 Calibration record

calibration_id
sensor_id
performed_at
performed_by
reference_method
reference_values
raw_values
calibration_parameters
uncertainty/notes
valid_from
valid_until (if applicable)
firmware/driver_version

## 19.3 Fire trial

Controlled fire trials should measure sensor response repeatability, time-to-detection, cross-sensor agreement, baseline contamination risk, recovery behavior and false-alarm behavior. Do not turn a single trial into a claimed accuracy number.

# 20. Hardware Verification & Benchmarks

| Test | Required evidence |
| --- | --- |
| Sensor read test | Every selected sensor produces expected valid values. |
| Sensor disconnect | Firmware detects unavailable sensor. |
| Sensor saturation | Saturation flag/quality behavior is correct. |
| Noise test | Quality decreases under injected/observed noise. |
| Warm-up | No false hazard during declared warm-up. |
| Calibration invalid | Reliability/availability changes correctly. |
| Heartbeat | Periodic report visible at Master. |
| Duplicate packet | Receiver deduplicates. |
| HMAC invalid | Packet rejected. |
| Offline buffer | Data retained while link is down. |
| Reconnect | Buffered packets sync without duplication. |
| Wi-Fi AP stress | Local portal remains responsive during sensing/telemetry. |
| Concurrent intelligence | Local inference does not starve sensor/network tasks. |
| Master failure | Local citizen emergency page remains available. |

## 20.1 Mandatory ESP32 concurrency benchmark

| Metric | Measure under combined workload |
| --- | --- |
| Sensing latency | Time from scheduled acquisition to usable sample. |
| Sampling jitter | Worst-case deviation from intended sample time. |
| Inference latency | Time for local intelligence cycle. |
| Wi-Fi response | Request/connection responsiveness. |
| Telemetry latency | Packet creation/HMAC/send time. |
| Packet loss | Loss during representative load. |
| Free heap | Minimum free heap/high-water behavior. |
| CPU usage | Per-task/overall utilization. |
| Queue depth | Peak queue occupancy. |
| Storage usage | Buffer growth during outage. |
| Power | Relative energy draw where instrumentation permits. |

Benchmark gate The firmware architecture is not considered validated merely because it compiles. The combined workload—sensor acquisition + Wi-Fi/local service + telemetry + HMAC + buffering + local intelligence—must be measured on the actual ESP32-S3 hardware.

# 21. Power & Thermal Requirements

## 21.1 Power domains

| Domain | Requirement |
| --- | --- |
| MCU | Power budget measured in active/idle/network states. |
| Sensors | Measure continuous vs duty-cycled draw. |
| Radio/network | Measure transmit/connect duty. |
| Display/local portal | Treat as a separate power budget if present. |
| Solar | Record charge behavior and operating assumptions. |
| Battery | Record chemistry, capacity, BMS/protection. |

## 21.2 Low-power behavior

Low-power operation may reduce sampling cadence or disable nonessential functions when configured, but must not silently disable critical safety sensing without updating node health/information condition.

## 21.3 Thermal

Outdoor enclosure design must ensure sensor readings are not materially biased by MCU/PMIC heating. The final physical design must document ventilation/thermal-isolation strategy where relevant.

# 22. Field Deployment & Ruggedization

| Aspect | V1 requirement |
| --- | --- |
| Outdoor exposure | Enclosure and connectors suitable for intended demo conditions. |
| Water ingress | Physical design prevents direct exposure of electronics to rain/spray. |
| Dust | Protect sensing electronics while preserving measurement validity. |
| Mounting | Stable orientation for sensors that require it. |
| Cable management | Strain relief and secure connectors. |
| Serviceability | Sensors replaceable without destroying the whole node. |
| Labeling | Node ID and hazard-facing status physically identifiable. |
| Recovery | Power-cycle and firmware recovery procedure documented. |

## 22.1 Sensor placement

Placement affects measurements. Temperature/PM/gas sensors must not be placed where the node's own heat, enclosure, solar panel or battery exhaust produces a systematic false signal. Water-level and rain sensors require physically appropriate installation relative to the environment.

## 22.2 Fire safety

Controlled fire demonstrations must use a controlled environment, safe distance, suitable supervision and non-hazardous test procedures. The hardware specification does not authorize unsafe open-flame deployment.

# 23. Schema Versioning & Backward Compatibility

## 23.1 Schema version

Every telemetry packet carries schema_version. A Master should reject incompatible schema versions explicitly rather than guessing field meaning.

## 23.2 Firmware/backend compatibility

| Change | Behavior |
| --- | --- |
| Added optional field | Older receiver may ignore if forward-compatible. |
| Changed unit | Requires schema/version or explicit conversion contract; never silently reinterpret. |
| Renamed field | Versioned contract. |
| Changed semantics | Must increment schema version and update all consumers. |
| Firmware driver update | Telemetry carries driver/calibration version where relevant. |

# 24. Edge Error & Recovery State Machine

BOOT
 ↓
SELF_TEST
 ├─ FAIL → DEGRADED / SENSOR_UNAVAILABLE
 └─ PASS
      ↓
SYNC_TIME (if possible)
      ↓
LOAD_CONFIG
      ↓
INITIALIZING
      ↓
LEARNING
      ↓
READY
      ↓
RUNNING
 ├─ communication loss → BUFFERING
 │                         ↓ reconnect
 │                      SYNCING
 │                         ↓
 │                      RUNNING
 ├─ sensor fault → DEGRADED
 │                   ↓ recovery/self-test
 │                 RUNNING
 └─ fatal system condition → SAFE_RESTART
                              ↓
                              BOOT

## 24.1 Rule

A sensor fault must not crash the entire node. A network outage must not stop sensing. A temporary storage error must not create undefined memory behavior.

# 25. Intelligence Packet vs Raw Telemetry

## 25.1 Why both exist

Raw telemetry is required for audit, replay, debugging and downstream validation. The intelligence packet is a compact decision-oriented summary that can travel through constrained links.

| Packet | Use |
| --- | --- |
| Raw telemetry | Measurements, diagnostics, provenance, replay. |
| Intelligence packet | Current local hazard states, confidence/severity/risk, information condition and key explanatory metadata. |
| Heartbeat | Node/system liveness and health summary. |
| SOS/local event | Urgent event requiring higher transmission priority where transport supports it. |

## 25.2 Intelligence packet conceptual shape

{
  "node_id": "NODE-001",
  "sequence": 1842,
  "timestamp": "...",
  "information_condition": "GOOD",
  "hazards": {
    "fire": {
      "evidence": "...",
      "confidence": "...",
      "severity": "...",
      "operational_risk": "...",
      "state": "CONFIRMED"
    }
  },
  "diagnostics_summary": {},
  "auth": {}
}

Exact schema is maintained jointly with the backend specification. The important invariant is that the compact packet is derived from the same intelligence implementation that can be inspected through the raw evidence path.

# 26. Security & Privacy on the Node

| Concern | V1 requirement |
| --- | --- |
| Telemetry integrity | HMAC. |
| Replay | Sequence + timestamp + event identity/nonce. |
| Secret storage | Protected device storage; no public source secret. |
| Firmware integrity | Use signed/reproducible firmware workflow where practical. |
| Debug access | Disable/remove production debug paths not required for demo. |
| Citizen data | Minimize local retention. |
| Logs | No secrets; bounded storage. |

## 26.1 Privacy rule

The field node should not collect citizen identity/location data unless the local architecture explicitly requires it for the emergency portal. Hazard telemetry is the primary node responsibility; citizen identity is handled at the citizen/backend layer.

# 27. Hardware Implementation Order

Freeze the exact ESP32-S3 board/module and record flash/PSRAM/pin budget.

Freeze the sensor set and electrical interfaces.

Build a sensor-driver bring-up firmware.

Implement diagnostics and per-sensor health reporting.

Implement heartbeat/time/sequence.

Implement canonical telemetry serialization.

Implement HMAC authentication.

Implement buffering and reconnect.

Implement local intelligence subset.

Implement local emergency service.

Run the combined concurrency benchmark.

Run calibration/controlled fire trials.

Connect to the Raspberry Pi Master.

Run golden-vector parity tests.

Only then add field-enclosure/power polish and broader node replication.

# 28. Hardware V1 Freeze Checklist

☐ 01  Exact ESP32-S3 board selected and documented.

☐ 02  GPIO/I2C/SPI/UART/ADC map frozen.

☐ 03  Exact sensor part numbers selected and documented.

☐ 04  Gas sensor calibration method documented.

☐ 05  PM/smoke sensor selected and characterized.

☐ 06  Water/rain/soil/vibration sensor capabilities documented if included.

☐ 07  Sampling intervals configured.

☐ 08  Sensor warm-up and saturation behavior defined.

☐ 09  Diagnostic fields implemented.

☐ 10  H_i/Q_i/R_i inputs available on node.

☐ 11  Baseline/anomaly logic implemented in C/C++.

☐ 12  Python/C++ golden vectors pass.

☐ 13  Heartbeat implemented.

☐ 14  Sequence + measurement timestamp implemented.

☐ 15  Receive timestamp assigned at receiver.

☐ 16  HMAC implemented and invalid packet rejected.

☐ 17  Replay/dedup logic implemented.

☐ 18  Offline buffer and sync implemented.

☐ 19  Local emergency page survives Master failure.

☐ 20  ESP32 concurrency benchmark completed.

☐ 21  Power/thermal measurements collected.

☐ 22  Hardware demo test completed safely.

# 29. Hardware / Edge Invariants

🔒 01  The field node is ESP32-S3-class with ESP-IDF + C/C++ in V1.

🔒 02  Historical STM32 dual-MCU architecture is not part of the current V1 unless explicitly re-approved.

🔒 03  Battery state is not sensor health.

🔒 04  Missing measurement is not zero.

🔒 05  Every telemetry packet has node identity, sequence, measurement timestamp and schema version.

🔒 06  Receive timestamp is assigned at the receiving system and is distinct from measurement time.

🔒 07  Node telemetry is authenticated before canonical ingestion.

🔒 08  Invalid HMAC means reject and audit.

🔒 09  Duplicate packets do not create duplicate downstream effects.

🔒 10  Offline buffer is bounded; memory must never grow without limit.

🔒 11  Sensing continues when communication is unavailable.

🔒 12  A sensor fault does not crash the node.

🔒 13  Local intelligence does not require the Master/cloud/internet.

🔒 14  Fire spread does not run on the ESP32.

🔒 15  Population/infrastructure exposure does not run on the ESP32.

🔒 16  Python is the reference implementation for shared intelligence mathematics; C/C++ is the embedded implementation.

🔒 17  Shared Python/C++ functions are tested with golden vectors.

🔒 18  Fire V1 reasoning is deterministic/configuration-driven; no mandatory fire classifier.

🔒 19  Local hazard state and regional Master state are distinct.

🔒 20  Master disagreement must not erase valid local hazard behavior.

🔒 21  The node may provide local emergency citizen access through a supported local network path.

🔒 22  Master failure and internet failure are distinct conditions.

🔒 23  Every configured sensor has explicit units, validity rules and diagnostics.

🔒 24  Gas is only represented in ppm when a defensible calibration/unit conversion exists.

🔒 25  Exact numeric sampling/power/threshold parameters remain measured/configured rather than guessed.

# 30. Source & Reconciliation Notes

The V2.3 intelligence baseline defines the edge-first philosophy, local health/quality/reliability/baseline/anomaly/hazard reasoning boundary, Raspberry Pi regional responsibilities and the ESP32 implementation constraint. fileciteturn20file2

The older V2.2 document and legacy project proposal contain useful historical details such as heartbeat/health reporting, embedded workload considerations and prior hardware responsibilities, but the current architecture supersedes their STM32-centric and earlier implementation choices. fileciteturn20file1 fileciteturn20file13

The legacy deck previously listed BME688, SX1262/SX1276, Meshtastic, SQLite/WAL and STM32+ESP32. Those values are intentionally not copied into the current V1 lock without revalidation; the current technology/compute architecture uses ESP32-S3 + C/C++ at the field node and Raspberry Pi + Python/PostGIS/backend services at the Master. fileciteturn20file4