NEXALERT

MULTI-HAZARD & SYNTHETIC TELEMETRY SPECIFICATION

Final V1 Engineering Baseline — Hazard Scenario Modeling, Synthetic Telemetry, Replay, Multi-Node Correlation & Ground-Truth Separation

# Document Control & Authority

| Field | Value |
| --- | --- |
| Document ID | NexAlert FINAL 09 |
| Status | Final V1 Engineering Baseline |
| Scope | Multi-hazard representation, synthetic scenario generation, telemetry-only simulation, replay and evaluation controls |
| Authority | Master System Architecture + final intelligence, hardware and data baselines |
| Primary Consumers | Backend, AI/ML, simulation, frontend, QA, SIH demo engineering |
| Implementation Priority | Mandatory for a trustworthy simulation/evaluation loop; hazard models remain independently configurable |
| Key Invariant | The simulator produces telemetry; the reasoning pipeline must infer the hazard state exactly as it would for hardware data. |
| Historical Architecture Rule | Legacy STM32 split, Meshtastic-first designs and other superseded concepts are not implementation targets unless explicitly re-approved. |

# 1. Purpose and System Role

NexAlert must demonstrate more than isolated sensor thresholding. The system must show that the same distributed telemetry pipeline can observe, assess, correlate and communicate several environmental hazards while remaining honest about uncertainty, sensor quality, communication gaps and model limitations. This document defines the scenario and telemetry layer that makes that possible.

## 1.1 Objectives

Generate reproducible telemetry streams for normal conditions, gradual change, rapid onset, recovery and multi-hazard conditions.

Exercise the exact live ingestion, authentication, validation, quality, baseline, anomaly, evidence, confidence, severity, risk and state pipeline.

Model correlated multi-node observations rather than producing independent random sensor streams.

Inject realistic missingness, stale data, drift, noise, spikes, contradictions, communication loss and recovery.

Keep simulator-only truth and labels separate from telemetry so the reasoning layer cannot accidentally read the answer.

Enable deterministic replay for debugging, visual demonstration, regression testing and algorithm comparison.

Support fire as the hero hazard and flood, pollution, landslide and extreme heat as reusable hazard paths under the same framework.

## 1.2 Non-objectives for V1

Do not build a universal learned classifier that claims to recognize every hazard from arbitrary sensor combinations.

Do not fabricate scientific ground truth where no real-world reference exists.

Do not feed labels, future state, simulator hazard IDs or hidden scenario parameters into live-like inference payloads.

Do not collapse independent hazards into one combined probability or scalar risk score.

Do not require heavy geospatial or fire-spread computation on ESP32 edge nodes.

# 2. Multi-Hazard Representation

Every hazard is represented as an independent reasoning vector. Shared infrastructure is allowed; shared truth is not. A node can simultaneously carry evidence for more than one hazard, and the regional system can maintain multiple incident hypotheses at once.

| Hazard | Primary signals | Supporting signals | Core evidence concept | V1 reasoning posture |
| --- | --- | --- | --- | --- |
| Fire | temperature, smoke/PM, calibrated gas if available | humidity, wind/environment context | coherent rapid thermal + particulate/gas change across trusted nodes | deterministic evidence + geospatial spread engine; classifier optional |
| Flood | water level, rainfall | pressure, soil moisture, upstream context | level rise + rainfall/level trend + spatial corroboration | trend/threshold evidence with persistence |
| Pollution | PM2.5/PM10, gas | temperature, humidity, wind/context | sustained concentration elevation with spatial/temporal coherence | configuration-driven evidence |
| Landslide | vibration, soil moisture | rainfall, tilt/displacement when available | unusual vibration/displacement/soil response linked to triggering conditions | evidence groups; optional future geotechnical model |
| Extreme Heat | temperature | humidity, pressure/context | persistent elevated heat index / temperature pattern | duration-aware evidence |
| Unknown | any subset | all available context | anomaly or cross-node inconsistency not assignable to supported hazard rules | must remain UNKNOWN rather than forced into a known class |

## 2.1 Independent hazard vectors

For node n and hazard h:  E_h(n,t) -> C_h(n,t) -> S_h(n,t) -> R_h(n,t) -> State_h(n,t)

The system stores and displays hazard-specific state independently. A fire event must not overwrite flood state, and an active pollution episode must not be hidden because the fire state is NORMAL.

## 2.2 Hazard state machine

| State | Meaning | Transition intent |
| --- | --- | --- |
| NORMAL | No sufficient evidence of the hazard | Remain normal while evidence stays below activation criteria |
| WATCH | Weak but actionable precursor or elevated environmental condition | Enter after configured persistence or supporting context |
| SUSPECTED | Meaningful evidence exists but confirmation criteria are not met | Escalate with persistence, corroboration or stronger evidence |
| CONFIRMED | Evidence passes configured confirmation logic | Issue/maintain incident state subject to authority workflow |
| CRITICAL | Confirmed hazard with high operational urgency | Fast escalation; safety-first handling |
| RESOLVED | Hazard no longer operationally active and stand-down criteria are met | Remain resolved; reactivation starts a new episode/version where appropriate |

# 3. Scenario Model

A scenario is a versioned, deterministic description of environment, nodes, hazard drivers, communication conditions, sensor faults and evaluation truth. It is not itself a telemetry message.

| Scenario component | Required fields / intent |
| --- | --- |
| Scenario identity | scenario_id, version, seed, description, author, created_at |
| Spatial setup | node coordinates, grid/region definition, terrain/fuel references when used, hazard source locations |
| Temporal setup | start_time, duration, sampling cadence(s), event timeline, replay speed |
| Environmental baseline | temperature, humidity, rainfall, wind, PM/gas background, water level, soil moisture, etc. |
| Hazard drivers | fire ignition/source and environmental drivers; flood rainfall/upstream pulse; pollution source; landslide trigger; heat wave profile |
| Sensor behavior | noise, drift, calibration error, missingness, stale values, dropout, hard failure, contradiction |
| Network behavior | packet loss, latency, partitions, node isolation, recovery, receiver availability |
| Ground truth | private expected hazard extent/state/timing; never sent through live telemetry path |
| Expected evaluation | event windows, target behaviors, acceptance criteria, known corner cases |

# 4. Synthetic Telemetry Architecture

Scenario Definition -> Environment Generator -> Hazard Driver -> Node Observation Model

Node Observation -> Sensor Fault/Quality Model -> Telemetry Serializer -> Real Ingestion Endpoint

(private truth branch -> evaluator / replay oracle only)

The telemetry branch must be indistinguishable, at schema level, from hardware telemetry. The only permitted simulator-specific metadata is transport-safe metadata that does not disclose the hidden hazard answer to reasoning services.

## 4.1 Same-path requirement

Synthetic telemetry uses the same authentication/validation contract as hardware telemetry, with simulator credentials or a clearly segregated trusted simulation identity.

The same receive timestamp, sequence checking, quality evaluation, baseline logic and anomaly/evidence engine are exercised.

Frontend simulation mode must change data source selection only; it must not replace the reasoning pipeline with precomputed visual states.

For fire spread, LIVE and SIMULATION invoke the same geospatial spread engine; simulation supplies inputs, not a pre-drawn answer.

# 5. Synthetic Sensor Observation Model

Synthetic values should be generated from a latent environmental trajectory and then transformed by sensor characteristics. This avoids visually plausible but statistically meaningless random streams.

x_observed(t) = x_environment(t) + bias(t) + noise(t) + event_response(t)

x_final(t) = apply_missingness_stale_drift_failure(x_observed(t))

Where appropriate, event response is hazard-specific and parameterized by sensor class. For example, a simulated smoke/PM channel can respond rapidly after ignition, while humidity may respond more slowly and can be used as supporting context rather than direct fire proof.

## 5.1 Baseline pattern families

| Pattern | Purpose | Examples |
| --- | --- | --- |
| Stationary | Normal conditions and false-positive testing | stable PM, stable temperature, stable water level |
| Diurnal | Realistic periodic behavior | temperature/humidity day-night cycle |
| Trend | Slow environmental movement | rising river level, drying soil |
| Pulse | Short-lived event | rain burst, pollution plume passage |
| Step | Abrupt regime change | sensor exposure to a new environment |
| Ramp | Rapid onset / escalation | fire temperature rise, water-level surge |
| Oscillatory | Periodic perturbation | vibration, machinery-like interference |
| Piecewise | Multiple phases | normal -> precursor -> event -> recovery |

# 6. Multi-Node Spatial Correlation

The simulator must preserve the spatial structure needed to demonstrate distributed intelligence. Nearby nodes should not all produce identical values, but a common hazard should create correlated changes with distance, delay and local variability.

impact_i(t) = source_intensity(t - delay_i) * spatial_attenuation(distance_i)

x_i(t) = background_i(t) + impact_i(t) + local_noise_i(t)

Spatial attenuation and delay are configurable. They are simulation constructs for testing system behavior, not claims of physical truth unless backed by an explicit model or source dataset.

## 6.1 Correlation classes

| Class | Definition | Use |
| --- | --- | --- |
| Independent | Node responds mostly to local noise/background | false-correlation tests |
| Locally correlated | Nearby nodes share a common event with different magnitude/timing | single incident corroboration |
| Regionally correlated | Many nodes experience a broad environmental regime | heat wave, regional pollution |
| Adversarial contradiction | One node disagrees sharply with a correlated group | sensor fault / spoofing resilience |
| Communication-only divergence | Underlying environment agrees but packets arrive late/missing | store-and-forward and freshness handling |

# 7. Fire Scenarios

Fire is the hero hazard and therefore receives the richest scenario family. V1 uses deterministic evidence and the configured fire-spread engine. A learned classifier is optional and must not be required for the core demonstration.

## 7.1 Fire scenario families

| Scenario | Telemetry behavior | Expected system response |
| --- | --- | --- |
| F0 Normal dry/ambient | normal diurnal temperature, PM and gas behavior | NORMAL; baseline becomes READY |
| F1 Small local ignition | rapid local temperature + PM/gas change, neighboring nodes delayed/attenuated | WATCH/SUSPECTED -> CONFIRMED if core evidence passes |
| F2 Fast escalation | steep temperature/PM rise and expanding corroboration | rapid transition toward CRITICAL |
| F3 False smoke spike | PM spike on one node without temperature corroboration and no spatial support | anomaly may be high; hazard confidence remains limited |
| F4 Sensor fault during fire | one critical sensor fails or drifts while peers remain coherent | incident survives; confidence/Information Condition degrade honestly |
| F5 Communication partition | event continues while some nodes are disconnected | local node state remains; regional picture becomes degraded/incomplete |
| F6 Multi-source fire | two spatially separate ignition sources | separate incidents when merge radius/time window not satisfied |
| F7 Changing wind/environment | future spread driver changes mid-run | historical truth frozen; future projection version recomputed from current frontier |
| F8 Receding / resolved | event intensity falls and burned area stabilizes | stand-down only after configured persistence and operational criteria |

## 7.2 Fire telemetry coupling

PM/smoke and gas channels should respond according to configured response curves, not identical copies of temperature.

Humidity can alter context but should not by itself confirm fire.

Wind belongs to environment/context and feeds the geospatial spread model; it is not a direct proof of combustion.

Sensor quality and reliability weight all evidence aggregation. A failed or untrusted channel must not silently become zero-valued evidence.

# 8. Flood Scenarios

Flood scenarios center on water level and temporal change, with rainfall and supporting sensors providing corroboration. The same anomaly-to-evidence-to-state framework is reused.

| Scenario | Primary trajectory | Expected behavior |
| --- | --- | --- |
| W0 Normal river/channel | stable or slow seasonal oscillation | NORMAL |
| W1 Gradual rise | persistent positive level trend with rainfall context | WATCH then SUSPECTED if persistence passes |
| W2 Rapid surge | steep level increase | fast escalation; severity rises with rate and magnitude |
| W3 Rain-only false alarm | heavy rain but no corresponding level rise | confidence remains bounded; no forced confirmation |
| W4 Sensor drift | slow baseline movement in one probe | baseline/reliability logic reduces trust; peers can maintain awareness |
| W5 Sensor failure | water-level signal unavailable | Information Condition DEGRADED/UNKNOWN; do not infer zero water level |
| W6 Recovery | level falls below operational thresholds with stable trend | RESOLVED after persistence / hysteresis |

# 9. Pollution Scenarios

| Scenario | Pattern | Expected behavior |
| --- | --- | --- |
| P0 Background | stable PM/gas with normal variation | NORMAL |
| P1 Local plume | rapid rise at one node then delayed nearby response | localized SUSPECTED/CONFIRMED depending on corroboration |
| P2 Regional episode | moderate correlated elevation across many nodes | higher coverage and confidence |
| P3 Single-sensor spike | isolated extreme reading | anomaly without automatic high-confidence hazard |
| P4 Sensor poisoning/drift | slowly elevated biased channel | quality/reliability degradation |
| P5 Wind-driven movement | source shifts with environmental context | spatial sequence changes while incident remains coherent |

# 10. Landslide Scenarios

Landslide telemetry is especially sensitive to sensor placement and false vibration. The simulator therefore emphasizes correlated trigger conditions and contradiction cases.

| Scenario | Pattern | Expected behavior |
| --- | --- | --- |
| L0 Stable slope | quiet vibration + normal soil/rainfall | NORMAL |
| L1 Rain-triggered instability | soil moisture/rainfall rising, then vibration anomaly | WATCH -> SUSPECTED |
| L2 Rapid movement | sharp vibration/displacement response with corroborating context | fast escalation |
| L3 Vibration nuisance | short isolated vibration burst without trigger context | anomaly may increase, evidence remains limited |
| L4 Partial node outage | best-positioned node unavailable | regional confidence degrades rather than fabricating continuity |

# 11. Extreme Heat Scenarios

| Scenario | Pattern | Expected behavior |
| --- | --- | --- |
| H0 Normal diurnal | expected daily temperature cycle | NORMAL |
| H1 Persistent heat | multi-hour/day elevated temperature | WATCH/SUSPECTED based on configured duration |
| H2 High temperature + humidity | temperature and humidity jointly worsen exposure context | higher severity |
| H3 Regional heat wave | correlated elevated temperature at multiple nodes | regional event with broad spatial support |
| H4 Sensor saturation/failure | sensor clips or goes unavailable | confidence/Information Condition degrade |

# 12. Unknown Hazard and Cross-Hazard Conditions

Unknown is a first-class state, not an error bucket. The system must be able to say: something changed materially, but available evidence does not justify assigning it to a supported hazard.

## 12.1 Unknown scenario families

Large anomaly with no configured hazard evidence group satisfied.

Conflicting sensors across hazard families with insufficient reliability to resolve the conflict.

Novel multi-signal event deliberately outside current rule coverage.

Simulated sensor pattern that looks hazardous but is intentionally generated as instrumentation interference.

## 12.2 Multi-hazard scenarios

Multi-hazard scenarios activate two or more independent hazard vectors. Examples include flood + pollution, fire + extreme heat, or heavy rain + landslide precursors. The system must preserve each hazard state independently and expose cross-hazard conflict when the same destination, route or response action becomes unsafe.

| Combination | Required test |
| --- | --- |
| Fire + extreme heat | heat context must not be mistaken for combustion; fire evidence still needs its own core group |
| Flood + pollution | water hazard and pollution hazard remain separate; downstream safe-location logic sees both |
| Rain + landslide | rain is supporting context; landslide confirmation requires appropriate evidence |
| Two fires | incident correlation must keep spatially/time-separated sources distinct |
| Fire + communication outage | local edge declaration persists while regional information degrades |

# 13. Sensor Fault and Data Quality Injection

Fault injection is mandatory because a resilient disaster network is defined as much by what it refuses to believe as by what it detects.

| Fault | Simulation rule | Expected intelligence effect |
| --- | --- | --- |
| Gaussian-like noise | increase variance within configured bounds | lower stability / anomaly certainty as appropriate |
| Spike | one or few extreme samples | high local anomaly possible; evidence depends on corroboration/persistence |
| Dropout | missing samples | coverage decreases; never substitute zero |
| Stale | repeat last valid measurement with old measurement timestamp | freshness degrades; new event must not be inferred from repeated old value |
| Drift | slow bias term changes over time | baseline/reliability logic should detect reduced trust |
| Bias step | persistent offset after fault event | quality/reliability or calibration checks should reduce trust |
| Hard failure | no valid reading / diagnostic flag | health gate can force reliability toward zero |
| Contradiction | one node/channel opposes trusted group | group agreement and sensor reliability expose disagreement |
| Packet reordering | out-of-order sequence arrival | dedup/replay/sequence logic handles safely |
| Replay packet | duplicate old packet | must not create a fresh event |
| Clock skew | measurement timestamp offset | freshness/time-quality logic handles bounded skew |
| Corruption | invalid schema / signature / range | reject or quarantine before reasoning |

# 14. Communication and Network Scenarios

| Scenario | Injected condition | Expected behavior |
| --- | --- | --- |
| N0 Healthy network | low loss, bounded latency | normal regional fusion |
| N1 Intermittent loss | bursty packet loss | store-forward and freshness behavior visible |
| N2 Node partition | one node isolated | local autonomy remains; regional coverage degrades |
| N3 Master unreachable | Master path unavailable | edge local emergency path remains available where designed |
| N4 Internet outage | cloud unavailable while local Master remains | local dashboard/local alert path continues |
| N5 Recovery storm | many buffered packets return together | deduplication, sequence ordering and catch-up must remain stable |
| N6 Adversarial duplicate | replayed signed telemetry | replay protection prevents double counting |

# 15. Ground Truth Separation

Ground truth is necessary for evaluation but must not contaminate inference. The scenario runner maintains a private evaluator branch that contains the intended hazard source, onset, footprint, affected cells and expected state windows. The telemetry branch contains only observations and diagnostics.

| Artifact | Visible to reasoning pipeline? | Purpose |
| --- | --- | --- |
| Telemetry observation | YES | what the system is allowed to infer from |
| Sensor diagnostics | YES | health/quality/reliability context |
| Receive timestamp / sequence | YES | freshness, ordering, replay protection |
| Scenario ID | OPTIONAL / controlled | replay identification only if it carries no answer |
| Hazard label | NO | evaluation oracle only |
| True hazard footprint | NO | evaluation of geospatial output |
| True onset/end time | NO | temporal detection evaluation |
| Simulator source intensity | NO | prevents answer leakage |
| Expected state transition | NO | tests actual state logic |
| Evaluation report | POST-RUN | accuracy / robustness measurement |

# 16. Scenario and Replay Schema

A canonical scenario manifest should be JSON/YAML compatible. The exact storage backend is an implementation choice; field semantics are normative.

scenario_id: FIRE_F3_FALSE_SMOKE

version: 1

seed: 240901

start_time: 2026-01-01T10:00:00Z

duration_s: 1800

cadence_s: 5

nodes: [...]

environment: {...}

drivers: {...}

sensor_faults: [...]

network_faults: [...]

ground_truth: {private: true, ...}

evaluation: {...}

## 16.1 Minimum scenario manifest fields

| Field | Type | Required | Meaning |
| --- | --- | --- | --- |
| scenario_id | string | yes | stable unique scenario identifier |
| version | integer/string | yes | scenario definition version |
| seed | integer | yes | deterministic RNG seed |
| start_time | timestamp | yes | simulation epoch |
| duration_s | number | yes | simulation length |
| cadence_s | number/list | yes | telemetry cadence or per-node schedule |
| nodes | array | yes | node definitions and locations |
| environment | object | yes | background and context generators |
| drivers | object | yes | hazard source/event generators |
| faults | array | no | sensor/network fault injections |
| ground_truth | private object | yes for evaluation scenarios | hidden oracle |
| evaluation | object | yes for test scenarios | acceptance windows and expected properties |

# 17. Determinism, Replay and Scenario Control

The same scenario manifest + software version + configuration version + seed must reproduce the same synthetic stream, subject to explicitly nondeterministic infrastructure components.

## 17.1 Replay controls

Reset to scenario start.

Play/pause and adjustable replay speed.

Seek to a timestamp without changing the underlying event sequence.

Jump to hazard onset, confirmation, peak, communication outage, recovery or resolution markers.

Replay telemetry through ingestion exactly as recorded/generated.

Re-run the same scenario after changing one configuration version to compare outputs.

Export an evaluation bundle containing manifest hash, software/config versions, telemetry identifiers and result summary.

## 17.2 Event sourcing rule

Historical observations and emitted intelligence should be append-only/versioned. Re-running a model against the same historical telemetry creates a new analysis version rather than rewriting the old result.

# 18. Evaluation Metrics

Synthetic telemetry is valuable only if it can reveal whether NexAlert behaves correctly. Evaluation must measure detection, uncertainty, resilience, localization and operational behavior rather than accuracy alone.

| Metric family | Examples |
| --- | --- |
| Detection | time-to-WATCH, time-to-SUSPECTED, time-to-CONFIRMED, missed-event rate |
| False alarm | false confirmation rate, nuisance alert rate, single-sensor false-positive resistance |
| Confidence quality | coverage-aware confidence degradation, agreement sensitivity, baseline-readiness behavior |
| Resilience | behavior under packet loss, stale data, node isolation, Master/internet outage |
| Correlation | correct merge/separation of incidents by spatial/temporal windows |
| Geospatial | footprint overlap/error against private simulation truth where a physical model is defined |
| Operational | severity/state transition stability, hysteresis behavior, stand-down correctness |
| Reproducibility | same seed/config produces same event stream and equivalent results |
| Performance | ingestion rate, inference latency, queue depth, replay throughput |

# 19. Golden Scenarios for V1

| ID | Scenario | Why it matters | Pass criterion |
| --- | --- | --- | --- |
| G01 | Normal baseline | proves system does not hunt noise | no false confirmation; baseline reaches READY |
| G02 | Fire with 3-node corroboration | hero path and distributed evidence | incident confirms with coherent spatial/temporal evidence |
| G03 | False smoke spike | tests conservative confidence | anomaly may rise; no unjustified confirmation |
| G04 | Flood rapid rise | tests trend + persistence | rapid escalation without threshold flapping |
| G05 | Pollution regional plume | tests multi-node correlation | regional state reflects correlated evidence |
| G06 | Landslide precursor | tests multi-signal evidence | WATCH/SUSPECTED reflects trigger + anomaly coherence |
| G07 | Extreme heat wave | tests duration/context | sustained state without sensitivity to single spikes |
| G08 | Multi-hazard fire + heat | tests independent vectors | both states preserved; no scalar collapse |
| G09 | Communication partition during fire | tests resilience | local declaration survives; regional info becomes degraded |
| G10 | Master/internet outage | tests local path separation | local emergency function remains where designed |
| G11 | Sensor drift + peer corroboration | tests reliability weighting | bad node influence is reduced, not silently trusted |
| G12 | Two separate fires | tests deterministic incident correlation | separate incidents remain separate |
| G13 | Changing wind mid-run | tests versioned future projection | past remains frozen; future projection recomputed |
| G14 | Replay exact match | tests determinism | same seed/config reproduces telemetry and expected outputs |

# 20. Implementation Structure

The simulator should be modular but not over-fragmented. A practical V1 package structure is:

simulation/

scenarios/          # manifests, validators, scenario registry

environment/        # background temporal/spatial generators

hazards/             # fire, flood, pollution, landslide, heat, unknown drivers

sensors/             # response functions, noise, drift, faults

network/             # latency, loss, partitions, replay behavior

telemetry/           # canonical serialization + same-path transport

oracle/               # private ground truth; never imported by inference

replay/               # seek, pause, checkpoint, deterministic rerun

evaluation/           # metrics, event matching, reports

fixtures/             # golden scenario manifests and expected properties

The backend owns orchestration; the frontend controls scenario selection/replay but does not become the simulator of record.

# 21. Configuration and Parameter Rules

All scenario parameters that influence telemetry generation or hazard reasoning must be versioned and auditable. No hidden constants may be introduced in simulation code when a behavior affects evaluation or the SIH demo.

| Parameter class | Examples |
| --- | --- |
| Temporal | cadence, persistence windows, event durations, replay speed |
| Spatial | source location, attenuation radius, node separation, correlation radius |
| Sensor | noise scale, drift rate, response lag, saturation, dropout probability |
| Network | latency distribution, packet-loss model, partition duration, retry behavior |
| Hazard | source intensity curve, onset profile, evidence thresholds, scenario drivers |
| Reasoning | hazard weights, z cap/lambda, confidence weights, hysteresis values |
| Geospatial | grid resolution, CRS, spread coefficients, warning/projection horizons |
| Evaluation | event matching radius/time window, metric thresholds, expected outcome windows |

# 22. Guardrails and Failure Modes

| Failure mode | Guardrail |
| --- | --- |
| Simulator directly tells frontend the answer | frontend renders backend state/geospatial outputs only |
| Hazard label leaks into telemetry | strict schema validation + separate oracle storage |
| Zero used for missing data | missing/stale must remain explicit in telemetry/quality layer |
| Independent hazards collapsed | separate hazard vectors, states and risk layers |
| All nodes behave identically | spatial attenuation + delay + local variability |
| False confidence from single bad sensor | reliability and corroboration gates |
| Replay changes history | versioned analysis; append-only historical record |
| Scenario irreproducible | seed + manifest + config/software version captured |
| Synthetic model presented as field truth | scenario metadata and documentation clearly identify synthetic nature |
| Legacy architecture returns by accident | current final baseline and document authority list are explicit |

# 23. V1 Acceptance Checklist

At least one deterministic scenario exists for each supported hazard and one Unknown case.

At least one multi-hazard scenario activates two independent hazard vectors concurrently.

Synthetic telemetry enters the same backend ingestion contract used by hardware telemetry.

Ground truth is inaccessible to production inference code.

Missing, stale, drifted, failed and contradictory sensor cases are represented explicitly.

Multi-node scenarios contain spatial correlation, delay and attenuation rather than duplicated streams.

Network partition and recovery scenarios exercise store-forward, freshness and deduplication behavior.

Fire scenarios invoke the same fire-spread engine as LIVE mode.

Changing environmental conditions create versioned future projections without rewriting historical truth.

Replay can pause, seek, restart and reproduce a scenario deterministically.

Golden scenarios have machine-checkable acceptance criteria.

Every parameter affecting a test result is versioned and traceable.

SIHDemo mode never depends on decorative or precomputed hazard visuals that bypass backend reasoning.

# 24. Final Engineering Position

NexAlert will not be judged by how many synthetic hazards it can make appear on a screen. The purpose of this layer is to prove that the same architecture can remain useful under normal conditions, uncertain evidence, multiple simultaneous hazards, broken sensors and broken communications. Synthetic telemetry is therefore a controlled test instrument: it creates difficult situations, preserves the observational boundary, and lets the actual NexAlert intelligence prove what it can and cannot infer.