NEXALERT

Communication, Distributed Resilience & Offline Operation

FINAL IMPLEMENTATION SPECIFICATION  •  V1 / SIH 2026

Core principle NexAlert must continue to communicate the truth it already knows when upstream infrastructure is unavailable. A network partition is a degraded information condition, not permission to fabricate certainty.

Document ID:

NEX-11-COMM-RES

Status:

FINAL / Implementation-ready

Authority:

Master System Architecture + Technical Requirements + Mathematical Intelligence + Hardware + Incident Operations

Supersedes:

Any legacy communication design that assumes STM32+ESP32 split, mandatory Meshtastic, or Master/cloud availability

# DOCUMENT MAP

| Section | Scope |
| --- | --- |
| 1. Purpose & invariants | What resilience means and what must never change |
| 2. Communication architecture | Node, Master, local and cloud paths |
| 3. Protocol and transport strategy | Wi-Fi, LoRaWAN/NB-IoT, IP services and selection rules |
| 4. Message classes | Telemetry, heartbeat, event, command, citizen and audit traffic |
| 5. Integrity, ordering and replay protection | HMAC, sequence, timestamps, deduplication |
| 6. Store-and-forward | Offline queues, prioritization, retry and persistence |
| 7. Master failure vs internet failure | Independent degraded modes and recovery |
| 8. Local emergency operation | Node-hosted emergency UI and captive portal |
| 9. Distributed fusion and authority | What happens during partition and disagreement |
| 10. Citizen communications | Online notification, offline access, SOS and acknowledgements |
| 11. Failure modes & operational states | Connectivity, power, clock and node isolation cases |
| 12. Resilience testing | Fault injection, soak, recovery and acceptance tests |
| 13. Configuration & implementation | Parameters, feature flags and code boundaries |
| 14. Acceptance criteria | Definition of done for the communication layer |
| 15. Locked invariants | Non-negotiable implementation constraints |

# 1. Purpose & Design Invariants

This document specifies how NexAlert moves telemetry, health state, hazard evidence, alerts, acknowledgements and citizen safety information across a distributed environment where links may be slow, intermittent, partitioned or completely unavailable. The communication layer is part of the safety architecture: it must preserve freshness, provenance and uncertainty rather than merely maximize delivery rate.

Resilience definition A component may be disconnected and still remain useful. The system degrades from regional intelligence -> local intelligence -> cached emergency information, rather than collapsing from “online” to “dead”.

| Invariant | Implementation rule |
| --- | --- |
| Local autonomy | ESP32 performs acquisition, diagnostics, local health/quality/reliability and configured local hazard reasoning without requiring cloud access. |
| Master autonomy | Raspberry Pi-class Master can ingest, fuse, serve authority UI and run geospatial hazard computation without internet. |
| Internet independence | Loss of internet must not erase node<->Master communication or local citizen emergency access. |
| Master-failure independence | Loss of Master must not erase the last known valid local emergency state from nodes that cached it. |
| Information honesty | Every consumer-visible state carries freshness / information-condition semantics; unknown is never converted to normal. |
| Canonical events | A single incident/alert event ID drives all delivery channels; channels do not invent separate truths. |
| Human authority | NexAlert recommends; an authorized human approves operational dispatch or response actions. |
| Replay safety | Authenticated sequence/timestamp/event identifiers prevent stale packets from being accepted as new state. |
| Store-forward | Critical and safety-relevant messages persist through bounded local queues and retransmit after recovery. |
| Telemetry/label separation | Synthetic telemetry may exercise the pipeline, but hidden scenario ground truth never enters inference inputs. |

# 2. Communication Architecture

NexAlert uses a layered communication model. The prototype can operate primarily over IP/Wi-Fi, while the architecture keeps lower-bandwidth transports available for field deployment. Transport choice is an implementation detail; event semantics and integrity rules remain transport-independent.

| Layer | Primary role | Minimum capability |
| --- | --- | --- |
| L0 — Sensor/MCU | Sensor acquisition and local state | ESP32 firmware, monotonic sequencing, diagnostics |
| L1 — Field node | Local buffering, health, edge reasoning, local emergency service | Wi-Fi AP/local HTTP service, secure uplink, store-forward |
| L2 — Master | Ingestion, cross-node fusion, incident management, GIS and local authority UI | Linux compute, database, message broker/API, geospatial engine |
| L3 — Cloud / remote | Remote notification, archival, remote access and future integrations | Internet connection; optional for core local operation |
| L4 — Citizen endpoint | Emergency information, acknowledgements and SOS | Browser/PWA online or local captive portal offline |

Reference topology ESP32 field nodes <-> local link <-> Raspberry Pi Master <-> optional internet/cloud. A node may additionally host a minimal local emergency page and cached event state so it remains useful during Master failure.

2.1 Logical paths

| Path | Preferred path | Fallback / degraded path |
| --- | --- | --- |
| Telemetry | Node -> Master ingestion API / message transport | Node buffer -> retry when link returns |
| Master -> node command | Authenticated command channel | Command queued until link restores; safety-critical commands require explicit expiry |
| Master -> cloud | HTTPS/WebSocket/push integration | No dependency for local authority operation |
| Citizen online | Cloud notification -> emergency web UI | Retry / recent cached notification |
| Citizen offline | Node Wi-Fi AP -> captive portal -> local emergency UI | No network access outside radio/AP reach |
| SOS | Citizen endpoint -> available local/online receiver | Persist locally and transmit when a permitted path returns |

# 3. Protocol & Transport Strategy

The prototype should prefer the transport that is simplest to validate end-to-end on the current hardware. Wi-Fi/IP is therefore the primary prototype path. LoRaWAN, NB-IoT, 4G/5G or other long-range transports remain supported as deployment adapters, not as hidden assumptions in business logic.

| Transport | Best use | Constraints | V1 status |
| --- | --- | --- | --- |
| Wi-Fi / TCP-IP | Node <-> Master; local AP/captive portal | Range, interference, power use | PRIMARY |
| Ethernet | Fixed Master / gateway backhaul | Wiring required | OPTIONAL |
| LoRaWAN | Low-rate long-range telemetry | Small payloads, downlink constraints, gateway dependence | ADAPTER |
| NB-IoT / LTE-M | Wide-area field connectivity | Modem, SIM/network coverage, power | ADAPTER |
| 4G/5G IP | High-rate remote backhaul | Coverage, power and cost | ADAPTER |
| BLE | Provisioning / local maintenance | Short range; not primary event transport | OPTIONAL |

Transport abstraction The application layer sends typed NexAlert messages. Each transport adapter is responsible for framing, connection lifecycle, retry and MTU constraints; it must not reimplement incident or hazard logic.

3.1 Network service boundaries

| Service | Transport | Purpose |
| --- | --- | --- |
| Node telemetry ingress | HTTPS or authenticated TCP/WebSocket | Receive telemetry, diagnostics and events |
| Node status/health | HTTPS or lightweight authenticated request | Heartbeat and on-demand diagnostics |
| Local citizen UI | HTTP over node AP | Emergency read-mostly local experience |
| Master authority UI | HTTPS/HTTP on LAN | Operational dashboard and incident control |
| Cloud sync | HTTPS/WebSocket | Remote notifications, backup and external integrations |

# 4. Message Classes & Canonical Envelope

All communication objects use a shared envelope so the backend, simulator, hardware and test harness can reason about message identity uniformly. Payload schemas vary by type, but identity, timing, provenance and authentication are common.

| Field | Requirement |
| --- | --- |
| message_id | Globally unique identifier for the message instance. |
| message_type | telemetry ¦ heartbeat ¦ health ¦ event ¦ alert ¦ command ¦ ack ¦ sos ¦ audit. |
| node_id / sender_id | Stable authenticated origin identifier. |
| sequence | Monotonic sender sequence for ordering/deduplication. |
| measurement_timestamp | Sensor-side event/measurement time; never overwritten by receive time. |
| received_timestamp | Assigned by receiver; used for transport freshness and delay analysis. |
| schema_version | Versioned payload contract. |
| priority | NORMAL ¦ HIGH ¦ CRITICAL; affects local queue scheduling. |
| ttl / expires_at | Optional explicit expiry for commands and transient alerts. |
| payload | Typed content for the message class. |
| auth | HMAC or final approved authenticated envelope. |

{
  "message_id": "evt_01H...",
  "message_type": "event",
  "sender_id": "NODE_001",
  "sequence": 18422,
  "measurement_timestamp": "2026-09-07T01:10:15Z",
  "received_timestamp": "2026-09-07T01:10:16Z",
  "schema_version": "1.0",
  "priority": "CRITICAL",
  "payload": { ... },
  "auth": {"alg":"HMAC-SHA256","value":"..."}
}

# 5. Integrity, Ordering, Freshness & Replay Protection

A packet that arrives is not automatically a packet that should be trusted. NexAlert validates origin, integrity, freshness and sequence semantics before the payload can affect operational state.

1. Authenticate the sender using its provisioned per-node secret and the configured HMAC algorithm.

2. Verify the message body and canonical serialization before parsing it into domain state.

3. Reject malformed, expired or otherwise invalid messages before they reach hazard reasoning.

4. Deduplicate using message_id and sender sequence.

5. Track receive time separately from measurement time; large transport delay lowers freshness but does not rewrite history.

6. Apply bounded clock-skew rules. A bad wall clock must not create false historical ordering when the sender sequence remains valid.

7. Persist the last accepted sequence window across reboot where practical; use a bounded replay cache for event IDs.

Golden rule Freshness answers “how recent is the information?” Integrity answers “did an authenticated sender produce it?” Neither one alone proves that the sensor value is correct.

| Condition | System action |
| --- | --- |
| Valid auth + new sequence + fresh timestamp | Accept and process normally. |
| Valid auth + delayed timestamp | Accept if within configured lateness window; mark transport delay/freshness accordingly. |
| Valid auth + duplicate sequence/event | Ignore as duplicate; do not create a second operational action. |
| Valid auth + sequence gap | Accept if policy permits, record missing range and request recovery/backfill if available. |
| Invalid HMAC | Reject; record security event; do not update hazard state. |
| Expired command | Reject; record expired command for audit. |
| Unknown sender | Reject and record unauthorized-source event. |

# 6. Store-and-Forward Architecture

Every node maintains a bounded persistent outbound queue for telemetry and event messages that could not be delivered. The queue is a ring-buffered safety mechanism, not an unbounded database.

| Queue class | Examples | Policy |
| --- | --- | --- |
| Q0 — CRITICAL | Emergency event, node-local hazard declaration | Highest priority; durable; retry aggressively with backoff; retain until delivered or explicitly expired. |
| Q1 — HIGH | Health degradation, escalation, critical diagnostics | Durable; retransmit after recovery. |
| Q2 — NORMAL | Telemetry samples, routine status | Bounded; oldest low-value samples may be compacted or dropped under pressure according to policy. |
| Q3 — BULK | Historical backfill / debug traces | Lowest priority; suspend under constrained links. |

6.1 Retry algorithm

delay = min(base_delay * 2**attempt, max_delay)
delay = jitter(delay)
retry until delivered, expired, or policy_drop
while CRITICAL queue non-empty: transmit Q0 before Q2/Q3

| Rule | Implementation |
| --- | --- |
| Backoff | Exponential backoff with bounded jitter; avoid synchronized node storms. |
| Compaction | Prefer compacting routine telemetry before dropping critical events. |
| Drain order | CRITICAL -> HIGH -> NORMAL -> BULK. |
| Recovery burst control | Limit packets/sec during reconnection so Master/network is not flooded. |
| Durability | Use crash-safe local storage or append-only journal plus bounded ring index. |
| Observability | Expose queue depth, oldest message age, retry count and drop count. |

# 7. Master Failure vs Internet Failure

These are separate failure domains and must produce separate information conditions. The system must never display “offline” as a single vague state when the actual fault is known.

| Mode | Master | Internet | Node<->Master | Required behavior |
| --- | --- | --- | --- | --- |
| A — Fully connected | UP | UP | UP | Full authority, cloud and citizen channels available. |
| B — Internet lost | UP | DOWN | UP | Local authority dashboard and distributed fusion continue; cloud delivery pauses; nodes buffer outward traffic if needed. |
| C — Master lost | DOWN | Any | DOWN | Nodes continue local sensing/edge reasoning; cached emergency state and node-hosted local emergency page remain available; central fusion unavailable. |
| D — Master + internet lost | DOWN | DOWN | DOWN | Field nodes operate independently; local emergency pages and local hazard states remain usable; no false regional claims. |
| E — Node isolated | UP | UP or DOWN | Single node isolated | Node continues local state and buffering; authority records node as unreachable/stale. |

Mandatory distinction Master failure is an application/compute failure. Internet failure is an upstream connectivity failure. Their UI states, recovery procedures and audit records must remain distinct.

7.1 Local emergency continuity requirement

Each field node caches the latest signed canonical emergency state relevant to its local area, including event ID, hazard, severity, state, issue/update time and freshness condition.

The node exposes a minimal read-mostly HTTP emergency page through its own Wi-Fi AP/local network service when configured and powered.

The local page must not require the Master, cloud, external DNS or internet access.

When the Master returns, the node and Master reconcile using event ID, version/update timestamp and authenticated source rules rather than blindly overwriting local state.

Local emergency content is explicitly labeled “LOCAL / LAST KNOWN” when the regional command view is unavailable.

# 8. Offline Citizen Emergency Operation

Offline access is not a separate product. It is the same canonical emergency information model served locally when the citizen cannot reach cloud infrastructure.

| Step | Behavior |
| --- | --- |
| 1. Discover | Citizen sees distinctive NexAlert Wi-Fi SSID in the local service area. |
| 2. Join | Citizen connects to the node/gateway AP; no internet assumption. |
| 3. Captive portal | DNS/HTTP redirect or explicit local landing page opens the emergency UI. |
| 4. Read | UI shows hazard, state, severity, distance/direction when computable, freshness and immediate safety action. |
| 5. Act | Citizen can view safe-location guidance where locally cached/calculable, and can use SOS/contact functionality supported by the local path. |
| 6. Recover | Once an uplink returns, queued citizen and node events synchronize to the canonical backend. |

Offline honesty The UI must state when content is cached/local and how old it is. It must never show a live regional map as though it were current when only a last-known local snapshot exists.

| Offline page block | Required behavior |
| --- | --- |
| Emergency status | Hazard + severity + state + issued/updated age. |
| Immediate action | Clear instruction or SHELTER_IN_PLACE when no viable safe route is known. |
| Map | Local cached geometry or last-known local footprint; freshness clearly visible. |
| Communication | Available local instructions; no fake “message sent” confirmation without a receiver/queue acknowledgement. |
| SOS | Accept locally where supported; show “queued / sent / delivered” as distinct states. |
| System status | LOCAL MODE / MASTER UNAVAILABLE / LAST KNOWN as appropriate. |

# 9. Distributed Fusion, Partition & Authority

Nodes may disagree with one another or with the Master’s broader picture. Distributed resilience means preserving valid local evidence while explicitly modeling uncertainty and source scope.

| Situation | Rule |
| --- | --- |
| Same hazard + close spatial/temporal evidence | Master may deterministically correlate into one incident using configured merge radius/time window. |
| Different hazards at same location | Keep independent hazard vectors; do not collapse them into one universal score. |
| Neighbor disagrees with node | Retain both authenticated observations; recompute confidence/evidence according to the mathematical intelligence specification. |
| Node declares local hazard while Master lacks external context | Do not erase the node’s valid local declaration solely because external evidence is absent. |
| Cloud disagrees with local Master state | Local operational state remains available; cloud disagreement becomes a reconciliation/audit event. |
| Partition split-brain | Do not allow two independently issued operational commands to silently overwrite each other; use event/version/authority metadata and human reconciliation where required. |

9.1 Source scope metadata

| Scope | Meaning |
| --- | --- |
| LOCAL_NODE | Evidence/state produced at one field node. |
| LOCAL_MASTER | State fused by the local Master from connected nodes. |
| REGIONAL_CLOUD | Broader context obtained through cloud/external systems. |
| CACHED | Previously valid state available while current authority path is unavailable. |
| UNKNOWN | Required information is missing or stale enough that a stronger claim is not supported. |

# 10. Citizen Notifications, Delivery & SOS

Citizen communication follows one canonical event with channel-specific delivery receipts. The communication system distinguishes issued, delivered, opened, acknowledged and unreachable outcomes.

| Stage | Definition |
| --- | --- |
| ISSUED | Authorized alert exists in the canonical alert store. |
| DISPATCHED | A delivery worker handed the alert to a configured channel/provider. |
| DELIVERED | Endpoint/provider confirms receipt where such confirmation exists. |
| OPENED | Citizen endpoint indicates that the emergency page/content was accessed. |
| ACKNOWLEDGED | Citizen explicitly confirms receipt where the workflow supports acknowledgement. |
| UNREACHABLE | No valid delivery path or endpoint confirmation exists within policy. |

No fake reachability A notification request is not proof that a phone received it. A fully powered-off phone cannot be awakened by the prototype. The audit trail must preserve the difference between attempted delivery and confirmed delivery.

10.1 SOS path

1. Citizen presses-and-holds or otherwise confirms SOS according to the configured UX.

2. Client creates a unique SOS event ID and timestamp.

3. Client transmits through the best available path: online service first when reachable; local NexAlert path when offline and supported.

4. Receiver validates identity/signature and deduplicates repeated submissions without outright blocking a user from reattempting an SOS.

5. Authority dashboard categorizes the request as IMMEDIATE / HIGH / STANDARD / VERIFY rather than using a misleading single numeric “danger score”.

6. Operational action remains human-approved. The system can recommend response priority and location context, but does not autonomously dispatch emergency services.

# 11. Failure Modes & Operational States

| Failure | Detection | Expected state | Recovery |
| --- | --- | --- | --- |
| Wi-Fi link loss | Heartbeat timeout / packet loss | NODE_ISOLATED | Continue local sensing + queue; reconnect/backoff. |
| Master process crash | Local health watchdog/API failure | MASTER_DEGRADED | Restart service; nodes continue local path. |
| Master power loss | Node heartbeat absence / gateway loss | MASTER_UNAVAILABLE | Node-local emergency service remains available if powered and configured. |
| Internet loss | Backhaul health check fails | CLOUD_UNAVAILABLE | Local Master/authority operation continues. |
| Clock drift | Clock sanity check | TIME_DEGRADED | Use sequence + monotonic time; mark wall-clock confidence low. |
| Queue full | Queue depth threshold | BUFFER_PRESSURE | Drop/compact lowest-priority bulk data first; preserve critical events. |
| Bad HMAC | Auth validation failure | SECURITY_REJECT | Discard payload; emit security/audit event. |
| Sensor stale | Measurement age exceeds threshold | TELEMETRY_DEGRADED | Lower quality/reliability; do not silently substitute zero. |
| Conflicting hazards | Evidence/correlation layer | MULTI_HAZARD_ACTIVE | Keep independent hazard states and user-visible conflict guidance. |

11.1 Information-condition propagation

GOOD — required communication inputs are recent enough for the intended decision.

DEGRADED — some relevant inputs or channels are missing/stale, but useful local or cached information remains.

UNKNOWN — information required to make the requested claim is unavailable or too stale to support it.

# 12. Resilience Validation & Fault-Injection Plan

Communication resilience is demonstrated through deliberate failure, not through a green network icon. Each test records time to detect, time to degrade, retained state, user-visible condition, recovery latency and data loss.

| Test ID | Injected fault | Expected result | Pass condition |
| --- | --- | --- | --- |
| R-01 | Internet unplugged from Master | Local authority operation continues | No loss of local incident control; cloud status becomes unavailable/degraded. |
| R-02 | Master process killed | Nodes keep sensing and buffering | Node-local state persists; cached emergency UI remains reachable where configured. |
| R-03 | Master physically powered off | Node local path continues | Emergency page does not depend on Master. |
| R-04 | Wi-Fi link loss 5 min | Node queues data | Critical events retained; no false resolved state. |
| R-05 | Reconnect 20 nodes simultaneously | Backpressure occurs | System drains without crash or unacceptable queue storm. |
| R-06 | Duplicate replay packet | No duplicate state transition | Exactly-once operational effect even if network is at-least-once. |
| R-07 | Invalid HMAC | Security reject | Payload never reaches hazard reasoning. |
| R-08 | Clock jump | Temporal logic degrades safely | No future/past corruption; sequence remains usable. |
| R-09 | Conflicting node evidence | Fusion recalculates confidence | Local valid hazard not erased solely by disagreement. |
| R-10 | Power cycle node | Persistent queue + latest emergency state restored | No corruption; recovery observable. |
| R-11 | Offline citizen joins AP | Captive portal works | Emergency content loads without internet. |
| R-12 | Repeated SOS taps | Events consolidated but not blocked | Single operational SOS case with audit trail of retries. |

Recommended benchmark evidence Capture packet loss, telemetry latency, queue depth, CPU/free heap, local page response time, HMAC verification cost, recovery time and number of lost/deduplicated messages. Include plots/tables in the validation document, not as marketing-only screenshots.

# 13. Configuration & Implementation Boundaries

| Parameter / flag | Suggested V1 default | Purpose |
| --- | --- | --- |
| heartbeat_interval_s | 30 | Fast node reachability indication. |
| health_report_interval_s | 300 | Detailed periodic diagnostics. |
| telemetry_queue_capacity | Configured per node storage budget | Bounded store-forward. |
| critical_retry_max_delay_s | 60 | Backoff ceiling for high-priority events. |
| normal_retry_max_delay_s | 300 | Backoff ceiling for routine data. |
| clock_skew_limit_s | Configurable | Timestamp sanity bound. |
| event_dedup_window_s | Configurable | Replay/duplicate handling window. |
| local_cache_max_age_s | Configurable by event class | Citizen-facing freshness guard. |
| captive_portal_enabled | true for field demo | Local emergency access. |
| cloud_sync_enabled | true when internet available | Optional remote delivery. |
| transport_primary | wifi_ip | Prototype transport adapter selection. |

13.1 Code ownership boundaries

| Component | Owns | Must NOT own |
| --- | --- | --- |
| ESP32 firmware | Sensor reads, diagnostics, local state, HMAC, queue, local emergency service | Regional GIS, population exposure, heavy routing, cross-node incident fusion |
| Master communication service | Transport adapters, authentication verification, ingestion, queues, synchronization | Sensor-driver logic |
| Incident service | Deterministic correlation, lifecycle, alert objects | Raw radio framing |
| Citizen delivery worker | Channel dispatch, receipts, retries | Hazard classification |
| Simulation service | Synthetic telemetry emission through real ingestion path | Injecting hidden labels into reasoning |
| Frontend | Presentation, user interaction and local-page rendering | Independent hazard truth or decorative spread simulation |

# 14. Acceptance Criteria

1. A node can continue sensing, performing configured local intelligence and buffering messages when the Master is unreachable.

2. Loss of internet does not disable the local authority dashboard or local node<->Master operation.

3. Loss of Master does not erase the last known emergency state from a field node that has cached it.

4. A citizen can reach the local emergency page from the NexAlert AP without internet access when the local service is configured and powered.

5. All accepted node messages are authenticated and deduplicated.

6. The backend preserves measurement time separately from receive time.

7. Critical events survive network interruption and are retried after reconnection.

8. The system makes the distinction between GOOD, DEGRADED and UNKNOWN visible at appropriate authority/citizen surfaces.

9. Split-brain or conflicting evidence does not silently erase valid local hazard declarations.

10. Citizen delivery states distinguish issued, dispatched, delivered, opened, acknowledged and unreachable where each stage is measurable.

11. SOS is never auto-rejected solely because the system has no current known hazard.

12. The communication layer passes replay, invalid-auth, power-cycle and partition tests without violating the mathematical or operational invariants in the rest of the NexAlert specification set.

# 15. Locked Invariants — Do Not Regress

LOCKED These rules are part of the NexAlert architecture contract. A future optimization is invalid if it breaks one of them.

Node <-> Master authentication is mandatory for trusted operational traffic.

Master failure and internet failure are separate states.

Local emergency operation must not require internet or cloud access.

Cached emergency information is labeled with scope and freshness; stale data is never presented as live.

Store-and-forward queues are bounded and priority-aware.

At-least-once transport may be used, but operational effects are deduplicated/idempotent.

Canonical event IDs are reused across alert delivery channels.

Synthetic telemetry follows the same ingestion path as hardware telemetry.

No frontend-only simulated hazard is allowed to become the source of truth.

NexAlert recommends; authorized humans approve operational dispatch and response actions.

SAFE_LOCATION / SHELTER_IN_PLACE behavior must remain explicit when no viable safe route is known.

Multi-hazard states remain independent; no universal combined hazard score is introduced merely for UI convenience.

Every resilience claim is testable with an explicit fault-injection scenario.

END OF SPECIFICATION

NexAlert remains useful when the network is not.