# Simulation, Replay & Scenario Control

NexAlert SIH26178 | Final Implementation Specification | Document 15

Purpose: define the deterministic simulation, replay, scenario-control, time-travel, fault-injection, and evaluation system used to test the same NexAlert intelligence pipeline that receives live hardware telemetry.

Authority: this specification derives from the locked Master Architecture, Mathematical Intelligence, Fire/Geospatial, Data, Multi-Hazard, Incident/Response, Communication/Resilience, and UI specifications. Simulation is a validation and demonstration system, not a hidden oracle for live inference.

# 1. Purpose and Core Contract

The simulation subsystem must make NexAlert testable under normal conditions, evolving hazards, sensor faults, network partitions, multi-node corroboration, and multi-hazard conflict. Every simulated observation enters the same telemetry ingestion, validation, health/quality, baseline, anomaly, evidence, confidence, severity, risk, incident, alert, and UI paths used by hardware-originated observations.

| Principle | Requirement |
| --- | --- |
| Same pipeline | Simulator emits telemetry/events only; it does not call downstream decision functions directly. |
| Deterministic | Given scenario version, seed, initial state, configuration version, and code build, the same run is reproducible. |
| Ground truth private | Hazard truth, injected faults, and expected outcomes are stored as evaluation metadata and are never sent into the reasoning path. |
| Time controlled | Clock can run slower, equal to, faster than real time, pause, seek to checkpoints, and replay exact event order. |
| Live-safe | Simulation mode cannot accidentally publish production citizen alerts or alter live node state without an explicit environment boundary. |
| Explainable | Every derived decision can be traced to telemetry, configuration, event history, and simulation step. |
| Replayable | Any run can be archived and replayed from its immutable input bundle. |

# 2. Simulation Environments

## 2.1 Required environment separation

| Environment | Purpose | External Effects |
| --- | --- | --- |
| SIMULATION | Development, testing, SIH demonstration | No citizen notifications, no real node writes |
| STAGING | Integration with near-production services | Sandbox-only notification adapters and synthetic identities |
| LIVE | Real hardware and approved operations | Production controls and human-approved alerts/actions |

Environment identity is mandatory in every run, event envelope, database record, API response, log line, and UI header. A SIMULATION run must not be able to authenticate as a LIVE event producer.

# 3. Simulation Architecture

The simulator is an event-producing subsystem around the canonical NexAlert pipeline. It contains a scenario engine, virtual node fleet, hazard/world model, fault injector, clock controller, transport emulator, ground-truth recorder, checkpoint manager, and run evaluator.

| Component | Responsibility | Must Not Do |
| --- | --- | --- |
| Scenario Engine | Loads versioned scenario definitions and schedules state changes | Bypass telemetry ingestion or write final alarm states |
| Virtual Node Fleet | Emulates sensors, diagnostics, battery/comm behavior, and node timing | Pretend to be actual ESP32 hardware |
| Hazard/World Model | Produces environmental evolution and hidden truth layers | Publish truth labels to intelligence services |
| Fault Injector | Applies missing/stale/noisy/drift/failure/contradiction/communication faults | Silently alter recorded ground truth |
| Transport Emulator | Models latency, loss, duplication, reordering, partitions, recovery | Hide packet loss from metrics |
| Clock Controller | Controls simulation time and event ordering | Move LIVE clock |
| Ground Truth Store | Persists truth and injected conditions for evaluation | Serve truth to production inference endpoints |
| Evaluator | Compares system outputs against ground truth and scenario expectations | Rewrite system outputs to make a test pass |

# 4. Canonical Simulation Data Flow

The mandatory path is: scenario definition -> virtual environment -> virtual node telemetry -> authentication/envelope validation -> ingestion -> health/quality -> reliability -> baseline/anomaly -> hazard evidence -> confidence/severity/risk -> incident correlation -> alert/response -> dashboard/citizen surfaces. Ground truth is written in parallel to a private evaluator store.

For fire scenarios, environmental/geospatial inputs drive the same fire-spread engine used by LIVE/SIMULATION UI. The frontend must never draw a decorative hazard shape independently of simulation state.

# 5. Scenario Model

| Field | Required Meaning |
| --- | --- |
| scenario_id | Stable human-readable identifier |
| version | Semantic or monotonically increasing scenario version |
| hazards | One or more independent hazard tracks |
| region | Geospatial extent, coordinate reference, and node placement |
| duration | Simulation horizon and optional warm-up period |
| clock | Start timestamp and simulation step/event rules |
| seed | Deterministic seed used by stochastic generators |
| initial_conditions | Environmental, node, network, and incident state |
| environmental_drivers | Rainfall, wind, temperature, humidity, terrain/fuel inputs as applicable |
| node_profiles | Sensor inventory, baseline status, health, battery/power, communications |
| faults | Timed or conditional telemetry/network faults |
| ground_truth | Private hidden hazard state for evaluation only |
| expected_outcomes | Acceptance assertions and measurable targets |
| safety_policy | Rules controlling external-effect adapters |

# 6. Scenario Families

| Family | Example Objective |
| --- | --- |
| Baseline / Normal | Verify stable telemetry, no false incidents, healthy baseline learning |
| Slow Fire Onset | Verify gradual evidence accumulation and persistence/hysteresis |
| Rapid Fire Escalation | Verify fast escalation and current/warning/projection surfaces |
| Flood Rising | Verify trend-relative water anomaly and multi-node corroboration |
| Pollution Event | Verify PM/gas/temperature evidence and degraded sensing behavior |
| Landslide/Vibration | Verify vibration-feature path and missing-data degradation |
| Extreme Heat | Verify temperature/humidity evidence without forcing unrelated hazards |
| Multi-Hazard | Fire + flood, pollution + heat, or independent simultaneous incidents |
| Contradictory Sensors | One strong signal vs. neighboring disagreement / bad probe |
| Network Partition | Node-to-Master loss, Master isolation, recovery and replay |
| Master Failure | Master unavailable while local node emergency path remains possible |
| Sensor Drift/Failure | Progressive drift, hard failure, stale values, correlated bias |
| Alert Lifecycle | Watch -> suspected -> confirmed -> critical -> resolved -> stand-down |
| Replay Regression | Re-run frozen scenario after code/config change and compare outputs |

# 7. Hazard Simulation Models

## 7.1 Fire simulation

Fire simulation uses the locked geospatial chain: DEM + fuel + moisture + wind -> directional rate of spread -> arrival-time propagation -> current/warning/projection geometry -> operational buffer -> hazard risk surface -> exposure. Wind is treated as FROM meteorological input and converted to propagation direction before directional spread calculation. Terrain slope/aspect is computed in metric projected coordinates. Non-burnable cells have zero ROS; the front routes around them. Arrival time is propagated through orthogonal and diagonal neighbors with metric distances.

When environmental drivers change, historical truth is frozen. The current frontier is extracted, future arrival times are reset from that frontier, and future propagation is recomputed. The new projection is versioned so the UI can distinguish a revised forecast from a rewritten past.

## 7.2 Flood / pollution / heat / landslide

These scenario generators focus on sensor-observable evolution rather than pretending to be high-fidelity national disaster physics. The generator may synthesize water-level trends, rainfall, PM spikes, gas response, temperature/humidity shifts, soil moisture changes, and vibration features using configurable functions and spatial correlation. The intelligence pipeline remains responsible for deciding what these observations mean.

# 8. Multi-Hazard Orchestration

Each hazard is simulated as an independent truth track and intelligence vector. The system must not collapse fire, flood, pollution, landslide, heat, or unknown-hazard risk into a universal scalar. A node may contribute evidence to more than one hazard vector when its sensor class is relevant.

| Rule | Behavior |
| --- | --- |
| Independent state | Each hazard maintains its own evidence/confidence/severity/risk/state |
| Shared observations | A telemetry observation may be mapped to multiple hazard-specific evidence groups |
| Shared incident? | Only when deterministic incident-correlation rules say observations belong to the same event |
| Conflicting hazards | Citizen/authority UI shows hazard-specific conflicts and safe-direction implications |
| Safe location | Ranking evaluates destination against all active hazards; nearest is not automatically safest |

# 9. Virtual Node and Telemetry Generator

A virtual node mirrors the canonical telemetry contract: node_id, sequence, measurement_timestamp, device monotonic time if applicable, location, measurements, diagnostics, power/battery, source, schema_version, and HMAC/auth envelope. Missing means absent/stale, never numeric zero unless the sensor truth is actually zero.

| Generator Control | Examples |
| --- | --- |
| Sampling | 1 s, 5 s, 30 s, 60 s or configured node rate |
| Noise | Gaussian, bounded, asymmetric, burst noise |
| Drift | Linear, piecewise, step, random walk |
| Missingness | Random, contiguous outage, sensor-specific outage |
| Staleness | Freeze value while timestamps advance or stop |
| Bias | Constant or time-varying offset |
| Correlation | Neighboring nodes move together with spatially decaying correlation |
| Contradiction | One group disagrees with otherwise correlated neighbors |
| Sensor failure | Hard failure / invalid flag / saturation |
| Power | Battery decline, brownout, recovery, sensor duty-cycle effects |
| Communication | Loss, duplication, delay, reorder, partition, reconnection |

# 10. Network and Resilience Simulation

| Fault | Expected System Observation |
| --- | --- |
| Packet loss | Quality/freshness degrades; no fabricated continuity |
| Duplicate packet | Idempotency by telemetry identity/sequence prevents duplicate state changes |
| Reordering | Measurement timestamp and sequence preserve ordering semantics |
| Partition | Local autonomy continues; Master marks node reachability degraded |
| Master power loss | Node-local emergency path remains available for configured local actions |
| Internet loss | Master/local system continues local operation; cloud delivery is degraded separately |
| Recovery | Buffered telemetry replays with bounded backoff and deduplication |
| Auth failure | Message rejected and audited; no state mutation |
| Replay attack | Old sequence/timestamp/event rejected per security policy |

# 11. Simulation Clock and Event Ordering

Simulation time is independent from wall time. The clock controller supports REALTIME, FAST_FORWARD, PAUSED, STEP, SEEK_TO_CHECKPOINT, and REPLAY modes. Every emitted event carries simulation timestamp and deterministic sequence/order metadata.

When multiple events share a timestamp, ordering is deterministic: configuration changes -> node health/availability -> sensor measurements -> network delivery -> ingestion/processing -> incident transitions -> notification adapters. The exact internal worker ordering must not change the persisted event sequence for a deterministic build.

| Mode | Behavior | Primary Use |
| --- | --- | --- |
| REALTIME | 1 simulation second = 1 wall-clock second | Demo / observation |
| FAST_FORWARD | Runs as fast as safe while preserving event ordering | Regression tests |
| PAUSED | No event advancement | Debugging |
| STEP | Advance one logical event or fixed step | Root-cause analysis |
| SEEK | Restore nearest checkpoint and re-run forward | What-if analysis |
| REPLAY | Play an immutable prior run | Audit / demonstration |

# 12. Checkpoints and Time Travel

Checkpoints are immutable snapshots containing scenario version, code/config identifiers, simulation clock, node state, pending events, transport queues, active incidents, baseline state, hazard-spread state where applicable, and random-generator state. Seeking never mutates the original run; it creates a child run with a parent_run_id and branch point.

# 13. Fault Injection Framework

Faults are first-class scenario objects with a start condition, scope, mutation, duration/termination rule, and expected evaluator effect. Fault definitions are recorded separately from the telemetry they produce so the evaluator can distinguish a system failure from an injected environmental or hardware condition.

| Fault Class | Examples | Pass Criterion |
| --- | --- | --- |
| Sensor integrity | CRC/schema/invalid-value condition | Rejected or degraded safely |
| Sensor quality | Noise, drift, saturation | Reliability responds; no unjustified confidence |
| Baseline | Insufficient history / freeze / recovery | Correct readiness/state transitions |
| Topology | Node isolation / route change | Local and regional state remain distinct |
| Transport | Delay/loss/reorder/duplicate | Correct freshness and idempotency |
| Authentication | Wrong HMAC / old sequence | Rejected and audited |
| Processing | Worker restart / queue pressure | No silent event loss; bounded recovery |
| Hazard change | Wind/rain/moisture shift | Future projection updates; past stays fixed |
| Citizen channel | Push unavailable / offline portal | Correct channel fallback behavior |

# 14. Ground Truth and Evaluation Separation

Ground truth is an evaluator-only representation of what the simulator intentionally created. For example, a fire cell may have ignition truth, arrival time, physical state, and exposure truth. The live intelligence path sees only telemetry and approved contextual inputs. This separation prevents label leakage and makes simulation useful as an adversarial test instead of a scripted demo.

Evaluation labels may be aligned to system timestamps after a run, but never injected into live inference requests. Expected outcomes define ranges and invariants rather than requiring an exact internal score when the architecture intentionally uses operational indices rather than probabilities.

# 15. Metrics and Evaluation

| Metric | Definition / Use |
| --- | --- |
| Detection delay | Time from truth crossing a defined evaluation threshold to first valid alert/incident state |
| False positive rate | Alerts/incidents produced when scenario truth is below defined hazard condition |
| False negative rate | Missed or late hazard conditions above evaluator threshold |
| Spatial overlap | Agreement between predicted/current/warning/projected geometry and truth at matching times |
| Arrival-time error | Prediction vs. truth for fire-cell arrival where truth supports timing |
| Confidence calibration review | Check whether evidence/coverage language behaves as documented; C_h is not treated as probability |
| Alert delivery latency | Event issue -> channel acceptance/open where observable |
| Recovery time | Fault injection -> restored valid operation |
| Data loss | Expected vs. persisted telemetry/events across failures |
| Determinism | Same bundle rerun produces equivalent persisted outputs within defined tolerance |
| Resource behavior | CPU, memory, queue depth, latency under accelerated simulation |

# 16. Golden Scenario Suite

| Scenario ID | Mandatory Assertion |
| --- | --- |
| G01-NORMAL-001 | Healthy nodes remain NORMAL with no spurious incident |
| G02-FIRE-SLOW-001 | Slow fire reaches WATCH/SUSPECTED before CONFIRMED according to configured persistence |
| G03-FIRE-FAST-001 | Rapid evidence can escalate without waiting through normal persistence |
| G04-FIRE-WIND-001 | Wind directionality matches FROM-to-TO conversion and directional ROS behavior |
| G05-FIRE-CHANGE-001 | Environmental change revises future projection without rewriting history |
| G06-FLOOD-TREND-001 | Water-level trend anomaly is evaluated relative to local baseline |
| G07-POLLUTION-001 | PM/gas evidence can trigger pollution path without falsely creating fire |
| G08-MULTI-001 | Two simultaneous hazards maintain independent states and risk surfaces |
| G09-CONTRADICTION-001 | One faulty sensor cannot erase corroborated valid evidence |
| G10-PARTITION-001 | Node partition triggers degraded information state and local continuity |
| G11-MASTER-OFF-001 | Master loss does not imply node-local emergency path loss |
| G12-SECURITY-001 | Bad HMAC/sequence is rejected and audited |
| G13-REPLAY-001 | Recorded run replays deterministically |
| G14-SOS-001 | SOS remains available even without a known hazard; no automatic rejection |
| G15-SHELTER-001 | No viable safe route produces SHELTER_IN_PLACE guidance |

# 17. Scenario Control Interface

Authority-only simulation controls expose scenario selection, environment, run ID, seed, clock mode, start/pause/step/reset, checkpoint seek, speed multiplier, injected faults, hazard-driver controls, node health overrides, transport controls, ground-truth visibility (simulator/evaluator role only), and export.

The public citizen surface never exposes ground truth, evaluator labels, hidden seeds, internal score formulas, or scenario-control endpoints.

| Control | Validation |
| --- | --- |
| Load scenario | Schema validated before any event can emit |
| Start | Environment and permissions verified; safety policy shown |
| Pause/step | No new event except explicit step action |
| Inject fault | Logged with actor, time, scope, old/new state |
| Seek | Creates child run; parent immutable |
| Reset | Creates new run or restores explicit baseline; does not truncate audit history |
| Export | Bundles scenario, config hashes, event manifest, outputs, metrics, evaluator results |

# 18. API and Run Lifecycle

Minimal backend surface: create scenario run, validate scenario, start/pause/resume/stop, advance step, create checkpoint, seek/branch, inject fault, set simulation parameter, query run state, query event stream, query ground truth for evaluator role, compute metrics, and export replay bundle.

Every mutating request is idempotent where practical and authenticated. Run status states are CREATED -> VALIDATED -> RUNNING -> PAUSED -> COMPLETED or STOPPED; FAILED is terminal for infrastructure failure and retains partial artifacts.

# 19. Replay Bundle Specification

| Artifact | Purpose |
| --- | --- |
| scenario.yaml/json | Exact scenario definition and version |
| config snapshot | Parameter registry values actually used |
| build metadata | Application/model/engine commit or image identifiers |
| seed + clock policy | Deterministic execution context |
| node manifests | Virtual node inventory and sensor configuration |
| event manifest | Ordered event stream or immutable event references |
| truth bundle | Private evaluator truth |
| output bundle | Observed system outputs and state transitions |
| metrics report | Computed performance results |
| hash manifest | Integrity check for all bundle parts |

# 20. Security and Safety Boundaries

• SIMULATION credentials cannot authenticate to LIVE ingestion or notification adapters.

• Ground-truth endpoints require evaluator/admin role and must be disabled on citizen-facing routes.

• Simulation alerts are visibly labeled in authority UI and cannot be presented as live emergency instructions.

• Notification adapters default to sink/console delivery in SIMULATION; real channels require explicit environment configuration and approval.

• Run creation, fault injection, parameter changes, seek/branch, and exports are audited.

• No simulation feature may mutate live node firmware, configuration, relay outputs, or citizen profiles.

# 21. Performance and Determinism Requirements

| Target | Requirement |
| --- | --- |
| Event throughput | Support at least 10x prototype telemetry rate during accelerated regression with bounded queue growth |
| Clock fidelity | No duplicate or missing logical steps caused solely by acceleration |
| Replay | Golden scenario produces same state transitions and event IDs under same build/config/seed |
| Checkpoint | Create/restore without corrupting incident or transport state |
| UI responsiveness | Simulation controls remain responsive while background workers process events |
| Backpressure | Simulator slows or buffers when ingestion capacity is exceeded rather than silently dropping truthless data |
| Resource visibility | CPU/memory/queue/latency counters available to operator during accelerated runs |

# 22. Testing and Acceptance Matrix

| Test | Pass Condition |
| --- | --- |
| Scenario schema | Invalid scenarios fail before execution with actionable validation errors |
| Pipeline parity | Simulation and hardware-originated telemetry reach the same ingestion contracts |
| Determinism | Rerun yields equivalent outputs and identical truth under pinned build/config/seed |
| Fault isolation | Injected fault appears in telemetry/diagnostics and does not contaminate evaluator truth |
| Fire geometry | Current/warning/projection are derived from arrival-time state and actual geometry |
| Multi-hazard | Independent hazard vectors persist; no universal combined score appears |
| Master outage | Local emergency capability survives Master loss in supported prototype path |
| Network recovery | Buffered events recover without duplicate incident transitions |
| Security | Simulation cannot cross environment boundary |
| Replay export | Bundle can reconstruct run provenance and metrics |

# 23. SIH Demonstration Scenarios

## 23.1 Hero demo: Fire detection -> spread -> alert

Start in SIMULATION with a healthy virtual node cluster. Gradually introduce correlated temperature/PM/gas evidence, show health/quality/reliability, transition to WATCH/SUSPECTED/CONFIRMED as configured, then visualize actual current/warning/projected fire geometry from the same spread engine. Change wind and demonstrate future projection revision while historical affected area remains stable. Issue an alert through the sandbox adapter and show acknowledgement/response lifecycle.

## 23.2 Resilience demo: connectivity loss

Partition a node or disconnect the Master from upstream services. Show stale/freshness changes, preserved local state, buffered telemetry, local emergency access path, and recovery/replay after reconnection. The dashboard must distinguish Master failure from internet/cloud failure.

## 23.3 Multi-hazard demo

Run a pollution event near one cluster while a separate flood/fire track evolves elsewhere. Show independent incident cards, independent risk surfaces, and a safe-location ranking that accounts for destination exposure rather than simply selecting the nearest shelter.

# 24. Implementation Rules for Vibe-Coding / Agents

• Build one vertical simulation slice first: scenario -> telemetry -> ingestion -> intelligence -> incident -> UI.

• Use the canonical telemetry schema; do not invent a simulation-only payload that bypasses production validation.

• Keep scenario generators, fault injectors, evaluators, and ground truth outside the live decision modules.

• Every new simulator parameter must be registered in the configuration/feature-flag registry with a default, unit, range, owner, and rationale.

• Write golden-vector tests for mathematical functions and deterministic replay tests for end-to-end event streams.

• Do not add autonomous actions merely because simulation makes them easy to script; NexAlert recommends and humans approve operational dispatch.

• When changing fire or geospatial logic, rerun geometry, environmental-change, non-burnable-cell, CRS, and arrival-time regression suites.

• When changing transport/security logic, rerun loss/reorder/duplicate/HMAC/replay/master-loss scenarios.

• Keep the simulator honest: it creates observations and conditions; it must not secretly answer the hazard question for the intelligence engine.

# 25. Final Simulation Invariants

• Simulation uses the same ingestion and reasoning path as hardware telemetry.

• Ground truth is private evaluation data, never an inference input.

• Multi-hazard truth and system state are independent by hazard.

• Fire simulation is geospatial and arrival-time based, not a decorative radius animation.

• Environmental changes revise future projection without rewriting the past.

• Faults are explicit, timestamped, auditable, and separable from truth.

• Replay is deterministic under a pinned scenario, seed, configuration, and build.

• Branching creates child runs; original runs remain immutable.

• Simulation cannot cross into LIVE control or citizen-notification paths without explicit safety gates.

• Every demo claim must be reproducible from an archived run bundle.