NEXALERT

Configuration, Parameter & Feature-Flag Registry

FINAL IMPLEMENTATION SPECIFICATION  •  V1 / SIH 2026

| Field | Value |
| --- | --- |
| Document ID | NEX-19-CONFIG-FLAGS |
| Status | FINAL / Implementation-ready |
| Authority | Master Architecture + TRD + Mathematical Intelligence + Geospatial + Hardware + Backend + Data + Multi-Hazard + Operations + Communication/Resilience + UI + Simulation + Security + Validation + Implementation |
| Purpose | Single controlled registry for tunable constants, thresholds, policy values, hardware limits, feature flags, and environment settings across Edge, Master, Backend, Simulation, and UI. |
| Core rule | Code reads configuration; code does not silently invent production thresholds. |
| Safety posture | Conservative defaults; unsafe or ambiguous values fail closed or become UNKNOWN/DEGRADED rather than fabricating certainty. |

# DOCUMENT MAP

1. Registry Governance & Rules

2. Configuration Namespaces

3. Edge Node & Sensor Parameters

4. Intelligence & Baseline Parameters

5. Hazard Reasoning Parameters

6. Fire / Geospatial Parameters

7. Incident / Alert / Response Parameters

8. Communication & Resilience Parameters

9. Security Parameters

10. Simulation / Replay / Scenario Parameters

11. UI / UX & Presentation Parameters

12. Environment Profiles & Deployment Overrides

13. Feature-Flag Registry

14. Validation Gates & Change Control

15. SIH Demo Profile

16. Implementation Checklist

# 1. Registry Governance & Rules

This registry is the authoritative catalog of named configuration. Every tunable value must have a stable key, unit/type, default, allowed range or enumerated values, owner, scope, and validation rule. Values affecting safety, incident state, alert issuance, geospatial propagation, authentication, or citizen communication require an explicit change record and corresponding validation tests.

## 1.1 Configuration classes

| Class | Meaning | Examples | Change policy |
| --- | --- | --- | --- |
| CONST | Mathematical constant or fixed protocol value | MAD scale 1.4826; Earth radius | Code review; rare change |
| TUNABLE | Operational model parameter | anomaly lambda; persistence window | Config change + tests |
| POLICY | Authority/operator policy | operational buffer; alert cooldown | Named owner + audit |
| LIMIT | Hard safety/resource ceiling | payload size; queue depth | Must pass load/failure tests |
| ENUM | Controlled state/mode value | NORMAL/WATCH/... | Code+contract change if altered |
| FLAG | Feature gate | fire.engine_v1 | Default OFF for experimental features |
| SECRET | Credential/key material | node HMAC secret | Never stored in source/config repo |
| PROFILE | Environment bundle | DEV/SIM/STAGE/SIH/PROD | Versioned snapshot |

## 1.2 Naming convention

Use lower_snake_case keys with domain prefix: edge.*, intelligence.*, hazard.*, fire.*, incident.*, comm.*, security.*, sim.*, ui.*, deploy.*, flag.*. Units belong in the registry description and schema, not in key names, except where ambiguity is operationally dangerous.

## 1.3 Runtime precedence

Built-in safe default.

Versioned application configuration file/profile.

Environment-specific override.

Secret manager / provisioning store for secrets.

Runtime operator controls only for explicitly whitelisted POLICY keys; never for core security primitives or immutable protocol semantics.

The effective configuration must be queryable and exposed in the Authority System/Audit surfaces as a version/hash, not as secret values.

# 2. Configuration Namespaces

| Namespace | Scope | Primary consumers | Typical risk |
| --- | --- | --- | --- |
| edge.* | ESP32 field node | firmware | High |
| sensor.* | Sensor drivers/calibration | ESP32 + ingestion | High |
| intelligence.* | Health/quality/baseline/anomaly | Edge + Master | High |
| hazard.* | Hazard evidence/state | Edge + Master | Critical |
| fire.* | Fire spread/geospatial | Master | Critical |
| incident.* | Correlation/state | Backend + Master | Critical |
| alert.* | Notification policy | Backend | Critical |
| comm.* | Transport/store-forward/offline | Edge + Master | High |
| security.* | Authentication/signing/replay | All | Critical |
| sim.* | Simulation/replay | Simulation | Medium |
| ui.* | Presentation behavior | Frontend | Medium |
| deploy.* | Deployment/runtime | All | High |
| flag.* | Feature exposure | All | High |

# 3. Edge Node & Sensor Parameters

| Key | Type / Unit | Default | Allowed / Notes | Owner |
| --- | --- | --- | --- | --- |
| edge.telemetry_interval_s | int / s | 5 | >=1; normal telemetry cadence | Firmware |
| edge.heartbeat_interval_s | int / s | 30 | >=5; lightweight liveness | Firmware |
| edge.health_report_interval_s | int / s | 300 | ~5 min detailed health | Firmware |
| edge.max_buffer_records | int / records | 2000 | bounded ring buffer; hardware-memory dependent | Firmware |
| edge.critical_queue_priority | enum | CRITICAL | critical before normal when transport constrained | Firmware |
| edge.local_service_enabled | bool | true | required for local emergency path on supported node/gateway | Firmware |
| edge.local_ap_ssid_prefix | string | NEXALERT- | must not contain secrets/PII | Firmware |
| edge.local_ap_max_clients | int | 8 | benchmark-dependent; must not starve sensing | Firmware |
| edge.sequence_counter_persist_every | int / records | 100 | persistent checkpoint cadence; replay risk trade-off | Firmware |
| sensor.temp.min_c | float / °C | -40 | validate against actual sensor | Hardware |
| sensor.temp.max_c | float / °C | 85 | validate against actual sensor | Hardware |
| sensor.humidity.min_rh | float / %RH | 0 | sensor-specific | Hardware |
| sensor.humidity.max_rh | float / %RH | 100 | sensor-specific | Hardware |
| sensor.water_level.min_m | float / m | 0 | negative only if sensor semantics permit | Hardware |
| sensor.pm.max_ugm3 | float / µg/m³ | 1000 | display/quality ceiling; calibrated sensor dependent | Hardware |
| sensor.gas.max_engineering | float | TBD | must be calibration-defined; no fake ppm | Hardware |
| sensor.stale_after_s | int / s | 30 | measurement freshness threshold; override per sensor | Firmware |
| sensor.failure_after_s | int / s | 120 | no valid reading/heartbeat window | Firmware |

Calibration constants (offset, gain, response curve, gas species mapping, sensor-native units) are node-specific assets, not universal globals. A calibrated sensor profile must identify hardware revision, calibration date, method, and validity interval.

# 4. Intelligence & Baseline Parameters

| Key | Type / Unit | Default | Allowed / Notes | Owner |
| --- | --- | --- | --- | --- |
| intelligence.health_weight_sum_tol | float | 0.0001 | weights must sum to 1 within tolerance | ML/Platform |
| intelligence.quality_floor | float 0-1 | 0.2 | below floor may suppress inference | ML |
| intelligence.reliability_floor | float 0-1 | 0.2 | node/signal below floor treated weak evidence | ML |
| intelligence.mad_scale | float | 1.4826 | robust-normal consistency constant | ML |
| intelligence.mad_epsilon | float | 1e-6 | prevents divide-by-zero | ML |
| intelligence.z_cap | float | 8 | bounds anomaly transform | ML |
| intelligence.anomaly_lambda | float | 3 | positive; controls saturation of A_i | ML |
| intelligence.baseline_min_samples | int | 120 | readiness gate; hazard/sensor specific overrides allowed | ML |
| intelligence.baseline_learning_timeout_s | int / s | 1800 | after timeout state may remain LEARNING/DEGRADED | ML |
| intelligence.baseline_freeze_on_confirmed | bool | true | prevent contaminated learning | ML |
| intelligence.recovery_min_good_samples | int | 30 | before returning to LEARNING/READY | ML |
| intelligence.trend_window_s | int / s | 300 | trend-relative water/soil context | ML |
| intelligence.drift_window_s | int / s | 3600 | longer quality/drift assessment | ML |

## 4.1 Sensor health / reliability weighting

The production pipeline remains H_i -> Q_i -> R_i = H_i Q_i -> baseline/anomaly. Battery is intentionally excluded from R_i and is surfaced as a power/health dimension. Hard failure gates H_i to zero. Missing data is never interpreted as numeric zero.

# 5. Hazard Reasoning Parameters

| Key | Type | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| hazard.min_core_coverage | float 0-1 | 0.67 | minimum core-evidence coverage before confidence assessment | Safety |
| hazard.coverage_weight | float | 0.30 | C_cov contribution | Safety |
| hazard.agreement_weight | float | 0.30 | C_agree contribution | Safety |
| hazard.temporal_weight | float | 0.20 | C_temp contribution | Safety |
| hazard.baseline_weight | float | 0.20 | C_base contribution | Safety |
| hazard.confidence_assess_floor | float 0-1 | 0.30 | below => confidence N/A | Safety |
| hazard.confirm_persistence_s | int / s | 30 | hazard-specific overrides permitted | Safety |
| hazard.critical_persistence_s | int / s | 10 | fast escalation window; cannot bypass safety evidence | Safety |
| hazard.resolve_clear_s | int / s | 120 | clear window before RESOLVED | Safety |
| hazard.watch_decay_s | int / s | 300 | hysteresis decay where applicable | Safety |
| hazard.state_merge_radius_m | float / m | 50 | incident correlation starting point | Operations |
| hazard.state_merge_window_s | int / s | 30 | incident correlation starting point | Operations |
| hazard.info_condition_min_reliability | float 0-1 | 0.40 | maps to GOOD/DEGRADED/UNKNOWN policy | Safety |

The numerical defaults above are implementation starting points and must be calibrated/validated against actual field trials and golden scenarios before production use. They are not probabilities or evidence that a hazard exists.

# 6. Fire / Geospatial Parameters

| Key | Type / Unit | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| fire.grid_cell_size_m | float / m | 25 | metric projected grid; benchmark on target area | Geo |
| fire.max_runtime_area_km2 | float / km² | 25 | simulation guardrail for demo runtime; scale tested separately | Geo |
| fire.neighbor_connectivity | enum | 8 | 8-neighbor arrival propagation in V1 | Geo |
| fire.meteorological_wind_is_from | bool | true | convert FROM to TO before propagation | Geo |
| fire.wind_midflame_factor | float 0-1 | 0.4 | WAF starting point; fuel/context dependent | Geo |
| fire.nonburnable_ros | float / m/s | 0 | barrier state | Geo |
| fire.warning_horizon_s | int / s | 1800 | warning zone horizon | Operations |
| fire.projection_horizon_s | int / s | 3600 | must be > warning horizon | Operations |
| fire.recompute_on_environment_change | bool | true | freeze history, reseed frontier, recompute future | Geo |
| fire.operational_buffer_m | float / m | 100 | policy buffer; distinct from physical footprint | Authority |
| fire.risk_time_decay_lambda | float | 0.001 | U_c exp(-lambda*tau); calibrate for UI use | Safety |
| fire.use_directional_vectors | bool | true | wind+slope directional vectors; never arbitrary scalar sum | Geo |
| fire.simplify_display_geometry | bool | true | display only; raw raster/geometry remains source of truth | Geo |

Fire engine must use a projected metric CRS. Meteorological wind direction is interpreted as FROM and converted to propagation direction. Physical footprint, warning/projected zones, and operational buffer are separate layers.

# 7. Incident / Alert / Response Parameters

| Key | Type | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| incident.correlation_radius_m | float / m | 50 | same hazard + within radius + temporal window => candidate same incident | Ops |
| incident.correlation_window_s | int / s | 30 | same incident temporal window | Ops |
| incident.reopen_window_s | int / s | 300 | post-resolve recurrence behavior | Ops |
| incident.alert_cooldown_s | int / s | 300 | fatigue guardrail; critical escalation may override | Ops |
| alert.require_human_approval | bool | true | NexAlert recommends; human approves | Authority |
| alert.max_retry_count | int | 8 | transport-specific retry cap | Backend |
| alert.exponential_backoff_base_s | float / s | 2 | retry base | Backend |
| alert.max_backoff_s | int / s | 300 | cap retry delay | Backend |
| alert.ttl_s | int / s | 3600 | alert expiration; emergency flow may differ | Backend |
| alert.resolution_requires_explicit_standdown | bool | true | no silent disappearance | Authority |
| sos.hold_seconds | int / s | 3 | press-and-hold UX; not auto reject outside hazards | Citizen |
| sos.priority_tiers | enum set | IMMEDIATE,HIGH,STANDARD,VERIFY | categorical; no fake numeric score | Authority |
| response.no_safe_route_action | enum | SHELTER_IN_PLACE | fallback when route/destination unavailable | Authority |

# 8. Communication & Resilience Parameters

| Key | Type | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| comm.hmac_algorithm | enum | HMAC-SHA256 | per-node transport/authentication | Security |
| comm.replay_window_s | int / s | 120 | timestamp freshness window; sequence is primary dedup primitive | Security |
| comm.max_clock_skew_s | int / s | 60 | adjust with time source quality | Security |
| comm.store_forward_enabled | bool | true | bounded ring buffer | Firmware |
| comm.critical_first_queue | bool | true | critical data before routine telemetry where transport constrained | Firmware |
| comm.retry_jitter_s | float / s | 1 | prevents synchronized retries | Firmware/Backend |
| comm.offline_portal_cache_s | int / s | 900 | local emergency state cache lifetime | Citizen |
| comm.local_gateway_health_interval_s | int / s | 15 | Master/gateway health heartbeat | Resilience |
| comm.master_failure_timeout_s | int / s | 45 | distinguish Master failure from internet failure | Resilience |
| comm.internet_failure_timeout_s | int / s | 60 | cloud reachability classification | Resilience |
| comm.partition_reconciliation_window_s | int / s | 600 | history reconciliation window | Backend |
| comm.max_offline_event_age_s | int / s | 86400 | stale event handling; still retain audit lineage | Backend |

# 9. Security Parameters

| Key | Type | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| security.signature_algorithm | enum | Ed25519 | signed emergency/event candidate where enabled | Security |
| security.hmac_secret_source | enum | secure_provisioning | never hard-code in repo | Security |
| security.nonce_bytes | int | 16 | cryptographic nonce size | Security |
| security.event_id_entropy_bits | int | 128 | unique event identifiers | Security |
| security.max_auth_failures_per_min | int | 30 | rate-limit; tune to deployment | Security |
| security.key_rotation_days | int | 90 | deployment policy; emergency rotation can be earlier | Security |
| security.revocation_cache_s | int / s | 3600 | bounded offline cache | Security |
| security.admin_session_ttl_s | int / s | 1800 | authority dashboard | Security |
| security.audit_retention_days | int / days | 365 | deployment/legal policy dependent | Security |

# 10. Simulation / Replay / Scenario Parameters

| Key | Type | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| sim.mode | enum | SIMULATION | SIMULATION/LIVE; explicit separation | Simulation |
| sim.random_seed | int | 20260907 | must be recorded in run metadata | Simulation |
| sim.tick_interval_s | float / s | 1 | scenario clock step | Simulation |
| sim.telemetry_jitter_s | float / s | 0.2 | synthetic transport timing variability | Simulation |
| sim.packet_loss_rate | float 0-1 | 0.00 | scenario override | Simulation |
| sim.duplicate_rate | float 0-1 | 0.00 | replay/dedup test | Simulation |
| sim.missing_rate | float 0-1 | 0.00 | missing != zero | Simulation |
| sim.stale_rate | float 0-1 | 0.00 | freshness/quality testing | Simulation |
| sim.bias_drift_rate | float | 0.00 | sensor drift scenario | Simulation |
| sim.contradiction_rate | float 0-1 | 0.00 | cross-sensor disagreement scenario | Simulation |
| sim.freeze_ground_truth_output | bool | true | ground truth never enters inference path | Safety |
| sim.replay_deterministic | bool | true | seed + scenario manifest + config hash | Simulation |
| sim.allow_live_transport | bool | false | must remain false for simulation environment | Safety |

# 11. UI / UX & Presentation Parameters

| Key | Type | Default | Notes | Owner |
| --- | --- | --- | --- | --- |
| ui.map.refresh_s | float / s | 2 | Authority map target; adaptive under load | Frontend |
| ui.citizen.emergency_first_view | bool | true | hazard/severity/state/distance/direction/freshness first | Citizen |
| ui.show_internal_scores_by_default | bool | false | C_h/S_h/R_h hidden from default citizen UX | Product |
| ui.show_information_condition | bool | true | GOOD/DEGRADED/UNKNOWN explicit | Product |
| ui.show_physical_vs_operational_zone | bool | true | distinct layers/labels | Safety |
| ui.colorblind_safe_palette | bool | true | mandatory design rule | UX |
| ui.sos_hold_seconds | int / s | 3 | must match sos.hold_seconds | Citizen |
| ui.offline_banner_enabled | bool | true | source/freshness/degraded status visible | Citizen |
| ui.simplify_fire_geometry_for_display | bool | true | does not modify source geometry | Frontend |

# 12. Environment Profiles & Deployment Overrides

| Profile | Purpose | Key characteristics |
| --- | --- | --- |
| DEV | local development | safe synthetic data; simulation only; secrets from local vault/env |
| TEST | automated tests | golden vectors, fault injection, deterministic seed; no external alerts |
| SIM | operator simulation | full simulator; same ingestion path; LIVE transport disabled |
| STAGE | integration / field rehearsal | realistic topology; sandbox notifications; full audit |
| SIH | competition demo | controlled demo scenario; local resilience enabled; external alert side effects disabled |
| PROD | future deployment | approved calibration + policy; real credentials; alert side effects enabled only with authority approval |

Profiles are versioned bundles. A deployment must expose profile name, registry version, effective configuration hash, git commit, firmware build, backend build, model/config asset hashes, and simulation seed when applicable.

# 13. Feature-Flag Registry

| Flag | Default | Purpose | Safety / rollout rule |
| --- | --- | --- | --- |
| flag.fire_engine_v1 | true | enable physics-informed cellular fire engine | mandatory for fire simulation; no decorative frontend-only spread |
| flag.fire_ml_classifier | false | optional learned fire classifier | OFF until real data shows improvement over deterministic baseline |
| flag.flood_engine_v1 | true | enable flood hazard path | use validated synthetic + real trial evidence |
| flag.pollution_engine_v1 | true | enable pollution reasoning | hazard-specific evidence rules |
| flag.landslide_engine_v1 | false | experimental landslide path | OFF until validated |
| flag.heat_engine_v1 | true | extreme heat reasoning | validated thresholds only |
| flag.distributed_fusion | true | Master cross-node fusion | must preserve local truth under disagreement |
| flag.local_offline_portal | true | offline citizen emergency UI | mandatory resilience path |
| flag.web_push | true | online citizen notifications | requires explicit subscription/permission |
| flag.authority_auto_dispatch | false | autonomous dispatch | must remain OFF; human approval required |
| flag.route_optimization | false | safe-route optimization | SHELTER_IN_PLACE fallback remains mandatory |
| flag.external_cell_broadcast | false | future authorized integration | not for prototype |
| flag.experimental_risk_heatmap | false | new risk algorithm | must be isolated and benchmarked |
| flag.telemetry_raw_archive | true | retain raw telemetry for audit/replay | bounded/retention policy |
| flag.synthetic_live_merge | false | mix simulation telemetry with live | must stay OFF in SIH/live unless specifically approved |

## 13.1 Flag safety classes

| Class | Examples | Default | Rule |
| --- | --- | --- | --- |
| SAFE | UI-only presentation | ON/OFF | may be runtime-toggled |
| CONTROLLED | simulation/experimental model | OFF | change requires test evidence |
| CRITICAL | security, dispatch, source-of-truth semantics | fixed/guarded | runtime toggle prohibited or role-gated |
| ENVIRONMENT | transport/external integration | profile-controlled | must match deployment profile |

# 14. Validation Gates & Change Control

Every new key requires schema/type/unit/default/range/owner and a test reference.

Every threshold used in a safety or state transition requires a golden scenario proving expected NORMAL/WATCH/SUSPECTED/CONFIRMED/CRITICAL/RESOLVED behavior.

Every fire parameter change requires geospatial benchmark plus no-regression test for non-burnable cells, wind FROM→TO conversion, 8-neighbor propagation, time horizons, and history-preserving recomputation.

Every communication/security parameter change requires replay, duplication, partition, authentication, and offline-path tests.

Every UI parameter that changes emergency prominence must pass citizen/authority acceptance tests and accessibility checks.

Production profile cannot contain TBD values, experimental flags, placeholder secrets, or simulation-only transport settings.

Configuration changes are versioned independently from application builds when possible; the effective configuration hash is stored in audit/event metadata.

Rollback means restoring a previously validated profile version, not manually editing individual values during an incident.

## 14.1 Registry validation matrix

| Check | Requirement | Failure action |
| --- | --- | --- |
| Schema | valid type, enum/range, unit | reject registry load |
| Cross-key consistency | warning horizon < projection horizon; SOS times match; weights sum | reject load or mark profile invalid |
| Safety ceilings | no value exceeds documented hard limit | reject profile |
| Secrets hygiene | no secrets in plain config | reject CI build |
| Feature compatibility | flag combinations valid | reject deployment |
| Migration | new/removed keys handled by version | block startup on unknown critical keys |
| Runtime observability | effective hash/version visible | degrade system if unavailable for authority diagnostics |

# 15. SIH Demo Profile

The SIH profile is a deterministic, low-risk demonstration configuration. It keeps the production architecture visible while preventing accidental real-world side effects. Simulation telemetry uses the same ingestion path as hardware, but simulation mode is explicit and external notification integrations remain disabled.

| Key | SIH value | Why |
| --- | --- | --- |
| deploy.profile | SIH | explicit competition environment |
| sim.mode | SIMULATION | no live side effects |
| sim.random_seed | 20260907 | deterministic replay |
| sim.replay_deterministic | true | repeatable demo |
| flag.authority_auto_dispatch | false | human remains in control |
| flag.local_offline_portal | true | demonstrate resilience |
| flag.external_cell_broadcast | false | no external broadcast |
| flag.synthetic_live_merge | false | keep simulation/live isolated |
| fire.warning_horizon_s | 1800 | clear warning visual |
| fire.projection_horizon_s | 3600 | clear projection visual |
| alert.require_human_approval | true | demonstrate approval workflow |
| ui.show_information_condition | true | demonstrate honest uncertainty |

# 16. Implementation Checklist

Create typed registry schema (Pydantic/JSON Schema-equivalent) shared by backend, simulator, and generated TypeScript types.

Generate a machine-readable config bundle from this registry; keep human-readable DOCX as governance artifact.

Add startup validation for ranges, enums, cross-key constraints, and forbidden production combinations.

Add config hash to telemetry metadata, incident records, alerts, simulation manifests, and audit trail where applicable.

Expose non-secret effective configuration in Authority > System > Configuration.

Keep secret material outside the registry: per-node HMAC secrets, signing keys, database credentials, push credentials, and deployment tokens use secure provisioning/secrets storage.

Add golden tests for every critical threshold and feature-flag branch.

Freeze the SIH profile before demo rehearsal; create an immutable snapshot and checksum.

Never hot-edit critical safety parameters during a live incident. Use a validated profile change and auditable deployment action.

When a value is unknown or not calibrated, use TBD/disabled in non-production profiles rather than inventing a plausible number.

# FINAL INVARIANTS

Configuration is data, not hidden logic.

No threshold becomes authoritative merely because it is convenient.

Confidence is not probability; risk is not probability.

Multi-hazard risk layers remain independent.

Local node truth is not erased by distributed disagreement.

Physical hazard footprint and operational policy buffers remain distinct.

Simulation cannot leak private ground truth into inference.

Autonomous dispatch remains disabled; NexAlert recommends and a human approves.

Master failure and internet failure are distinct operating conditions.

An unknown value leads to honest degradation, not fabricated certainty.